3GPP TSG RAN WG4 Meeting #99-e		R4-2108260

Electronic Meeting, May 19th - 27th, 2021

Agenda item:       6.1.6

Source: 	Qualcomm

Title: 	WF on LBT models for NR-U RRM performance requirements

Document for:     Discussion and Decision

# 1	Introduction

This document contains the agreements and discussions in RAN4#99-e in the email thread [99-e][210] NR_unlic_RRM_2 for the topics concerning DL and UL CCA models applicable to both dynamic and semi-static channel access mechanisms. The complete discussion summary on this topic can be found in the document [1].

Background information: In RAN4#97-e meeting, NR-U RRM performance requirements we first discussed, with the agreements captured in the WF [2]. Agreements on CCA models for NR-U RRM performance requirements from RAN#98-e and RAN#98-bis-e meetings were captured in the WF [3][4].

The following colour code is used to highlight the agreements in this meeting:

Agreements in the GTW session

Agreements in the 1st round

Agreements in the 2nd round

FFS items and candidate options

# 2	General

PCCA dependency on Es/Iot (Issue 2-1-2)

- CCA DL success probabilities are applicable to any value of Es/Iot.

- CCA UL success probabilities are applicable to any value of Es/Iot.

Requirement classification for statistical testing (Issue 2-1-3)

- Determine that TCs under CCA with 0 < PCCA <1 are subject to statistical testing.

  - Send LS to inform RAN5 about the RAN4 decision.

# 3	DL CCA models during RRM tests

CCA success probabilities for DL CCA model in typical test cases (Issue 2-2-1)

- For LBE: P1 = 0.75, P2 = 0.75

- For FBE: P = 0.9375

DRX CCA model (Issue 2-2-2)

- The existing DL CCA model in non-DRX shall also apply when DRX is used.

- Regardless of whether DRX is used or not, prior to each DBT window, the test equipment shall determine whether the CCA attempt is successful.

# 4	UL CCA models during RRM tests

CCA success probability in UL (Issue 2-3-1)

- RAN4 to adopt the following CCA success probabilities for UL CCA model in typical test cases

  - For LBE: P = 0.75

  - For FBE: P = 0.87

Limitation of CCA failures in UL (Issue 2-3-2)

- Include limitation of the UL CCA failures LCCA_UL on the UL CCA model.

Test case list to include UL CCA failures (Issue 2-3-3)

- FFS: Define “MAC CE based TCI state switch delay” test case with UL CCA failure

Noise pattern used for modeling UL CCA failures (Issue 2-3-4)

- OCNG pattern is used for noise generation during the UL CCA detection time (TCCA) within the UL resources where the UE needs to assess the UL CCA.

- During the UL CCA detection time the test equipment should generate energy level 3 dB above the energy detection threshold defined in TS 37.106.

# References

- R4-2108382, “Email discussion summary: [99-e][210] NR_unlic_RRM_2”, Nokia, Nokia Shanghai Bell, RAN4#99-e.

- R4-2017089, “WF on NR-U RRM performance,” Nokia, Nokia Shanghai Bell, RAN4#97-e.

- R4-2104088, “WF on CCA models for NR-U RRM performance requirements,” Qualcomm, RAN4#98-e.

- R4-2105710, “WF on CCA models for NR-U RRM performance requirements,” Qualcomm, RAN4#98-bis-e