3GPP TSG-RAN WG4 Meeting # 99-e        										     	R4-2108437

Electronic Meeting, 19th-27th of May, 2021

Agenda item:			9.12.1, 9.12.1.1, 9.12.1.2, 9.12.1.3, 9.12.1.4

Source:	Moderator (THALES)

Title:	Email discussion summary for [99-e][312] NTN_Solutions_Part1

Document for:	Information

# Introduction

This discussion summary document captures general issues related to RAN4 RF part Rel-17 NR NTN WI, including system parameters, NTN architecture, and regulatory discussions, including exemplary bands. It contains a summary of the contributions under sections and subsections 9.12.1.1, 9.12.1.2, 9.12.1.3, 9.12.1.4 at TSG-RAN WG4 #99-e, together with identified key open issues and recommends topics/questions to be handled via email discussions. The goal of this document is to provide recommendation on prioritization of discussion.

Please also note the draft TSG-RAN WG4 #99-e meeting agenda with respect to NTN topic:

9.12	Solutions for NR to support non-terrestrial networks (NTN)	[NR_NTN_solutions]

9.12.1	General and work plan	[NR_NTN_solutions-Core]

* Include candidate band discussion for FR2

9.12.1.1	System parameters	[NR_NTN_solutions-Core]

9.12.1.2	NTN architecture	 [NR_NTN_solutions-Core]

9.12.1.3	Regulatory information	[NR_NTN_solutions-Core]

9.12.1.4	Others 	[NR_NTN_solutions-Core]

9.12.2	Coexistence aspects	[NR_NTN_solutions-Core]

9.12.2.1	Coexistence scenarios and Simulation assumptions	[NR_NTN_solutions-Core]

9.12.2.2	Simulation results 	[NR_NTN_solutions-Core]

9.12.3	RF requirements 	[NR_NTN_solutions-Core]

9.12.3.1	Network side requirements 	[NR_NTN_solutions-Core]

9.12.3.2	UE requirements 	[NR_NTN_solutions-Core]

9.12.4	RRM core requirements	[NR_NTN_solutions-Core]

9.12.4.1	General	[NR_NTN_solutions-Core]

9.12.4.2	GNSS-related requirements 	[NR_NTN_solutions-Core]

9.12.4.3	Timing requirements	[NR_NTN_solutions-Core]

9.12.4.4	Measurement requirements	[NR_NTN_solutions-Core]

For informative purpose, RAN4#99-e E-meeting Arrangements and Guidelines proposed the following schedule:

- Moderators provide initial summary (Draft) by Monday May 17th, 7pm UTC

- Companies can provide comments on initial summary by Tuesday May 18th, 7pm UTC

- Moderators kick off email discussion (Wednesday May 19th)

- Companies provide comments for the 1st round (Wednesday May 19th – Friday 9am UTC May 21st)

- Moderators summarize the status and possible proposals, recommending what decisions can be made for 1st round. A formal t-doc will be used (Friday 7pm UTC May 21st)

- Moderators kick off 2nd round email discussion (no later than Monday 3am UTC May 24th)

- After receiving the summary from moderators, session chair may approve documents, make agreements or assign new CRs, WFs, LSs, etc. (Monday 8am UTC May 24th)

- Draft WF/LS and revised CRs/TPs shall be shared by Monday 7pm UTC May 24th

- Formal tdocs of WF/LS/CRs/TPs shall be uploaded to the Inbox by Wednesday 3am UTC, May 26th.

- Companies provide comments for the 2nd	round (no later than Wednesday 7pm UTC May 27th)

- Moderators provide 2nd round summary with a formal tdoc by Thursday 8am UTC, May 27th.

A total of 16 TDocs have been identified for this agenda (please also see the Annex for the details, with all the observations/proposals), however only 14 were available after the submission deadline for RAN4#99-e:

| TDoc Number | TDoc Type | Title | Company | Status | General Purpose | Agenda Item |
| --- | --- | --- | --- | --- | --- | --- |
| R4-2110813 | discussion | Discussion of FR2 (Ka/Ku) Satellite bands for NR based satellite networks | Intelsat, Hughes, Inmarsat, ESA, Thales, Fraunhofer | available | Discussion | 9.12.1 |
| R4-2110614 | other | Discussion on system parameters for NTN | ZTE Corporation | available | Approval | 9.12.1.1 |
| R4-2110688 | discussion | On NTN System parameters | Nokia, Nokia Shanghai Bell | available | Approval | 9.12.1.1 |
| R4-2110413 | other | Discussion on system parameters on NTN system | Huawei, HiSilicon | available | Approval | 9.12.1.1 |
| R4-2111423 | discussion | Reference Deployment Scenario for NTN MSS S-Band | THALES, Hughes/EchoStar, Inmarsat, ESA | available | Discussion | 9.12.1.1 |
| R4-2109053 | discussion | Discussion on NTN System parameters for NTN | CATT | available | Discussion | 9.12.1.1 |
| R4-2109116 | discussion | Open issues for NTN architecture | CATT | available | Discussion | 9.12.1.2 |
| R4-2109504 | discussion | NTN reference point | CMCC | available | Discussion | 9.12.1.2 |
| R4-2111460 | discussion | On the Rx Parameters and Rx Testing Setup for NTN gNB | THALES | available | Discussion | 9.12.1.2 |
| R4-2110120 | other | Reference points and reference model for NTN | Ericsson | available | Approval | 9.12.1.2 |
| R4-2110194 | other | Discussion on RF interfaces for NR to support non-terrestrial networks | Xiaomi | available | Approval | 9.12.1.2 |
| R4-2110615 | other | Discussion on NTN architecture | ZTE Corporation | available | Approval | 9.12.1.2 |
| R4-2110993 | discussion | About Ka-band proposed for NR-NTN in Rel-17 | Hughes/EchoStar, Inmarsat, Thales, Fraunhofer | available | Discussion | 9.12.1.3 |
| R4-2110118 | other | NTN - Regulatory and spectrum aspects | Ericsson | withdrawn | Approval | 9.12.1.3 |
| R4-2109327 | discussion | Band plan and regulatory requirements related to L-band UL and S-band DL operation | GLOBALSTAR Inc. | Available | Approval | 9.12.1.3 |
| R4-2109117 | discussion | On NTN band definition | CATT | Reserved | Discussion | 9.12.1.3 |

Moderator note1: Please note that Proposal 4 from R4-2111423 (“Reference Deployment Scenario for NTN MSS S-Band”) will be further discussed in [99-e][313] NTN_Solutions_Part2 since it concerns discussions for the coexistence analysis and related RAN4 simulation work.

Moderator note2: Please note that Proposal 4 from R4-2111270 (“NTN - On measurement requirements”) from AI 9.12.4.4 ([99-e][229] NR_NTN_solutions_RRM_1) will be further discussed in [99-e][312] NTN_Solutions_Part1 since it has been decided that it concerns discussions related to RF work.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

Identified topics and issues for the 1st round:

- Topic #1: NTN Architecture Aspects

  - Issue 1-1-1: Testing Set-up;

  - Issue 1-1-2: Testing Procedure;

  - Issue 1-1-3: DUT Detailed Description;

  - Issue 1-1-4: Feasibility of REFSENS Test;

  - Issue 1-1-5: Feasibility of Blocking Test;

  - Issue 1-1-6: Feasibility of Intermodulation Test;

  - Issue 1-1-7: RF requirements and conduct test based on Port A;

  - Issue 1-2-1: Service Link RF requirements;

  - Issue 1-3-1: Type of connexion between NTN-Gateway and Non-NTN Infrastructure gNB functions;

  - Issue 1-3-2: NTN Capacity;

  - Issue 1-3-3: Gateway + Satellite as a Repeater;

- Topic #2: NTN Generic Parameters

  - Issue 2-1-1: Frequency Range Definition

  - Issue 2-1-2: BW Configuration

  - Issue 2-1-3: Supported Channel Bandwidth per Operating Band

  - Issue 2-1-4: Channel Spacing/Channel Raster

  - Issue 2-2-1: New TS for RF NTN

  - Issue 2-3-1: Reference assumption for the amplification process

- Topic #3: FR1 Generalities

  - Issue 3-1-1: S-Band Reference Operational Deployment Scenario;

  - Issue 3-1-2: Possible Band Configuration for S-Band with SCS 15 kHz; Note: discussion can be separated from coexistence parameters for [99-e][313] NTN_Solutions_Part2

  - Issue 3-1-3: Possible Band Configuration for S-Band with SCS 30 kHz and SCS 60 kHz; Note: discussion can be separated from coexistence parameters for [99-e][313] NTN_Solutions_Part2

  - Issue 3-1-4: FRF values for S-Band;

  - Issue 3-1-5: S-Band Channel Raster;

  - Issue 3-1-6: S-Band Synchronization Raster;

  - Issue 3-1-7: TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation; Note: discussion can be separated from coexistence parameters for [99-e][313] NTN_Solutions_Part2

  - Issue 3-1-8: S-band Extension; Note: discussion can be separated from coexistence parameters for [99-e][313] NTN_Solutions_Part2

  - Issue 3-2-1: Possible Band Configuration for L-Band; Note: discussion can be separated from coexistence parameters for [99-e][313] NTN_Solutions_Part2

  - Issue 3-2-2: L-Band Channel Raster;

  - Issue 3-3-1: Mixed Pairing – General Discussion;

  - Issue 3-3-2: Potential Mixed Pairing Configuration; Note: discussion can be separated from coexistence parameters for [99-e][313] NTN_Solutions_Part2

  - Issue 3-3-3: Existent Regulation with Respect to Mixed Pairing;

- Topic #4: FR2 Generalities

  - Issue 4-1-1: NR NTN FR2 Introduction to Specification;

  - Issue 4-2-1: General Aspects on Rel-17 NR NTN FR2 Coexistence Studies

  - Issue 4-2-2: Ka-Band Frequency Range for NTN Deployment Scenario in NTN-TN FR2 Coexistence Analysis

  - Issue 4-3-1: NTN SI & WI Scope with respect to VSAT and ESIMs/ESOMPs;

# Topic #1: NTN Architecture Aspects

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109116 | CATT | Proposal 1: It is proposed to consider wired connection between NTN Gate-way and non-NTN infrastructure gNB function as the typical case. Observation: It seems no problem to develop RF requirements and conduct test based on Port A in Figure 2-2 for FR1. Proposal 2: It is proposed to develop RF requirements and conduct test based on Port A in Figure 2-2 for FR1. Figure 2-2 Interface and reference point for FR1 |
| R4-2109504 | CMCC | Observation 1: it is suggested to use the interface between gateway and non-NTN gNB as Rx reference point considering non-NTN gNB is the last component in the Rx RF linkage from UE to gNB. Observation 2: RAN4 could define reference point where there is no interface definition in RAN3 with the assumption that the message format received from satellite gateway has been transferred to match NR message format and could be demodulated. Proposal 1: it is suggested to take satellite + gateway as a single black-box entity without any dedicated Rx requirements and take the interface between gateway and non-NTN gNB as Rx reference point to define/test Rx link RF requirements. |
| R4-2111460 | THALES | Proposal 1: The NTN DUT device for testing Rx NTN gNB parameters shall be composed of the RU NTN, Feederlink Emulator, GW, Non-NTN infrastructure gNB and the required signal generators for 5G NR NTN wanted signal, 5G NR (NTN) interfering signal and CW interfering signal. Proposal 2: Based on different combinations and following similar testing procedures as in e.g. TS 38.141, the throughput measurement point at the Non-NTN infrastructure gNB can be used to determine the 5% throughput loss. Proposal 3: The NTN test setup can use a Feederlink Emulator or not. Proposal 4: The NTN test setup can use a NTN Channel Emulator or not, with Doppler or without Doppler. Observation 1: The quality of the radio at the satellite payload side is the most predominant factor of the Rx tests for the gNB side of the NTN service link. Figure 2. Rx Parameters: Possible REFSENS Test (Alt2) Figure 3. Rx Parameters: Possible Blocking Test Figure 4. Rx Parameters: Possible Intermodulation Test |
| R4-2110120 | Ericsson | Proposal 1: RAN4 should handle gateway + satellite as a repeater and specify needed requirements for gateway + satellite in a new NTN repeater specification. |
| R4-2110194 | Xiaomi | Proposal: it is proposed that defining RF requirements for service link shall be considered as a priority. |
| R4-2110615 | ZTE Corporation | Proposal 1: to discuss what’s functionality expected for gNB and what’s kind of requirement to be tested; Proposal 2: for Case A, at least the baseband capability of gNB to serve the service link should be tested, and the corresponding requirement should be defined at the reference point between gateway and gNB; Proposal 3: for Case B/C, to define reference point between gateway and gNB for the purpose of testing baseband capability of NTN system; |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

RAN4#98-bis-e meeting agreements:

- RAN4 shall consider the architecture defined by RAN3 as baseline and shall allow further potential modifications if required pending further check on test feasibility.

- RAN4 shall consider the architecture defined by RAN3 as baseline for test setup pending on further check on test feasibility.

RAN4#98-bis-e meeting agreements:

- Baseline assumption: The linkage between NTN Gateway and non-NTN gNB is up to implementation and without 3GPP standardized solution

  - Pending on further check on the test feasibility of Rx requirements on gNB side of service link

- RAN4 to decide if there are any testing concerns for Satellite + feeder link + NTN-Gateway + gNB as a single entity, and why.

Sub-topic description: On the Rx Parameter Testing for the Service Link

Moderator note1: Therefore, the goal of this section is to first decide if the tests are feasible or not.

RAN4#98-bis-e R4-2106147:

| Company | Rx parameter on the service link for (Satellite + feeder link + NTN-Gateway + gNB) as a single entity | Expressed Testing Concern (or other comment) |
| --- | --- | --- |
| Ericsson | REFSENS being the basis for many other Rx requirements, we would like to understand how the following 3 requirements would be tested (test set up + test procedure) : REFSENS Blocking Rx intermodulation |  |
| Nokia | Similar comment as Ericsson on Rx parameters. Also, even if this is seen as a single entity, requirements will need to be defined to ensure aspects as timings. We should emphasize that this design, illustrated in issue 1-3, is targeted RAN1/2 and has a note explicit highlighting this limitation in the RAN3 CR. |  |
| Inmarsat |  | No concern expressed. We need more details from the concerned companies regarding the specific issues. REFSENS for example in our view does not present any specific issues if the whole (Satellite + feeder + NTNGW + gNB) is considered as a black box. Assuming the feeder link is left to implementation, the collective losses remaining can be calculated and subsumed into a single IL value, while the rest can follow same principles as for terrestrial gNB. It’s also unclear how timing concerns apply, since the satellite + feeder + NTNGW are transparent elements in the RF link path. Please provide further details on the issues, if any. |
| THALES |  | Only a comment: As already explained in the GTW session, we have to be careful when describing the architecture (and this is the reason for which RAN3 architecture comes back many times) since there might be a capacity issue if NTN architecture considers (Satellite+Feederlink+GateWay) as a single entity (Repeater). At least for the satellite case, the number of beams may be very high, and this would complicate a lot the connectivity at the GW side with the Non-NTN infrastructure. |

Moderator note2: Further, in R4-2106147 (Email discussion summary for [98-bis-e][307] NTN_Solutions_Part1), a short list of Rx parameters with testing concerns are available. The moderator also asked explanations for why testing could not be feasible.

Open issues and candidate options before e-meeting:

Issue 1-1-1: Testing Set-up

- Proposals

  - Option 1: It is proposed to develop RF requirements and conduct test based on Port A

- Recommended WF

  - Develop RF requirements and conduct test based on Port A

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Disagree | To take any position on this proposal, we need first to make sure we all have the same definition of port A. According to TS 36.104/38.104, an antenna connector (port A) is the connector at the conducted interface of BS type 1-C. The requirements applied at this BS antenna connector (port A) for a single transmitter or receiver. And, in case of multi-carrier transmission with one or multiple transmitter antenna connectors, the requirements apply for each transmitter antenna connector. So, for receiver requirements, when BS is configured to receive multiple carriers, all the throughput requirements are applicable for each received carrier. So, if we adopt this proposal, this would means a satellite will have those antenna connectors accessible for testing, which would also mean a considerable increase of size and weigth…. We think a type 1-O BS would be more relevant for NTN but, of course, welcome satellite’s manufacturer to comment further and give their inputs. |
| CATT | Agree | To avoid confusion with Port A defined in 38.104/36.104, it can be named differently for this port. Whether antenna connector can be accessible or not will highly determine whether we use BS type 1-C or 1-O. Open for further discussion. Especially satellite manufacturer and operator inputs are expected. |
| Huawei | Agree partially | It seems that we discuss this topic in both thread 312 and 314. Maybe the diagram can be further improved. |
| Qualcomm | Agree | The proposal aligns with the baseline assumptions from RAN4#98bis-e. We are open to further discuss the BS type. |
| Hughes/EchoStar | Agree | The proposal aligns with the baseline assumptions from RAN4#98bis-e. |
| Nokia | Agree partially | This can only be agreed if the NTN payload + feeder link do not introduce any inaccuracies, as at the gNB it needs to be the same, so this conditions should be added. |
| Intelsat | Agree | The proposal aligned with the baseline assumptions from RAN4#98bis-e |
| Xiaomi | Agree | Agree with the assumption. |
| THALES | Agree | Very interesting discussion. Actually, we think that NTN should consider all BS types in FR1, for the NTN payload: 1-C, 1-H, 1-O. However, in AIT phase (i.e. during Assembly, Integration and Test), the NTN-Payload is usually tested in conducted mode, meaning that the (radiant) antenna elements are disconnected before connecting the test equipment. We could therefore start with 1-C, 1-H, but there should (probably) not be any restriction for 1-O. In any case, we should treat with priority conducted mode testing. However, very important, RAN4 should reconsider/adapt the reference measurement channels for NTN. As also explained in our contribution R4-2111460, the proposed testing procedures/test setups may have several alternatives (with different channel models, with or without feeder link emulator, with wired connections or OTA, etc.). We only wanted to show that Rx testing procedures are feasible and in different conditions. |
| ZTE |  | Even if this is cabled connected between gateway and gNB, then it’s still necessary to test Rx demod performance, if putting all the components within the chamber, it might be a bit challenging, instead if tested by reference point between gateway and gNB, it might be beneficial for conformance testing. |
| Inmarsat | Agree | The proposal aligns with the baseline assumptions from RAN4#98bis-e. BS type could be created to capture key differences with TN BS |

It seems that there is not a strong disagreement.

Therefore, the moderator suggests the following proposals for the WF:

Proposal 1-1-1-1: RAN4 should first consider conducted mode testing (e.g. 1-C, 1-H) for NTN gNB. 1-O can be considered in a later stage.

Note1: in AIT phase (i.e. during Assembly, Integration and Test), the NTN-Payload is usually tested in conducted mode, meaning that the (radiant) antenna elements are disconnected before connecting the test equipment. RAN4 could therefore start with 1-C and/or 1-H, but there should (probably) not be any restriction for 1-O.

Note2: For NTN gNB please refer to RAN3 NTN architecture.

Proposal 1-1-1-2: RAN4 should reconsider/adapt the reference measurement channels for NTN use case.

Issue 1-1-2: Testing Procedure

- Proposals

  - Option 1: Based on different combinations and following similar testing procedures as in e.g. TS 38.141-like, the throughput measurement point at the Non-NTN infrastructure gNB can be used to determine the 5% throughput loss.

- Recommended WF

  - Similar testing procedures as in 38.141-like. Based on different combinations and following similar testing procedures as in e.g. TS 38.141-like, the throughput measurement point at the Non-NTN infrastructure gNB can be used to determine the 5% throughput loss.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Agree | Actually, all Rx requirements are based on throughput measurement and this can only be done at the “Non-NTN infrastructure gNB” with a transparent architecture. |
| CATT | Agree |  |
| Huawei | Agree |  |
| Hughes/EchoStar | Agree |  |
| Nokia | Agree |  |
| Intelsat | Agree |  |
| Xiaomi | Agree |  |
| THALES | Agree | We are happy to see that all companies agree with this assumption. |
| ZTE | Agree |  |
| Inmarsat | Agree |  |

It seems that there is a complete agreement for the WF and for the Option 1.

Therefore, the moderator suggests the following proposals for the WF:

Proposal 1-1-2-1: RAN4 shall consider for NTN framework similar testing procedures as in TS 38.141-like. Based on different combinations and following similar testing procedures as in e.g. TS 38.141-like, the throughput measurement point at the Non-NTN infrastructure gNB can be used to determine the 5% throughput loss.

Note1: For the Non-NTN infrastructure gNB, please refer to RAN3 NTN architecture.

Issue 1-1-3: DUT Detailed Description

- Proposals

  - Option 1: The NTN DUT device for testing Rx NTN gNB parameters shall be composed of the RU NTN, (potentially) Feederlink Emulator, GW, Non-NTN infrastructure gNB and the required signal generators for 5G NR NTN wanted signal, 5G NR (NTN) interfering signal and CW interfering signal.

- Recommended WF

  - The NTN DUT device for testing Rx NTN gNB parameters shall be composed of the RU NTN, (potentially) Feederlink Emulator, GW, Non-NTN infrastructure gNB and the required signal generators for 5G NR NTN wanted signal, 5G NR (NTN) interfering signal and CW interfering signal.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Disagree for the time being | More detailed description is needed. What is the “RU NTN”? A clear detailed definition would be needed here. Also, what is the interface to RU NTN, is it air interface or antenna connector? Is the feederlink emulator a certified product? What is the feederlink+GW emulator and is it a certified product? The signal generators are not part of the NTN DUT, they should come from TE vendors. |
| CATT | Agree in principle. | Generally is ok. Further revisit on details is not precluded. |
| Huawei | Agree partially | Maybe it's a feasible approach to test the satellite + GW together. If this approach is usually used for satellite vendor or some organizations, that's very helpful. However, some details can be further discussed in the future. More input are needed. |
| Nokia |  | Further study is needed. |
| Intelsat | Agree |  |
| THALES | Agree | As also explained in our contribution R4-2111460, the proposed testing procedures/test setups may have several alternatives (with different channel models, with or without feeder link emulator, with wired connections or OTA, etc.). We only wanted to show that testing is feasible and in different conditions. Instead of RU NTN we may consider NTN-Payload RF. Also we don’t need to consider feederlink emulator as certified product. Actually the NTN-Payload RF can be directly connected to the GW. All alternatives are possible. However, very important, RAN4 should reconsider/adapt the reference measurement channels for NTN. |
| ZTE |  | Whether it’s conducted or OTA for reference point should be clarified. |
| Inmarsat | Agree partially | The approach is fine but WF proposal may need to be expanded with more details. It must be noted that satellites can vary a lot from one another, depending on orbit, constellation, architecture. Same holds for feeder link and GW antennas. The system should be considered in its entirety as a black box for now. |

It seems that there might be some partial agreement if WF is adapted accordingly. Therefore, after removing “RU NTN” the moderator suggests the following proposals for the WF:

Proposal 1-1-3-1: The NTN DUT device for testing Rx NTN gNB parameters shall be composed of the NTN-Payload RF, (potentially) Feederlink Emulator, GW, Non-NTN infrastructure gNB and the required signal generators for 5G NR NTN wanted signal, 5G NR (NTN) interfering signal and CW interfering signal.

Note1: For the Non-NTN infrastructure gNB, please refer to RAN3 NTN architecture.

Or alternatively:

Proposal 1-1-3-1: The NTN DUT device for testing Rx NTN gNB parameters shall be composed of the NTN-Payload RF, GW, Non-NTN infrastructure gNB and the required signal generators for 5G NR NTN wanted signal, 5G NR (NTN) interfering signal and CW interfering signal.

Note1: For the Non-NTN infrastructure gNB, please refer to RAN3 NTN architecture.

Issue 1-1-4: Feasibility of REFSENS Test

- Proposals

  - Option 1: REFSENS test is feasible

- Recommended WF

  - REFSENS test is feasible for (Satellite + feeder link + NTN-Gateway + Non-NTN infrastructure gNB) as a single entity

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Disagree for the time being | We understand there was very little time to collect all needed information but we can’t make any decision on such basic figure, additional information is required. Same questions as above for 1-1-3. Again, the interface between RU NTN and the NTN channel emulator shall be clarified: is it air interface? Or is it antenna connectors (which means the satellite antenna is not included in the RU NTN)? From the figure, it seems only antenna connector(s) is considered for RU NTN. Does that means there won’t be any product with integrated antenna? This would look strange to us as conducted products are always more builky, which is not expected for satellite. It would be very helpful to get more detailed description on how the antenna is integrated in a satellite (some product description and/or photos would be welcome). UE should not be part of this picture, we should have a signal generator instead. |
| CATT |  | Need to clarify whether taking BS type 1-C or BS type 1-O as the starting point. Issue 1-1-1 needs to be clarified at first. |
| Nokia |  | Further study is needed |
| THALES | Agree | As also explained in our contribution R4-2111460, the proposed testing procedures/test setups may have several alternatives (with different channel models, with or without feeder link emulator, with wired connections/conducted testing or OTA testing, etc.). We only wanted to show that testing is feasible and in different conditions. As explained in R4-2111460 we can have a signal generator instead of the UE. Instead of RU NTN we may consider NTN-Payload RF. Also we don’t need to consider feederlink emulator as certified product. Actually the NTN-Payload RF can be directly connected to the GW. All alternatives are possible. All these are very small details, we wanted to capture as much as possible (please see for example the other tests). There are many possible alternatives. We only wanted to show that testing is feasible and in different conditions, because in RAN4#98bis-e we were told by some companies that some of Rx (satellite) testing procedures are difficult to handle, and there were some expressed concerns. We can assume starting with BS type 1-C or BS type 1-H. BS type 1-O can be consider later on. However, very important, RAN4 should reconsider/adapt the reference measurement channels for NTN. |
| ZTE |  | Similar as previous comments. |
| Inmarsat | Agree | We agree that REFSENS test is feasible. Further details can be discussed. |

It seems that there might be some partial agreement if WF is adapted accordingly. Therefore, after adding “NTN-Payload RF” and “Signal Generator for the wanted signal” to remove any misunderstanding, the moderator suggests the following proposals for the WF:

Proposal 1-1-4-1: REFSENS test is feasible for (Satellite + feeder link + NTN-Gateway + Non-NTN infrastructure gNB) as a single entity.

Note1: For the Non-NTN infrastructure gNB, please refer to RAN3 NTN architecture.

Proposal 1-1-4-2: REFSENS test can be performed in conducted mode, by considering BS type 1-C and/or 1-H. 1-O can be considered FFS for the time being.

Issue 1-1-5: Feasibility of Blocking Test

- Proposals

  - Option 1: Blocking test is feasible

- Recommended WF

  - Blocking test is feasible for (Satellite + feeder link + NTN-Gateway + Non-NTN infrastructure gNB) as a single entity

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Disagree | Same comments as for 1-1-4 |
| CATT | Disagree | We are not sure throughput can be measured at non-NTN infrastructure gNB. The interface between NTN-Gateway and non-NTN infrastructure gNB is non-standard. The connection can be a RF connection, fiber or wireless, it’s unclear how to specify throughput at this point. |
| Nokia |  | Further study is needed |
| THALES | Agree | As also explained in our contribution R4-2111460, the proposed testing procedures/test setups may have several alternatives (with different channel models, with or without feeder link emulator, with wired connections/conducted testing or OTA testing, etc.). We only wanted to show that Rx testing is feasible and in different conditions. As explained in R4-2111460 we can have a signal generator instead of the UE. There are very small details, we wanted to capture as much as possible (please see for example the other tests). There are many possible alternatives. We only wanted to show that testing is feasible and in different conditions, because in RAN4#98bis-e we were told by some companies that some of Rx (satellite) testing procedures are difficult to handle, and there were some expressed concerns. The raised points are very small details. We don’t understand what is still unclear. We can assume starting with BS type 1-C or BS type 1-H. BS type 1-O can be consider later on. The figure has been also updated with NTN-Payload RF: |
| ZTE |  | It;s better to include any kind of connections method. |
| Inmarsat | Agree | Question for the companies that disagree: if we take the gNB as a single black box, why do we need to specify internal interfaces? What if we consider that the NTN part is an RRH/RRU? |

It seems that there might be some partial agreement if WF is adapted accordingly. Therefore, after adding “NTN-Payload RF” in order to remove any misunderstanding, the moderator suggests the following proposals for the WF:

Proposal 1-1-5-1: Blocking test is feasible for (Satellite + feeder link + NTN-Gateway + Non-NTN infrastructure gNB) as a single entity

Note1: For the Non-NTN infrastructure gNB, please refer to RAN3 NTN architecture.

Proposal 1-1-5-2: Blocking test can be performed in conducted mode, by considering BS type 1-C and/or 1-H. 1-O can be considered FFS for the time being.

Issue 1-1-6: Feasibility of Intermodulation Test

- Proposals

  - Option 1: Intermodulation test is feasible

- Recommended WF

  - Intermodulation test is feasible for (Satellite + feeder link + NTN-Gateway + Non-NTN infrastructure gNB) as a single entity

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Disagree | Same comments as for 1-1-4 |
| CATT | Disagree | Same comments as for 1-1-5 |
| Nokia |  | Further study is needed |
| THALES | Agree | As also explained in our contribution R4-2111460, the proposed testing procedures/test setups may have several alternatives (with different channel models, with or without feeder link emulator, with wired connections/conducted testing or OTA testing, etc.). We only wanted to show that Rx testing is feasible and in different conditions. As explained in R4-2111460 we can have a signal generator instead of the UE. There are very small details, we wanted to capture as much as possible (please see for example the other tests). There are many possible alternatives. We only wanted to show that testing is feasible and in different conditions, because in RAN4#98bis-e we were told by some companies that some of Rx (satellite) testing procedures are difficult to handle, and there were some expressed concerns. The raised points are very small details. We don’t understand what is still unclear. We can assume starting with BS type 1-C or BS type 1-H. BS type 1-O can be consider later on. The figure has been also updated with NTN-Payload RF: Basically, DUT is to be tested as a NTN NR gNB/BS (according to RAN3 architecture). |
| ZTE |  | Similar comment as Ericsson, it’s welcome to have commercial product on NNT antenna |
| Inmarsat | Agree | Same comment as above. |

It seems that there might be some partial agreement if WF is adapted accordingly. Therefore, after adding “NTN-Payload RF” in order to remove any misunderstanding, the moderator suggests the following proposals for the WF:

Proposal 1-1-6-1: Intermodulation test is feasible for (Satellite + feeder link + NTN-Gateway + Non-NTN infrastructure gNB) as a single entity.

Note1: For the Non-NTN infrastructure gNB, please refer to RAN3 NTN architecture.

Proposal 1-1-6-2: Intermodulation test can be performed in conducted mode, by considering BS type 1-C and/or 1-H. 1-O can be considered FFS for the time being.

Issue 1-1-7: RF requirements and conduct test based on Port A

- Proposals

  - Option 1: No problem to develop RF requirements and conduct test based on Port A for FR1.

- Recommended WF

  - There is no longer a concern with respect to developing RF requirements and conduct test based on Port A

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Disagree | See our previous comments. We don’t think 1-C is the most relevant BS type for NTN, but we would strongly welcome more information from satellite companies as asked precisely in the previous issues. |
| CATT | Agree | If type 1-C is agreed as the starting point. |
| Qualcomm | Agree |  |
| Hughes/EchoStar | Agree |  |
| Nokia | Disagree | We do not think all the concern have been mitigated since so many aspects are still open and discussed. |
| Intelsat | Agree |  |
| THALES | Agree | As also explained in our contribution R4-2111460, the proposed testing procedures/test setups may have several alternatives (with different channel models, with or without feeder link emulator, with wired connections/conducted testing or OTA testing, etc.). We only wanted to show that Rx testing is feasible and in different conditions. As explained in R4-2111460 we can have a signal generator instead of the UE. There are very small details, we wanted to capture as much as possible (please see for example the other tests). There are many possible alternatives. We only wanted to show that testing is feasible and in different conditions, because in RAN4#98bis-e we were told by some companies that some of Rx (satellite) testing procedures are difficult to handle, and there were some expressed concerns. The raised points are very small details. We don’t understand what is still unclear. We can assume starting with BS type 1-C or BS type 1-H. BS type 1-O can be consider later on. Basically, DUT is to be tested as a NTN NR gNB/BS (according to RAN3 architecture), and similarly as to TN NR gNB. However, very important, RAN4 should reconsider/adapt the reference measurement channels for NTN |
| Inmarsat | Agree | If there are still concerns, we should discuss the details further, but we don’t have a specific concern. We may need to define a different BS type. |

It seems that there might be some partial agreement if WF is adapted accordingly. The moderator therefore suggests the following proposals for the WF:

Proposal 1-1-7-1: Evidence has been provided to show that Rx testing is possible when considering (NTN-Payload + feeder link + NTN-Gateway + Non-NTN infrastructure gNB) as a single entity.

Proposal 1-1-7-2: There is no longer a concern with respect to develop RF requirements and conduct Rx tests when considering (NTN-Payload + feeder link + NTN-Gateway + Non-NTN infrastructure gNB) as a single entity.

Proposal 1-1-7-3: In a first step, RAN4 can assume BS type 1-C or BS type 1-H for NTN gNB tests. BS type 1-O type can be consider later on for OTA NTN tests.

### Sub-topic 1-2

RAN4#98-e Agreements:

- RAN4 shall define the corresponding RF requirements for service link between UE and satellite

Sub-topic description Service Link Assumptions

Open issues and candidate options before e-meeting:

Issue 1-2-1: Service Link RF requirements

- Proposals

  - Option 1: Defining RF requirements for service link shall be considered as a priority.

- Recommended WF

  - Defining RF requirements for service link shall be considered as a priority.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Disagree | Priority comparing to what? Better to keep RAN4#98-e agreement |
| CATT | Partly agree | What does it mean by “considered as a priority” ? Is the intention to define requirement for other interface except service link? |
| Huawei |  | There is no need to review previous agreement since the time is limited. |
| Qualcomm | Partly agree | Share similar concerns as CATT. |
| Hughes/EchoStar | Agree | This the core requirement of RAN4 |
| Nokia | Disagree | We should not change previous agreements |
| Intelsat | Agree |  |
| THALES | Agree | Focus on service link. We can also keep RAN4#98-e agreement |
| ZTE |  | There are no much meaning to have such prority since coexistence work is already done for service link. |
| Inmarsat | Partially agree | New wording: RAN4 shall define RF requirement only for the service link |

After receiving all comments, the moderator suggests the following proposals for the WF:

Proposal 1-2-1-1: RAN4 shall define RF requirement only for the service link.

### Sub-topic 1-3

Sub-topic description Connectivity between NTN-GW and Non-NTN infrastructure gNB

RAN4#98-e Agreements:

- FFS whether RAN4 shall define RF requirements for the linkage between NTN-Gateway and gNB

  - Companies are encouraged to further clarify and discuss the assumption of the linkage between NTN-Gateway and gNB

Open issues and candidate options before e-meeting:

Issue 1-3-1: Type of connexion between NTN-Gateway and Non-NTN Infrastructure gNB functions

- Proposals

  - Option 1: The linkage between NTN-Gateway and Non-NTN Infrastructure gNB functions is expected to be typically implemented with a non-RF connection

  - Option 2: The linkage between NTN-Gateway and Non-NTN Infrastructure gNB functions is expected to be typically implemented with a RF connection

- Recommended WF

  - TBA

Question: Which option (listed above) do you prefer? Please provide your answer(s) e.g. “Yes” or “No”.

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 | Comments Option 2 |
| --- | --- | --- |
| CATT | Both are potentials |  |
| Huawei | Yes |  |
| Qualcomm | If option 1 is confirmed as the typical implementation, we can deprioritize option 2. | If option 1 is confirmed as the typical implementation, we can deprioritize option 2. |
| Hughes/EchoStar | Yes | Yes |
| Nokia | We are fine to use this assumption for satellite based deployments. |  |
| Intelsat | Yes | Yes |
| THALES | Basically Yes, for capacity reasons. This is the typical implementation. | Option 2 could be deprioritized. |
| Inmarsat | Yes | Yes, but no requirement definition is expected. This is left to implementation and the whole Satellte + feeder link + GW + gNB should be considered as a single entity. |

After receiving all comments, the moderator suggests the following proposals for the WF:

Proposal 1-3-1-1: The linkage between NTN-Gateway and Non-NTN Infrastructure gNB functions is expected to be typically implemented with a non-RF connection.

Issue 1-3-2: NTN Capacity

Moderator Note1: As already explained in the GTW session from RAN4#98-bis-e, we have to be careful when describing the architecture (and this is the reason for which RAN3 architecture comes back many times) since there might be a capacity issue if NTN architecture considers (Satellite+Feederlink+GateWay) as a single entity (Repeater). At least for the satellite case, the number of beams may be very high, and this would complicate a lot the connectivity at the GW side with the Non-NTN infrastructure.

- Proposals

  - Option 1: Expressed concerns with NTN capacity issue if (Satellite+Feederlink+GW) as a Repeater.

  - Option 2: No expressed concerns with NTN capacity issue if (Satellite+Feederlink+GW) as a Repeater.

- Recommended WF

  - TBA

Question: Which option (listed above) do you prefer? Please provide your answer(s) e.g. “Yes” or “No”.

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 | Comments Option 2 |
| --- | --- | --- |
| CATT | Actually it’s unclear how to consider it as repeater since the interface between Gateway and ground gNB is non-standard. |  |
| Huawei | Yes |  |
| Qualcomm | Yes | If tread Satellite+Feederlink+GW as a Repeater, there would be no network timing adjustment at all. |
| Hughes/EchoStar | Yes |  |
| THALES | Yes, expressed concerns because the Repeater will have to handle thousands of connections between GW and Non-NTN Infrastructure gNB. Moreover, if Satellite+Feederlink+GW as repeater, also expressed timing adjustments concerns (non-deterministic timing errors because of the RF interface with the Non-NTN Infrastructure gNB) |  |
| Inmarsat | Yes | No, it’s not possible to consider Satellite+feederlink+GW as a Repeater because it has problematic consequences on the satellite payload architecture. |

After receiving all comments, the moderator suggests the following proposals for the WF:

Proposal 1-3-2-1: RAN4 acknowledges received expressed concerns with NTN capacity if (Satellite+Feederlink+GW) is considered as a Repeater.

Issue 1-3-3: Gateway + Satellite as a Repeater

Moderator Note: The moderator kindly reminds the previous agreements:

RAN4#98-bis-e meeting agreements:

- Baseline assumption: The linkage between NTN Gateway and non-NTN gNB is up to implementation and without 3GPP standardized solution

  - Pending on further check on the test feasibility of Rx requirements on gNB side of service link

- RAN4 to decide if there are any testing concerns for Satellite + feeder link + NTN-Gateway + gNB as a single entity, and why.

RAN4#98-bis-e meeting agreements:

- RAN4 shall consider the architecture defined by RAN3 as baseline and shall allow further potential modifications if required pending further check on test feasibility.

- RAN4 shall consider the architecture defined by RAN3 as baseline for test setup pending on further check on test feasibility.

Therefore, according to RAN4#98e-bis decisions, Repeater discussions should continue only if there are testing concerns for Satellite + feeder link + NTN-Gateway + gNB as a single entity.

- Proposals

  - Option 1: Postpone discussion to another meeting, and only if Rx testing with satellite RF is not possible.

  - Option 2: RAN4 should handle gateway + satellite as a repeater and specify needed requirements for gateway + satellite in a new NTN repeater specification.

  - Option 3: Take satellite + gateway as a single black-box entity without any dedicated Rx requirements and take the interface between gateway and non-NTN gNB as Rx reference point to define/test Rx link RF requirements.

- Recommended WF

  - TBA

Question: Which option (listed above) do you prefer? Please provide your answer(s) e.g. “Yes” or “No”.

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 | Comments Option 2 | Comments Option 3 |
| --- | --- | --- | --- |
| Ericsson | No, it’s difficult to postposne the discussion as it’s a key one to specify requirements | Yes | This seems to be a new proposal, was it mentioned in any tdoc? It would need more detailed description to better understand pros and cons. |
| CATT | No | No | No It is related to the discussion for previous question. |
| Huawei | Not sure we can discuss the Rx test very specifically. | No | No. Since we have no requirements for feederlink. |
| Qualcomm | No We should follow the baseline assumption from RAN4#98bis-e meeting | No | No Option 3 is a superset of option 2? It is not clear how to define requirements. It will make the Rx requirements more complicated. |
| Hughes/EchoStar | No. To agree that Satellite + feeder link + NTN-Gateway + gNB as a single entity consistent with RAN3 | No | No |
| Nokia | No | This might be possible | No |
| Intelsat | No | No | No |
| Xiaomi | No | No | No |
| THALES | Please note that the Issue 1-3-3 concerns “Gateway + Satellite as a Repeater” It might be some misunderstanding. Actually our TDOC R4-2111460 shows that testing Rx parameters at NTN Payload side is possible. So we think that we should postpone (Gateway + Satellite) as a Repeater discussion because testing (Satellite+GW+Non-NTN infrastructure) as a single entity does not have any concerns. We can also continue to use RAN4#98bis-e agreement, of course. | No | No |
| Inmarsat | No | No | No |

The moderator suggest to continue using continue to use RAN4#98bis-e agreement. No tentative proposal is made.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Please see above |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1-1: Testing Set-up; | Tentative agreements: Proposal 1-1-1-1: RAN4 should first consider conducted mode testing (e.g. 1-C, 1-H) for NTN gNB. 1-O can be considered in a later stage. Note1: in AIT phase (i.e. during Assembly, Integration and Test), the NTN-Payload is usually tested in conducted mode, meaning that the (radiant) antenna elements are disconnected before connecting the test equipment. RAN4 could therefore start with 1-C and/or 1-H, but there should (probably) not be any restriction for 1-O. Note2: For NTN gNB please refer to RAN3 NTN architecture. Proposal 1-1-1-2: RAN4 should reconsider/adapt the reference measurement channels for NTN use case. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 1-1-2: Testing Procedure; | Tentative agreements: Proposal 1-1-2-1: RAN4 shall consider for NTN framework similar testing procedures as in TS 38.141-like. Based on different combinations and following similar testing procedures as in e.g. TS 38.141-like, the throughput measurement point at the Non-NTN infrastructure gNB can be used to determine the 5% throughput loss. Note1: For the Non-NTN infrastructure gNB, please refer to RAN3 NTN architecture. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 1-1-3: DUT Detailed Description; | Tentative agreements: Proposal 1-1-3-1: The NTN DUT device for testing Rx NTN gNB parameters shall be composed of the NTN-Payload RF, (potentially) Feederlink Emulator, GW, Non-NTN infrastructure gNB and the required signal generators for 5G NR NTN wanted signal, 5G NR (NTN) interfering signal and CW interfering signal. Note1: For the Non-NTN infrastructure gNB, please refer to RAN3 NTN architecture. Or alternatively: Proposal 1-1-3-1: The NTN DUT device for testing Rx NTN gNB parameters shall be composed of the NTN-Payload RF, GW, Non-NTN infrastructure gNB and the required signal generators for 5G NR NTN wanted signal, 5G NR (NTN) interfering signal and CW interfering signal. Note1: For the Non-NTN infrastructure gNB, please refer to RAN3 NTN architecture. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 1-1-4: Feasibility of REFSENS Test; | Tentative agreements: Proposal 1-1-4-1: REFSENS test is feasible for (Satellite + feeder link + NTN-Gateway + Non-NTN infrastructure gNB) as a single entity. Note1: For the Non-NTN infrastructure gNB, please refer to RAN3 NTN architecture. Proposal 1-1-4-2: REFSENS test can be performed in conducted mode, by considering BS type 1-C and/or 1-H. 1-O can be considered FFS for the time being. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 1-1-5: Feasibility of Blocking Test; | Tentative agreements: Proposal 1-1-5-1: Blocking test is feasible for (Satellite + feeder link + NTN-Gateway + Non-NTN infrastructure gNB) as a single entity Note1: For the Non-NTN infrastructure gNB, please refer to RAN3 NTN architecture. Proposal 1-1-5-2: Blocking test can be performed in conducted mode, by considering BS type 1-C and/or 1-H. 1-O can be considered FFS for the time being. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 1-1-6: Feasibility of Intermodulation Test; | Tentative agreements: Proposal 1-1-6-1: Intermodulation test is feasible for (Satellite + feeder link + NTN-Gateway + Non-NTN infrastructure gNB) as a single entity. Note1: For the Non-NTN infrastructure gNB, please refer to RAN3 NTN architecture. Proposal 1-1-6-2: Intermodulation test can be performed in conducted mode, by considering BS type 1-C and/or 1-H. 1-O can be considered FFS for the time being. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 1-1-7: RF requirements and conduct test based on Port A; | Tentative agreements: Proposal 1-1-7-1: Evidence has been provided to show that Rx testing is possible when considering (NTN-Payload + feeder link + NTN-Gateway + Non-NTN infrastructure gNB) as a single entity. Proposal 1-1-7-2: There is no longer a concern with respect to develop RF requirements and conduct Rx tests when considering (NTN-Payload + feeder link + NTN-Gateway + Non-NTN infrastructure gNB) as a single entity. Proposal 1-1-7-3: In a first step, RAN4 can assume BS type 1-C or BS type 1-H for NTN gNB tests. BS type 1-O type can be consider later on for OTA NTN tests. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 1-2-1: Service Link RF requirements; | Tentative agreements: Proposal 1-2-1-1: RAN4 shall define RF requirement only for the service link. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 1-3-1: Type of connexion between NTN-Gateway and Non-NTN Infrastructure gNB functions; | Tentative agreements: Proposal 1-3-1-1: The linkage between NTN-Gateway and Non-NTN Infrastructure gNB functions is expected to be typically implemented with a non-RF connection. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 1-3-2: NTN Capacity; | Tentative agreements: Proposal 1-3-2-1: RAN4 acknowledges received expressed concerns with NTN capacity if (Satellite+Feederlink+GW) is considered as a Repeater. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 1-3-3: Gateway + Satellite as a Repeater; | Tentative agreements:- Candidate options:- Recommendations for 2nd round: The moderator suggest to continue using continue to use RAN4#98bis-e agreement. No tentative proposal is made. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #2: Generic Parameters

Main technical topic overview. The structure can be done based on sub-agenda basis.

Moderator note: This section is applicable to both FR1 and FR2.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109053 | CATT | Proposal 1: It is proposed to develop a separate set of RF specifications for NTN, including BS RF and UE RF. Proposal 2: The frequency range definition should be aligned with NR. Proposal 3: The same set of band coding and signaling design should be used for NTN and NR. The NTN band is numbered in reverse order from the maximum NR band number in each FR. Proposal 4: The common definition for channel bandwidth, transmission bandwidth configuration, minimum guard band, and RB alignment in 38.104 can be reused for NTN system. Proposal 5: The supported channel bandwidth per operating band should be defined based on operator input. Proposal 6: The channel spacing in 38.104 can be reused for NTN. - Exact definition pending channel raster decision. |
| R4-2109504 | CMCC | Proposal 1: it is suggested to take satellite + gateway as a single black-box entity without any dedicated Rx requirements and take the interface between gateway and non-NTN gNB as Rx reference point to define/test Rx link RF requirements. |
| R4-2110120 | Ericsson | Proposal 1: RAN4 should handle gateway + satellite as a repeater and specify needed requirements for gateway + satellite in a new NTN repeater specification. |
| R4-2111270 | Nokia, Nokia Shanghai Bell | The amplification may be based on (TR 38.821): - Constant gain. The receive and transmit gains are constant i.e. independent of the input signal to the amplifier. - Constant Equivalent Isotropic Radiated Power (EIRP). The receive and transmit gains of the satellite are adjusted based on the received signal to obtain a constant target EIRP. - Constant power at receiver. The satellite may attempt to compensate the loss of the radio channel to ensure constant power at the NTN-GW, i.e. feeder link equalization. - Constant power spectral density. The satellite may amplify the received signal to achieve the same power power in all subcarriers. Observation 3 : For transparent satellite payload, several different types of on-board satellite amplification procedure are possible Observation 4: The transparent satellite amplification type impacts UE and network interpretation of measurements. Proposal 3 : RAN4 to study and agree on the reference assumption to be used for the on-board satellite amplification process. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description: Frequency and Channel Generic Parameters

Open issues and candidate options before e-meeting:

Issue 2-1-1: Frequency Range Definition

- Proposals

  - Option 1: The frequency range definition should be aligned with NR.

- Recommended WF

  - TBA

Question: Do you partially agree/disagree with the Option 1 stated above? Please provide your views on the Option 1 stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Disagree | There is no good reason to align NTN bands with NR bands, they might have different frequency range. For example, if we take the S-band which is currently considered for NTN, it covers only the upper 30 MHz of band n65. It would not make sense to specify a new NTN band with n65 frequency range, this would very confusing. |
| CATT | Agree | Some clarification, it is not about a specific band, e.g. Band 65. It is about FR1 and FR2 definition. |
| Huawei | Disagree | It is too general. The regulatory should be considered. |
| Qualcomm | Disagree | It is not necessary to align NTN band and NR band. The frequency for NTN depends on the service of the spectrum. A clarification question: is it possible that NTN and NR work in the same band/frequency range? |
| Hughes/EchoStar | Partially agree | NTN identified band/range should be specified where it should be |
| Nokia |  | The proposal is not fully understood. NTN band should be defined on the basis of regulatory inputs and 3GPP specification support. |
| Intelsat | Disagree | The use of the term ‘aligned’ is ambiguous. NTN has its own requirements and use cases and should not be constrained by NR. |
| Xiaomi |  | It is not clear what does “The frequency range definition” mean. |
| Samsung | Partially Agree | The frequency ranges, let’s say FR1 and FR2, is somehow classified according to the different method to be applied for testing, namely Conducted or OTA. If the same rational can be applied to satellite equipment in different frequency ranges, it seems fine to reuse the frequency ranges. |
| THALES | Partially Agree | In some cases, at least for FR1 MSS bands, it might be useful. For FR2, not necessary. |
| ZTE | Disagree | Different band would be used for NTN system. |
| Apple | Partially agree | Frequency bands are not necessarily aligned with existing NR bands and can have completely different frequency ranges or combinations thereof. If the question is about the frequency range, then NTN bands shall be of course aligned with existing FR1 and FR2 definitions. |
| Inmarsat | Disagree | There is no reason for this to happen. If anything the contrary, we may need to adjust the frequency range definition to address the differences in NTN. NTN has different requirements. |

The proposal is considered to be too general. Moderator suggests the potential WF:

Proposal 2-1-1-1: The frequency range definition for MSS S-band should be aligned with NR.

Issue 2-1-2: BW Configuration

- Proposals

  - Option 1: The common definition for channel bandwidth, transmission bandwidth configuration, minimum guard band, and RB alignment in 38.104 can be reused for NTN system.

- Recommended WF

  - TBA

Question: Do you partially agree/disagree with the Option 1 stated above? Please provide your views on the Option 1 stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Agree | If satellite companies consider the specified SU is feasible, this would be fine to us. |
| CATT | Agree |  |
| Huawei | Agree |  |
| Qualcomm | Agree |  |
| Hughes/EchoStar | Agree |  |
| Intelsat | Agree |  |
| Xiaomi | Agree |  |
| Samsung | Agree |  |
| THALES | Agree |  |
| ZTE |  | Could start with that, however it should rely on the final RF requirement |
| Apple | Agree | As a side remark, why do we refer only to TS 38.104 in the proposed Option 1? From the UE perspective TS 38.101-1 also matters. |
| Inmarsat | Agree |  |

All companies agree. Moderator suggests the potential WF:

Proposal 2-1-2-1: The common definition for channel bandwidth, transmission bandwidth configuration, minimum guard band, and RB alignment in 38.104 and 38.101-1 can be reused for NTN system.

Issue 2-1-3: Supported Channel Bandwidth per Operating Band

- Proposals

  - Option 1: The supported channel bandwidth per operating band should be defined based on operator input.

- Recommended WF

  - TBA

Question: Do you partially agree/disagree with the Option 1 stated above? Please provide your views on the Option 1 stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Agree | Aligned with RAN4 current practice. |
| CATT | Agree |  |
| Huawei | Agree | However, irregular channel bandwidths are not considered for NTN. |
| Qualcomm | Agree |  |
| Hughes/EchoStar | Agree | Aligned with RAN4 |
| Nokia | Agree | Normal procedure |
| Intelsat | Agree |  |
| Xiaomi | Agree |  |
| Samsung | Agree |  |
| THALES | Agree |  |
| ZTE | Agree |  |
| Inmarsat | Agree |  |

All companies agree. Moderator suggests the potential WF (after adding “NTN”):

Proposal 2-1-3-1: The supported channel bandwidth per operating band should be defined based on NTN operator input.

Issue 2-1-4: Channel Spacing/Channel Raster

- Proposals

  - Option 1: The channel spacing in 38.104 can be reused for NTN. Exact definition pending channel raster decision.

- Recommended WF

  - TBA

Question: Do you partially agree/disagree with the Option 1 stated above? Please provide your views on the Option 1 stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Agree |  |
| CATT | Agree |  |
| Huawei | Agree |  |
| Hughes/EchoStar | Agree |  |
| Nokia | Agree |  |
| Intelsat | Agree |  |
| Xiaomi | Agree |  |
| Samsung | Agree |  |
| THALES | Agree |  |
| ZTE | Agree | Channel raster should be agreed firstly. |
| Inmarsat | Agree |  |

All companies agree. Moderator suggests the potential WF:

Proposal 2-1-4-1: The channel spacing in 38.104 can be reused for NTN. Exact definition pending channel raster decision.

### Sub-topic 2-2

Sub-topic description: Generic discussions on specifications

Open issues and candidate options before e-meeting:

Issue 2-2-1: New TS for RF NTN

- Proposals

  - Option 1: It is proposed to develop a separate set of RF specifications for NTN, including BS RF and UE RF.

  - Option 2: Other.

- Recommended WF

  - Discussion required to see if UE RF requires separated RF specification. Please also see Proposal NTN-2.2 of RP-210791 (RAN#91-e agreement): “New TS capturing the radio transmission and reception requirements for Satellite node” (i.e. BS RF), which means that there is already an agreement for the BS RF NTN separate set of specifications.

  - Moderator note: Result may be different if NTN UE FR1 or NTN UE FR2

  - Moderator note: At least in FR1, the TN and NTN UE specification should be probably the same in order to allow interoperability between TN and NTN.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Partially agree | NTN BS RF shall be separate TS as agreed in RAN. For UE NTN FR1, it’s may be too early to decide if we need separate specs or not, that would depend on how requirements would differ from TN to NTN. But if we go for re-using 38.101-1, it shall then be separate clauses, like it was done for NB-IoT, MTC, Sidelink, … |
| CATT | Agree |  |
| Huawei | Partially agree | A separate specification for NTN BS RF is needed. For UE, we can further study for it. |
| Qualcomm | Partially agree | Share the similar view as Ericsson |
| Hughes/EchoStar | Partially agree | Similar view as Ericsson |
| Nokia | Partially agree | As explained by Ericsson |
| Intelsat | Partially Agree | Similar view as Ericsson |
| Xiaomi | Partially Agree | Agree with the view from Ericsson |
| Samsung | Partially agree | Share similar view as Ericsson |
| THALES | Agree | However, result may be different if UE NTN FR1 or UE NTN FR2. UE NTN FR2 may require a different specification, while UE NTN FR1 may use similar specification as 38.101-1 (potentially with different clauses for NTN) |
| ZTE | Partially agree | Similar view as Ericsson and Huawei |
| Apple | Partially agree | Similar to views from other companies, it might a bit early to conclude on the UE RF. We need to have more UE RF related discussions to understand whether they will be different comparing to terrestrial networks. |
| Inmarsat | Partially Agree | Ericsson comment seems reasonable. However, for NTN FR1, there might be different cases. Some FR1 NTN UE classes should have same specs as TN, some may be different. Multiple types of UEs have been considered so far from the study phase. We should not close the door either way. |

Moderator suggests the potential WF:

Proposal 2-2-1-1: UE NTN FR1 may use similar specification as TS 38.101-1 (potentially with different clauses for NTN).

### Sub-topic 2-3

Sub-topic description: Reference assumption for the amplification process

Open issues and candidate options before e-meeting:

Issue 2-3-1: Reference assumption for the amplification process

- Proposals

  - Option 1: RAN4 to study and agree on the reference assumption to be used for the on-board satellite amplification process.

  - Option 2: Implementation issue, not in the scope of RAN4.

- Recommended WF

  - TBA

Question: Which option (listed above) do you prefer? Please provide your answer(s) e.g. “Yes” or “No”.

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 | Comments Option 2 |
| --- | --- | --- |
| Ericsoon | Yes This proposal was made in the RRM AI, but this inormation seems to be needed for a proper measurements management | No The detailed implementation is most likely product specific, but some amplification levels and possibly corresponding signalling need to be specified |
| CATT | Agree |  |
| Nokia | Yes – reference assumptions should be defined. |  |
| THALES | RAN4 should use the assumptions from section 5.1.1, TR 38.821. However, most of the satellites consider mainly 2 options (Option 2 and Option 3): 2. Constant Equivalent Isotropic Radiated Power (EIRP). The receive and transmit gains of the satellite are adjusted based on the received signal to obtain a constant target EIRP. 3. Constant power at receiver. The satellite may attempt to compensate the loss of the radio channel to ensure constant power at the NTN-GW, i.e. feeder link equalization. These 2 typical implementations therefore allow some control of the satellite amplification level. | We think is just an implementation issue. |
| Inmarsat | At surface it looks like it should be left for implementation, but comment from Ericsson is legitimate. | At surface it looks like it should be left for implementation, but comment from Ericsson is legitimate. |

Moderator suggests the potential WF:

Proposal 2-3-1-1: RAN4 to study and agree on the reference assumption to be used for the on-board satellite amplification process.

Proposal 2-3-1-2: RAN4 to consider Constant Equivalent Isotropic Radiated Power (EIRP) and/or Constant Power at Receiver options for on-board satellite amplification process.

Proposal 2-3-1-3: RAN4 to consider reference assumption for the amplification process as implementation issue, not in the scope of RAN4.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Please see above |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 2-1-1: Frequency Range Definition | Tentative agreements: Proposal 2-1-1-1: The frequency range definition for MSS S-band should be aligned with NR. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 2-1-2: BW Configuration | Tentative agreements: Proposal 2-1-2-1: The common definition for channel bandwidth, transmission bandwidth configuration, minimum guard band, and RB alignment in 38.104 and 38.101-1 can be reused for NTN system. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 2-1-3: Supported Channel Bandwidth per Operating Band | Tentative agreements: Proposal 2-1-3-1: The supported channel bandwidth per operating band should be defined based on NTN operator input. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 2-1-4: Channel Spacing/Channel Raster | Tentative agreements: Proposal 2-1-4-1: The channel spacing in 38.104 can be reused for NTN. Exact definition pending channel raster decision. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 2-2-1: New TS for RF NTN | Tentative agreements: Proposal 2-2-1-1: UE NTN FR1 may use similar specification as TS 38.101-1 (potentially with different clauses for NTN). Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 2-3-1: Reference assumption for the amplification process | Tentative agreements: Proposal 2-3-1-1: RAN4 to study and agree on the reference assumption to be used for the on-board satellite amplification process. Proposal 2-3-1-2: RAN4 to consider Constant Equivalent Isotropic Radiated Power (EIRP) and/or Constant Power at Receiver options for on-board satellite amplification process. Proposal 2-3-1-3: RAN4 to consider reference assumption for the amplification process as implementation issue, not in the scope of RAN4. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #3: FR1 Generalities

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2110614 | ZTE Corporation | Proposal 1: to postpone the SU discussion until there are clear agreement for out-of-band emission requirement and in-band emission requirements defined for NTN; Proposal 2: propose channel raster as 100kHz for NTN L-band and S-band; |
| R4-2110413 | Huawei, HiSilicon | Proposal 1: It’s proposed that the default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for NTN S-band is 190MHz. Proposal 2: 100 kHz channel raster can be used for NTN S-band. Observation 1: For Synchronization raster, one solution is to reuse current work frame for NTN system, but for applicable SS raster entries per operating band RAN4 may need to further study it. |
| R4-2111423 | THALES, Hughes/EchoStar, Inmarsat, ESA | In Figure 1 it is represented a reference NTN deployment scenario that can be identified for operation in MSS S-band, with one NTN operator using 30 MHz in UL and 30 MHz in DL: Figure 1. Reference Deployment Scenario Proposal 1: RAN4 shall consider the reference deployment scenario in MSS S-band with channel bandwidths of 5, 10, 15, 20 MHz for SCS=15 kHz configuration. Proposal 2: RAN4 shall consider the reference deployment scenario in MSS S-band with channel bandwidths of 10, 15, and 20 MHz for SCS=30 kHz and 60 kHz configuration. Proposal 3: Is up to NTN operator network design which FRF value shall be further used, independently of the coexistence analysis. [To be discussed in [99-e][313] NTN_Solutions_Part2:] Proposal 4: For the purpose of coexistence analysis between adjacent bands, 20 MHz bandwidth size can be considered for simulation purpose. Observation 1: However, in order to simplify the simulations, coexistence analysis could take into account the 30 MHz configuration as an example for NTN deployment in S-band. |
| R4-2109053 | CATT | Proposal 1: It is proposed to develop a separate set of RF specifications for NTN, including BS RF and UE RF. Proposal 2: The frequency range definition should be aligned with NR. Proposal 3: The same set of band coding and signaling design should be used for NTN and NR. The NTN band is numbered in reverse order from the maximum NR band number in each FR. Proposal 4: The common definition for channel bandwidth, transmission bandwidth configuration, minimum guard band, and RB alignment in 38.104 can be reused for NTN system. Proposal 5: The supported channel bandwidth per operating band should be defined based on operator input. Proposal 6: The channel spacing in 38.104 can be reused for NTN. - Exact definition pending channel raster decision. Proposal 7: It is proposed to define the channel raster for NTN bands as SCS based for both FR1 and FR2. - 15kHz is considered for FR1 and 60kHz is considered for FR2 for NTN channel raster - NR-ARFCN can be reused from 38.104. - Δshift is not needed for NTN bands. Proposal 8: It is proposed RAN4 to investigate whether some optimization on the sync raster is needed considering the cell search requirement. |
| R4-2109327 | GLOBALSTAR Inc. | Proposal 1: RAN4 shall also consider the mixed pairing of (L-band) and (S-band) in the related NTN work item. Proposal 2: RAN4 shall also consider the pairing of 1610 - 1618.725 MHz UL (L-band) and 2483.5 – 2500 MHz DL (S-band) in the related NTN work item. Proposal 3: 3GPP should take the relevant FCC and ETSI regulation into account when defining Tx and Rx NTN requirements for the pairing of 1610 - 1618.725 MHz UL and 2483.5 – 2500 MHz DL. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1

Sub-topic description: S-Band Related Discussions

Note: discussion can be separated from coexistence parameters for [99-e][313] NTN_Solutions_Part2

Open issues and candidate options before e-meeting:

Issue 3-1-1: S-Band Reference Operational Deployment Scenario

- Proposals

  - Option 1: 30 MHz, 1 Operator

- Recommended WF

  - RAN4 shall use S-Band Reference Operational Deployment Scenario with 30 MHz, 1 Operator

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Partially agree | What we agree on is to specify a new NTN band covering 1980-2010 MHz for UL and 2170-2200MHz for DL. |
| CATT | Partially agree | Same view as Ericsson. |
| Huawei |  | Should this issue for coexistence study be discussed in thread 313? We can avoided duplicated discussion. |
| Qualcomm | Disagree | Per R4-2111423, satellite operators propose the 20MHz as the maximum channel bandwidth for reference deployment scenario. |
| Hughes/EchoStar | Partially agree | The S band already identified for FR1 exemplary band 30 UL and 30 DL (1980-2010 and 2170-2200 MHz) |
| Samsung |  | Not sure what is the intention to discuss the reference operational deployment scenario here, is it for coexistence study? |
| THALES | Agree | NTN operators expressed willingness to use the entire band. This is a typical deployment scenario we should consider in RAN4. |
| ZTE |  | For CBW, it rely on the operator’s request. |
| Apple |  | The issue is not clear. If the question is about the band, then yes, the S-band can be defined as the one having 30MHz in both DL and UL. If the question is about co-existence scenario, then it should be discussed in a separate thread. |

It seems to be some confusion between Reference Deployment Operational scenario for S-Band (i.e. [99-e][312] NTN_Solutions_Part1) and Adjacent Band Coexistence Scenarios/Analysis (i.e. [99-e][313] NTN_Solutions_Part2).

The proposal only says that we should consider as Reference Deployment Operational scenario the entire 30 MHz UL/30 MHz DL MSS S-Band with only 1 NTN Operator. The BW size is a different topic, the NTN operator organises uses the spectrum as it wishes.

Therefore, the moderator proposes the following WF:

Proposal 3-1-1-1: RAN4 shall use S-Band Reference Operational Deployment Scenario with only 1 NTN Operator using 1980-2010 MHz for UL and 2170-2200 MHz for DL.

Issue 3-1-2: Possible Band Configuration for S-Band with SCS 15 kHz

- Proposals

  - Option 1: 5, 10, 15, 20 MHz (same as n65)

  - Option 2: 5, 10, 15, 20, 25, 30 MHz

  - Option 3: postpone the SU discussion until there are clear agreement for out-of-band emission requirement and in-band emission requirements defined for NTN

- Recommended WF

  - A TN UE implementation should allow connectivity with NTN. Therefore, a TN UE using n65 should be able to connect to NTN MSS S-band.

  - The supported channel bandwidth per operating band should be defined based on NTN operator input, and therefore Option 1 should be considered

Question: Which option (listed above) do you prefer? Please provide your answer(s) e.g. “Yes” or “No”.

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 | Comments Option 2 | Comments Option 3 |
| --- | --- | --- | --- |
| Ericsson | Ok | Ok | Channel BW is not really a SU discussion (the number of RBs per channel BW is not depending on the band). |
| CATT | OK | OK | Same view as above. |
| Huawei | OK | OK |  |
| Qualcomm | Yes | No. per input of satellite operators’ input in R4-2111423, |  |
| Hughes/EchoStar | Yes | No | No |
| Samsung | Yes Would like go with the requirement of satellite operators |  |  |
| THALES | Yes | No, see R4-2111423 | No |
| ZTE | okay | okay |  |
| Apple | Why do we need to align with the TN band n65? The intention of this option is not clear. | Ok | Channel bandwidth is not SU, as the latter defines the number of available RBs within a particular channel bandwidth. |
| Inmarsat | Yes |  |  |

All companies seem to agree with the 1st Option. Therefore, the moderator proposes the following WF:

Proposal 3-1-2-1: RAN4 shall consider the following bandwidth size configuration for MSS S-Band with SCS 15 kHz: 5, 10, 15, 20 MHz (same as n65).

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Partially agree | There is no need to align TN UE with NTN UE, a NTN UE will have anyway some specific features to support, with its own requirement. But we agree channel BW definition per band is up to operators. |
| CATT | Partially agree | NTN UE can have specific requirement. |
| Qualcomm | Partially agree | Have concerns on ‘a TN UE using n65 should be able to connect to NTN MSS S-band.’ It depends on the NTN UE requirements. Agree with the second bullet. |
| Hughes/EchoStar | Partial Agree | First bullet, at least for the upper 30 MHz portion of the n65. Agree with 2nd |
| THALES | Agree | The upper 30 MHz portion of n65 has been designed to integrate MSS S-band for terrestrial. A TN UE implementation in n65 should allow connectivity with NTN. Therefore, a TN UE using the upper 30 MHz portion of n65 should be able to connect to NTN MSS S-band. We therefore should keep the same (allowed) configuration as per n65. Another option would be to increase the channel bandwidth of n65 to 30 MHz, but it would require extra work and should be treated with less priority. The supported channel bandwidth per operating band should be defined based on NTN operator input, and therefore Option 1 should be considered. |
| ZTE | Partially agree | Similar view as Ericsson |
| Apple | Partially agree | The first sentence is misleading, "a TN UE using n65 should be able to connect to NTN MSS S-band." Our understanding is that a UE supporting e.g. band n65 will not be able to use the NTN MSS band in the same frequency range because NTN will require the corresponding functional extensions introduced by RAN1 and RAN2. We agree with the statement "The supported channel bandwidth per operating band should be defined based on NTN operator input" but do not agree that only the CBWs in Option 1 should be considered (e.g. the CBWs in Option 2 should also be considered) |

Issue 3-1-3: Possible Band Configuration for S-Band with SCS 30 kHz and SCS 60 kHz

- Proposals

  - Option 1: 10, 15, 20 MHz (same as n65)

  - Option 2: 10, 15, 20, 25, 30 MHz

  - Option 3: postpone the SU discussion until there are clear agreement for out-of-band emission requirement and in-band emission requirements defined for NTN

- Recommended WF

  - A TN UE implementation should allow connectivity with NTN. Therefore, a TN UE using n65 should be able to connect to NTN MSS S-band.

  - The supported channel bandwidth per operating band should be defined based on NTN operator input, and therefore Option 1 should be considered

Question: Which option (listed above) do you prefer? Please provide your answer(s) e.g. “Yes” or “No”.

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 | Comments Option 2 | Comments Option 3 |
| --- | --- | --- | --- |
| Ericsson | Ok | Ok | Channel BW is not really a SU discussion (the number of RBs per channel BW is not depending on the band). |
| CATT | OK | ok | Same view as above |
| Huawei | OK | OK |  |
| Qualcomm | Yes | No |  |
| Hughes/EchoStar | Yes | No | No direct relation with SU |
| Xiaomi | Yes |  |  |
| Samsung | Yes Would like go with the requirement of satellite operators |  |  |
| THALES | Yes | No, see R4-2111423 | No |
| ZTE | okay | okay |  |
| Apple | Why do we need to align with the TN band n65? The intention of this option is not clear. | Ok | Channel bandwidth is not SU, as the latter defines the number of available RBs within a particular channel bandwidth. |
| Inmarsat | Yes |  |  |

All companies seem to agree with the 1st Option. Therefore, the moderator proposes the following WF:

Proposal 3-1-3-1: RAN4 shall consider the following bandwidth size configuration for MSS S-Band with SCS 30 kHz and SCS 60 kHz: 10, 15, 20 MHz (same as n65).

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Partially agree | There is no need to align TN UE with NTN UE, a NTN UE will have anyway some specific features to support, with its own requirement. But we agree channel BW definition per band is up to operators. |
| CATT | Partially agree | NTN UE can have specific requirement. |
| Qualcomm | Partially agree | Have concerns on ‘a TN UE using n65 should be able to connect to NTN MSS S-band.’ It depends on the NTN UE requirements. Agree with the second bullet. |
| Hughes/EchoStar | Partial Agree | First bullet-at least for the upper 30 MHz portion of the n65. Agree with 2nd |
| THALES | Agree | The upper 30 MHz portion of n65 has been designed to integrate MSS S-band for terrestrial. A TN UE implementation in n65 should allow connectivity with NTN. Therefore, a TN UE using the upper 30 MHz portion of n65 should be able to connect to NTN MSS S-band. We therefore should keep the same (allowed) configuration as per n65. Another option would be to increase the channel bandwidth of n65 to 30 MHz, but it would require extra work and should be treated with less priority. The supported channel bandwidth per operating band should be defined based on NTN operator input, and therefore Option 1 should be considered |
| ZTE | Partially agree |  |
| Apple | Partially agree | The first sentence is misleading, "a TN UE using n65 should be able to connect to NTN MSS S-band." Our understanding is that a UE supporting e.g. band n65 will not be able to use the NTN MSS band in the same frequency range because NTN will require the corresponding functional extensions introduced by RAN1 and RAN2. We agree with the statement "The supported channel bandwidth per operating band should be defined based on NTN operator input" but do not agree that only the CBWs in Option 1 should be considered (e.g. the CBWs in Option 2 should also be considered) |

The moderator proposes the following WF:

Proposal 3-1-3-2: Channel BW definition per MSS S-band is up to NTN operators.

Or alternatively

Proposal 3-1-3-2: The supported channel bandwidth per operating band should be defined based on NTN operator input.

Issue 3-1-4: FRF values for S-Band

- Proposals

  - Option 1: Is up to NTN operator network design which FRF value shall be further used, independently of the coexistence analysis.

- Recommended WF

  - Is up to NTN operator network design which FRF value shall be further used, independently of the coexistence analysis.

  - Note: RAN4 should not restrict any FRF value, even if for coexistence analysis in adjacent bands the FRF value can be set to 1.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Disagree | Of course, it’s up to NTN operator to decide which FRF to use. But, if in the coexistence study it’s decided and agreed that FRF=1 is generating too much interference, this means such deployment shall also be forbidden for NTN. |
| CATT | Disagree | We need to use the typical FRF for co-existence study. |
| Huawei | Disagree | Share the same view with Ericsson |
| Qualcomm | Agree |  |
| Hughes/EchoStar | Agree | Yes, if referring to deployment. For co-existence simulation, need to weigh on what’s realistic simulation purposes |
| THALES | Agree | With respect to the deployment, the FRF may follow different combinations, depends on the density and of the operation scenarios. Also the BW sizes per each beam may not be equal. So all this should be left for operator use. We also agree that FRF1 may generate too much interference between beams, but it depends on the operational scenarios. For example, for some reason, the operator may deploy only one single beam. For coexistence simulations we need to see what is realistic. The approach from RAN4#98bis-e is reasonable to start first with FRF1. |
| ZTE | Disagree | Share the same view with Ericsson |
| Apple | Agree | From the RAN4 perspective we do not put any restrictions on FRF. |
| Inmarsat | Agree | For deployment, it is up to NTN operators to decide an acceptable configuration for their systems. |

It seems to be some confusion between Reference Deployment Operational scenario for S-Band (i.e. [99-e][312] NTN_Solutions_Part1) and Adjacent Band Coexistence Scenarios/Analysis (i.e. [99-e][313] NTN_Solutions_Part2). For the coexistence analysis, the approach from RAN4#98bis-e is reasonable i.e. to start first with FRF1.

The moderator proposes the following WF:

Proposal 3-1-4-1: Is up to NTN operator network design which FRF value shall be further used for a real deployment situation, independently of the coexistence analysis.

Issue 3-1-5: S-Band Channel Raster

- Proposals

  - Option 1: 100 kHz

- Recommended WF

  - A TN UE implementation should allow connectivity with NTN. Therefore, a TN UE using n65 should be able to connect to NTN MSS S-band, and therefore the same channel raster should be used.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| CATT | disagree | Seems not necessary to align with TN. SCS based could be possible. |
| Huawei | Agree Option 1 |  |
| Qualcomm | agree partially | Option 1 is OK since it is in line with the channel raster for n65. Disagree with the first half of sentence in recommended WF. Whether TN UE on band n65 can connect to MSS S-band is determined by NTN UE requirements. |
| Hughes/EchoStar | Partially Agree | Option 1 is OK |
| Xiaomi | Agree |  |
| Samsung | Agree Option1 |  |
| THALES | Agree | Option 1 or WF with changes if TN UE is allowed to use upper 30 MHz portion of n65. For this reason, a TN UE implementation should allow connectivity with NTN. |
| ZTE | Agree Option 1 |  |
| Apple | Partially agree | Option 1 is Ok. However, the first sentence in the suggested WF is incorrect. Our understanding is that a UE supporting e.g. band n65 will not be able to use the NTN MSS band in the same frequency range because NTN will require the corresponding functional extensions introduced by RAN1 and RAN2. |
| Inmarsat | Agree with Option 1 |  |

All companies seem to agree with Option 1, but not entirely with the proposed WF. The moderator proposes the following WF:

Proposal 3-1-5-1: RAN4 shall consider a 100 kHz MSS S-Band Channel Raster.

Issue 3-1-6: S-Band Synchronization Raster

- Proposals

  - Option 1: For Synchronization raster, one solution is to reuse current work frame for NTN system, but for applicable SS raster entries per operating band RAN4 may need to further study it.

- Recommended WF

  - A TN UE implementation should allow connectivity with NTN. Therefore, a TN UE using n65 should be able to connect to NTN MSS S-band, and therefore a similar synchronization raster should be used.

Question: Do you partially agree/disagree with the recommended way forward stated above? Please provide your views on the recommended Way Forward stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| CATT |  | Need further study. |
| Huawei | Agree Option 1 |  |
| Qualcomm | agree partially | similar synchronization raster is OK Disagree with the first half of sentence in recommended WF. Whether TN UE on band n65 can connect to MSS S-band is determined by NTN UE requirements. |
| Hughes/EchoStar | Agree option 1 |  |
| Xiaomi | Agree | The principle can be reused, but FFS on the SS raster entries per operating band |
| Samsung | Agree option 1 |  |
| THALES | Agree | Ok with option 1. For the WF, if the TN UE can use the upper 30 MHz portion of n65, is no longer a requirement, is a default characteristic that should be taken into account for the design of NTN UE. |
| ZTE | Agree Option 1 |  |
| Apple | Agree partially | Option 1 is Ok. However, the first sentence in the suggested WF is incorrect. Our understanding is that a UE supporting e.g. band n65 will not be able to use the NTN MSS band in the same frequency range because NTN will require the corresponding functional extensions introduced by RAN1 and RAN2. |
| Inmarsat | Agree with Option 1 |  |

All companies (except 1) seem to agree with Option 1, but not entirely with the proposed WF. The moderator proposes the following WF:

Proposal 3-1-6-1: With respect to MSS S-Band Synchronization Raster, one solution is to reuse current NR work frame for NTN system, but for applicable SS raster entries per operating band RAN4 may need to further study it.

Issue 3-1-7: TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation

- Proposals

  - Option 1:  190 MHz for default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for NTN S-band;

- Recommended WF

  - TBA

Question: Do you partially agree/disagree with the Option 1 stated above? Please provide your views on the Option 1 stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson |  | Not sure what to comment here, this is related to the band definition anyway… |
| CATT |  | Depending on band definition. |
| Huawei | Agree |  |
| Qualcomm |  | Depends on the spectrum |
| Hughes/EchoStar |  | Unclear question |
| Samsung |  | Depend on the band definition, and yes for the operating band 1980-2010 MHz/UL and 2170-2200MHz/DL. |
| THALES | Agree | Depends on the band definition. However, if only 20 MHz is allowed for BW configuration, this means that the NTN UE will use e.g. the lower 20 MHz for both UL and DL or the upper 20 MHz for both UL and DL, etc. So the proposed default channel separation is reasonable. |
| ZTE | Agree |  |

The moderator proposes the following WF:

Proposal 3-1-7-1: RAN4 should consider 190 MHz for default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for NTN S-band;

Note: Maximum BW sizes of 20 MHz may apply.

Issue 3-1-8: S-band Extension

- Proposals

  - Option 1: Consider S-band Extension 2483.5 – 2500 MHz for DL as part of NTN work item

- Recommended WF

  - TBA

Question: Do you partially agree/disagree with the Option 1 stated above? Please provide your views on the Option 1 stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Disagree | We are fine to consider this new band in a new WI on operator’s request, as it’s common practice for any TN band in RAN4. But it was already agree to consider S-band and L-band in the scope of this NTN WI, we should stick to this agreement and not add more band in that WI, |
| Huawei |  | Maybe we should consider not to increase the workload for NTN if we want to finish it timely in Release 17. |
| Qualcomm | Disagree | We should complete the example bands, i.e., S-band and L-band first. |
| Hughes/EchoStar | Disagree | Agree with Qualcomm |
| Globalstar |  | The issue is not entirely clear. However, referring forward to issue 3-3-2, we support adding a combination of 1610 - 1618.725 MHz UL (L-band) and 2483.5 – 2500 MHz DL (S-band) |
| Xiaomi |  | Share the same view with Qualcomm |
| Samsung | Disagree | Prefer to focus on the agreed example bands at this stage |
| ZTE |  | To have flexible access between S band L band , or flexible duplexer, this need more discussion since it will increase UE implementation difficulties. |
| Inmarsat | Disagree | Prefer to focus on exemplary band. |

There is some disagreement for considering S-band extension in this NTN WI. In any case, this proposal should be combined with the Sub-topic 3-3.

Moderator suggests the following WF:

Proposal 3-1-8-1: RAN4 may consider S-band extension in a new WI on operator’s request, as it’s common practice for any TN band in RAN4.

### Sub-topic 3-2

Sub-topic description: L-Band Related Discussions

Note: discussion is separated from coexistence parameters for [99-e][313] NTN_Solutions_Part2

Open issues and candidate options before e-meeting:

Issue 3-2-1: Possible Band Configuration for L-Band

- Proposals

  - Option 1: postpone the SU discussion until there are clear agreement for out-of-band emission requirement and in-band emission requirements defined for NTN

- Recommended WF

  - TBA

Question: Do you partially agree/disagree with the Option 1 stated above? Please provide your views on the Option 1 stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Disagree | The SU is not band related, why should we postpone this? |
| CATT | Disagree | Same view as Ericsson. |
| Qualcomm | Disagree | Both S-band and L-band are selected as example bands |
| Hughes/EchoStar |  | This should be an issue when addressing L-band |
| ZTE | agree | SU band istightly related with out of band emission requirements |
| Inmarsat |  | Let’s first address S band |

There is no recommendation from the moderator on this issue.

Issue 3-2-2: L-Band Channel Raster

- Proposals

  - Option 1: 100 kHz

- Recommended WF

  - TBA

Question: Do you partially agree/disagree with the Option 1 stated above? Please provide your views on the Option 1 stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Globalstar | Agree | Since the L-band belongs to the NR FR1 frequency range, it is natural to assume that 100kHz channel raster will be inherited from the baseline NR functional design. |
| Xiaomi | Agree |  |
| THALES | Agree | Agree if the equivalent NR terrestrial band is using 100kHz channel raster. |
| ZTE | Agree with option 1 |  |

All companies seem to agree with Option 1. The moderator proposes the following WF:

Proposal 3-2-2-1: RAN4 shall consider a 100 kHz MSS L-Band Channel Raster.

### Sub-topic 3-3

Sub-topic description: S-band and L-band Hybridization

Note: discussion is separated from coexistence parameters for [99-e][313] NTN_Solutions_Part2

Open issues and candidate options before e-meeting:

Issue 3-3-1: Mixed Pairing – General Discussion

- Proposals

  - Option 1: RAN4 shall also consider the mixed pairing of (L-band) and (S-band) in the related NTN work item.

- Recommended WF

  - TBA

Question: Do you partially agree/disagree with the Option 1 stated above? Please provide your views on the Option 1 stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Disagree | Again, this should be considered in a separate dedicated WI, as it’s common practice for any TN band in RAN4. We should stick to the 2 bands agreement and not add more band in this NTN WI, |
| Qualcomm | Disagree | Share the same view as Ericsson |
| Hughes/EchoStar | Disagree | Same view as Qualcomm and Ericsson |
| Globalstar | Agree | Referring forward to issue 3-3-2, we support adding a combination of 1610 - 1618.725 MHz UL (L-band) and 2483.5 – 2500 MHz DL (S-band). |
| ZTE | Disagree |  |
| Inmarsat | Disagree | We appreciate the intent, but this should be considered as a separate WI release-independent. |

It seems to be some disagreement for considering mixed pairing of (L-band) and (S-band) in the related NTN work item.

Moderator suggests the following WF:

Proposal 3-3-1-1: RAN4 may consider mixed pairing of (L-band) and (S-band) in a separate dedicated WI, as it’s common practice for any TN band in RAN4.

Issue 3-3-2: Potential Mixed Pairing Configuration

- Proposals

  - Option 1: RAN4 shall also consider the pairing of 1610 - 1618.725 MHz UL (L-band) and 2483.5 – 2500 MHz DL (S-band) in the related NTN work item.

- Recommended WF

  - TBA

Question: Do you partially agree/disagree with the Option 1 stated above? Please provide your views on the Option 1 stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Disagree | Again, this should be considered in a separate dedicated WI, as it’s common practice for any TN band in RAN4. We should stick to the 2 bands agreement and not add more band in this NTN WI, |
| Huawei |  | Not sure whether the mixed pairing configuration can be considered as L-band. |
| Qualcomm | Disagree | Share the same view as Ericsson |
| Hughes/EchoStar | disagree | Same view as Qualcomm and Ericsson |
| Globalstar | Agree | We support adding a combination of 1610 - 1618.725 MHz UL (L-band) and 2483.5 – 2500 MHz DL (S-band) for the NTN related work item. |
| Samsung | Disagree | Prefer to focus on the agreed example bands at this stage |
| ZTE | Disagree |  |
| Inmarsat | Disagree | We appreciate the intent, but this should be addressed later as a separate WI, release independent. Let’s first address the exemplary band. |

It seems to be some disagreement for consider the pairing of 1610 - 1618.725 MHz UL (L-band) and 2483.5 – 2500 MHz DL (S-band) in the related NTN work item.

Moderator suggests the following WF:

Proposal 3-3-2-1: RAN4 may consider pairing of 1610 - 1618.725 MHz UL (L-band) and 2483.5 – 2500 MHz DL (S-band) in a separate dedicated WI, as it’s common practice for any TN band in RAN4.

Issue 3-3-3: Existent Regulation with Respect to Mixed Pairing

- Proposals

  - Option 1: 3GPP should take the relevant FCC and ETSI regulation into account when defining Tx and Rx NTN requirements for the pairing of 1610 - 1618.725 MHz UL and 2483.5 – 2500 MHz DL.

- Recommended WF

  - TBA

Question: Do you partially agree/disagree with the Option 1 stated above? Please provide your views on the Option 1 stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Agree | But in a dedicated WI for this new band. |
| Qualcomm | Agree |  |
| Hughes/EchoStar | Agree | When submitting in independent release WI |
| Globalstar | Agree |  |
| THALES | Agree |  |
| ZTE | Agree |  |
| Inmarsat | Agree | Yes, in future WI. |

All companies seem to agree. Moderator suggests the following WF:

Proposal 3-3-3-1: 3GPP should take the relevant FCC and ETSI regulation into account when defining Tx and Rx NTN requirements for the pairing of 1610 - 1618.725 MHz UL and 2483.5 – 2500 MHz DL.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Please see above |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Note: discussion can be separated from coexistence parameters for [99-e][313] NTN_Solutions_Part2

|  | Status summary |
| --- | --- |
| Issue 3-1-1: S-Band Reference Operational Deployment Scenario; | Tentative agreements: Proposal 3-1-1-1: RAN4 shall use S-Band Reference Operational Deployment Scenario with only 1 NTN Operator using 1980-2010 MHz for UL and 2170-2200 MHz for DL. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 3-1-2: Possible Band Configuration for S-Band with SCS 15 kHz; | Tentative agreements: Proposal 3-1-2-1: RAN4 shall consider the following bandwidth size configuration for MSS S-Band with SCS 15 kHz: 5, 10, 15, 20 MHz (same as n65). Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 3-1-3: Possible Band Configuration for S-Band with SCS 30 kHz and SCS 60 kHz; | Tentative agreements: Proposal 3-1-3-1: RAN4 shall consider the following bandwidth size configuration for MSS S-Band with SCS 30 kHz and SCS 60 kHz: 10, 15, 20 MHz (same as n65). Proposal 3-1-3-2: Channel BW definition per MSS S-band is up to NTN operators. Or alternatively Proposal 3-1-3-2: The supported channel bandwidth per operating band should be defined based on NTN operator input. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 3-1-4: FRF values for S-Band; | Tentative agreements: Proposal 3-1-4-1: Is up to NTN operator network design which FRF value shall be further used for a real deployment situation, independently of the coexistence analysis. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 3-1-5: S-Band Channel Raster; | Tentative agreements: Proposal 3-1-5-1: RAN4 shall consider a 100 kHz MSS S-Band Channel Raster. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 3-1-6: S-Band Synchronization Raster; | Tentative agreements: Proposal 3-1-6-1: With respect to MSS S-Band Synchronization Raster, one solution is to reuse current NR work frame for NTN system, but for applicable SS raster entries per operating band RAN4 may need to further study it. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 3-1-7: TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation; | Tentative agreements: Proposal 3-1-7-1: RAN4 should consider 190 MHz for default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for NTN S-band; Note: Maximum BW sizes of 20 MHz may apply. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 3-1-8: S-band Extension; | Tentative agreements: Proposal 3-1-8-1: RAN4 may consider S-band extension in a new WI on operator’s request, as it’s common practice for any TN band in RAN4. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 3-2-1: Possible Band Configuration for L-Band; | Tentative agreements:- Candidate options:- Recommendations for 2nd round: There is no recommendation from the moderator on this issue. |
| Issue 3-2-2: L-Band Channel Raster; | Tentative agreements: Proposal 3-2-2-1: RAN4 shall consider a 100 kHz MSS L-Band Channel Raster. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 3-3-1: Mixed Pairing – General Discussion; | Tentative agreements: Proposal 3-3-1-1: RAN4 may consider mixed pairing of (L-band) and (S-band) in a separate dedicated WI, as it’s common practice for any TN band in RAN4. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 3-3-2: Potential Mixed Pairing Configuration; | Tentative agreements: Proposal 3-3-2-1: RAN4 may consider pairing of 1610 - 1618.725 MHz UL (L-band) and 2483.5 – 2500 MHz DL (S-band) in a separate dedicated WI, as it’s common practice for any TN band in RAN4. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |
| Issue 3-3-3: Existent Regulation with Respect to Mixed Pairing; | Tentative agreements: Proposal 3-3-3-1: 3GPP should take the relevant FCC and ETSI regulation into account when defining Tx and Rx NTN requirements for the pairing of 1610 - 1618.725 MHz UL and 2483.5 – 2500 MHz DL. Candidate options:- Recommendations for 2nd round: Discuss proposals for 2nd round and agree if possible by the end of the meeting. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #4: FR2 Generalities

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2110813 | Intelsat, Hughes, Inmarsat, ESA, Thales, Fraunhofer | Proposal 1: As part of the Rel-17 NR-NTN WI, a technical study on Ka band for FR2 co-existence analysis will be carried out to identify further technical issues, associated to the deployment of NTN supported by NR (FDD mode) in the satellite Ka band. However, the normative work on Ka band and other bands >10 GHz can be carried out as a part of a release independent work item. For the Rel-17 technical study, - RAN4 will use UL (27-30 GHz) frequency range of Ka band to do co-existence study in Rel-17 NR-NTN WI; - Ka band/FR2 co-existence analysis can be started once the S band/FR1 co-existence analysis principles are stable enough; - Other bands such as Ku & Q/V band will leverage the Ka band/FR2 co-existence study approach in the future. Proposal 2: For NTN coexistence study purposes, RAN should consider the uplink part of satellite “Ka band”, corresponding with the 27.0 – 30.0 GHz range, which includes both “FSS” and “MSS” denomination, since the provision of VSAT and ESIM services from GSO and NGSO is allowed in FSS as well as MSS denominated spectrum within said range, with different denomination depending on regional regulations, as regulated by ITU-R regulations and recently harmonized on a global basis by WRC Resolutions 156 [8], 169 [9] and 173 [10]. Proposal 3: It has been clarified throughout the SI and WI definition that NTN covers both VSAT and ESIMs/ESOMPs (which are a type of mobile VSATs operating in FSS spectrum). Hence, the frequency bands considered for 3GPP NTN RAN4 work should include spectrum relevant to VSAT and ESIM/ESOMP operation. |
| R4-2110688 | Nokia, Nokia Shanghai Bell | Proposal 1: Postpone NTN NR FR2 band introduction to specification until the devission of the 7-24GHz range have been settled and a FDD band in the FR2 range have been defined. Observation 1: There are no intention not to define FR2 support for NTN. However, from a time management perspective given the timeline for Rel-17 focus should be on FR1 support for NTN. |
| R4-2109053 | CATT | Proposal 1: It is proposed to develop a separate set of RF specifications for NTN, including BS RF and UE RF. Proposal 2: The frequency range definition should be aligned with NR. Proposal 3: The same set of band coding and signaling design should be used for NTN and NR. The NTN band is numbered in reverse order from the maximum NR band number in each FR. Proposal 4: The common definition for channel bandwidth, transmission bandwidth configuration, minimum guard band, and RB alignment in 38.104 can be reused for NTN system. Proposal 5: The supported channel bandwidth per operating band should be defined based on operator input. Proposal 6: The channel spacing in 38.104 can be reused for NTN. - Exact definition pending channel raster decision. Proposal 7: It is proposed to define the channel raster for NTN bands as SCS based for both FR1 and FR2. - 15kHz is considered for FR1 and 60kHz is considered for FR2 for NTN channel raster - NR-ARFCN can be reused from 38.104. - Δshift is not needed for NTN bands. Proposal 8: It is proposed RAN4 to investigate whether some optimization on the sync raster is needed considering the cell search requirement. |
| R4-2110993 | Hughes/EchoStar, Inmarsat, Thales, Fraunhofer | Observation 1: ITU-RR designates primary allocations to the FSS and MSS in the commercial Ka band (17.3-20.2 GHZ and 27.0-30.0 GHz). Observation 2: Satellite broadband access provided across Ka-band are accomplished primarily through the use of VSAT type terminals. Observation 3: Satellite in Ka-band provide broadband access, including Internet connectivity, on platforms in motion via ESIM. Observation 4: Satellite broadband services via VSAT and ESIM operate across the entire Ka band. Proposal 1: RAN4 to consider defining the entire “Ka Band” as an NTN-NR band in FR2 range for GEO and NGSO based satellite access as proposed in RP-200638 [2]. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

RAN-P decisions from RP-210791:

- Agreed Proposal NTN-1.1: “For frequencies above 10 GHz, any work can be limited to VSAT, ESIM service and terminals.”

- Agreed Proposal NTN-1.2: “The Satellite Ka band refers to [17.3 – 20.2 GHz] on the downlink and [27.0 – 30.0 GHz] on the uplink as allocated by ITU-R to satellite services. Some of this range is designated as FSS and some as MSS.”

Moderator note: Note that RAN-P decisions from RP-210791 can be used in RAN4:

### Sub-topic 4-1

Sub-topic description: NR NTN FR2 Introduction to Specification

Open issues and candidate options before e-meeting:

Issue 4-1-1: NR NTN FR2 Introduction to Specification

- Proposals

  - Option 1: 2-step approach:

    - As part of the Rel-17 NR-NTN WI, a technical study on Ka band for FR2 co-existence analysis will be carried out to identify further technical issues, associated to the deployment of NTN supported by NR (FDD mode) in the satellite Ka band.

    - The normative work on Ka band and other bands >10 GHz can be carried out as a part of a release independent item.

  - Option 2: 1-step approach:

    - Postpone NTN NR FR2 band introduction to specification until the decision of the 7-24 GHz range has been settled and a FDD band in the FR2 range has been defined. Note: There are no intention not to define FR2 support for NTN. However, from a time management perspective given the timeline for Rel-17 focus should be on FR1 support for NTN.

- Recommended WF

  - TBA

Question: Which option (listed above) do you prefer? Please provide your answer(s) e.g. “Yes” or “No”.

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 | Comments Option 2 |
| --- | --- | --- |
| Ericsson | No We already know we won’t be able to specify any FR2 NTN band in Rel-17, based on RAN agreements (Proposal NTN-1.3), with the current NTN WI description. | Yes We should first focus on FR1 NTN to give RAN4 a chance to finalize the work in Rel-17. |
| Huawei | No | Yes |
| Hughes/EchoStar | Yes | No. The study 7-24 GHz range has concluded and nothing ongoing right now |
| Nokia | No | Yes |
| THALES | Yes | No. It does not make sense if no intention not to define FR2 support for NTN. |
| Panasonic | Yes | No |
| ZTE | No | Yes |
| Apple | No As commented several times, FR2 NTN bands will require FDD operation in the FR2 range, core functionality and requirements for which are missing in the existing specifications. | Yes |
| Inmarsat | Yes. We think this proposal is quite reasonable. | No. |

4 companies replied YES and 5 companies replied NO for the 2-Step Approach with respect to NR NTN FR2 Introduction to Specification.

It does not make sense if no intention not to define FR2 support for NTN.

Moderator suggests to continue discussion on the GTW session (25th of May 2021) or in the 2nd round of RAN4#99-e.

### Sub-topic 4-2

Sub-topic description: Rel-17 NR NTN FR2 Coexistence Studies

Open issues and candidate options before e-meeting:

Issue 4-2-1: General Aspects on Rel-17 NR NTN FR2 Coexistence Studies

- Proposals

  - Option 1: For the Rel-17 technical study,

    - RAN4 will use UL (27-30 GHz) frequency range of Ka band to do co-existence study in Rel-17 NR-NTN WI;

    - Ka band/FR2 co-existence analysis can be started once the S band/FR1 co-existence analysis principles are stable enough;

    - Other bands such as Ku & Q/V band will leverage the Ka band/FR2 co-existence study approach in the future.

- Recommended WF

  - TBA

Question: Do you partially agree/disagree with the Option 1 stated above? Please provide your views on the Option 1 stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Disagree | We could start discussion on FR2 coexistence simulations once FR1 requirements are finalized. Let’s focus first on FR1. |
| Huawei | Disagree | We share the same view with Ericsson. |
| Hughes/EchoStar | Agree |  |
| Nokia | Disagree | Same view as Ericsson and in our contribution |
| THALES | Agree | Coexistence simulations for UL Ka would be very useful in Rel-17. The number of coexistence cases is much lower than for S-band. |
| Panasonic | Agree |  |
| Apple | Disagree | We should focus on completing the core functionality with FR1 bands. |
| Inmarsat | Agree |  |

4 companies Disagree and 4 companies Agree to consider FR2 coexistence studies in Rel-17:

- RAN4 will use UL (27-30 GHz) frequency range of Ka band to do co-existence study in Rel-17 NR-NTN WI;

- Ka band/FR2 co-existence analysis can be started once the S band/FR1 co-existence analysis principles are stable enough;

- Other bands such as Ku & Q/V band will leverage the Ka band/FR2 co-existence study approach in the future.

Moderator suggests to continue discussion on the GTW session (25th of May 2021) or in the 2nd round discussion of RAN4#99-e.

Issue 4-2-2: Ka-Band Frequency Range for NTN Deployment Scenario in NTN-TN FR2 Coexistence Analysis

- Proposals

  - Option 1: For NTN coexistence study purposes, RAN should consider the uplink part of satellite “Ka band”, corresponding with the 27.0 – 30.0 GHz range, which includes both “FSS” and “MSS” denomination, since the provision of VSAT and ESIM services from GSO and NGSO is allowed in FSS as well as MSS denominated spectrum within said range, with different denomination depending on regional regulations, as regulated by ITU-R regulations and recently harmonized on a global basis by WRC Resolutions 156 [8], 169 [9] and 173 [10].

- Recommended WF

  - TBA

Question: Do you partially agree/disagree with the Option 1 stated above? Please provide your views on the Option 1 stated above.

| Company | Agree, agree partially, disagree | Comments |
| --- | --- | --- |
| Ericsson | Disagree | We could start discussion on FR2 coexistence simulations once FR1 requirements are finalized. Let’s focus first on FR1. |
| Huawei | Disagree | We share the same view with Ericsson. |
| Hughes/EchoStar | Agree |  |
| Nokia | Disagree | Same view as Ericsson and in our contribution |
| Samsung |  | Note that the Res. 173 invite ITU-R to develop the technical and regulatory provisions for the operation of ESIM during WRC-23 study cycle under AI 1.16, the expected output regulation need to be taken into account for the NTN specification on Ka band in 3GPP, however, relevant studies are still undergoing in ITU-R. |
| THALES | Agree |  |
| Panasonic | Agree |  |
| Apple | Disagree | We should focus on completing the core functionality with FR1 bands. |
| Inmarsat | Agree |  |

4 companies Disagree and 4 companies Agree to consider the uplink part of satellite “Ka band”, corresponding with the 27.0 – 30.0 GHz range, which includes both “FSS” and “MSS” denomination, since the provision of VSAT and ESIM services from GSO and NGSO is allowed in FSS as well as MSS denominated spectrum within said range, with different denomination depending on regional regulations, as regulated by ITU-R regulations and recently harmonized on a global basis by WRC Resolutions 156 [8], 169 [9] and 173 [10].

Moderator suggests to continue discussion on the GTW session (25th of May 2021) or in the 2nd round discussion of RAN4#99-e.

### Sub-topic 4-3

Sub-topic description: NTN SI & WI scope

RAN-P decisions from RP-210791:

- Agreed Proposal NTN-1.1: “For frequencies above 10 GHz, any work can be limited to VSAT, ESIM service and terminals.”

- Agreed Proposal NTN-1.2: “The Satellite Ka band refers to [17.3 – 20.2 GHz] on the downlink and [27.0 – 30.0 GHz] on the uplink as allocated by ITU-R to satellite services. Some of this range is designated as FSS and some as MSS.”

Moderator note: Note that RAN-P decisions from RP-210791 can be used in RAN4:

Issue 4-3-1: NTN SI & WI scope with respect to VSAT and ESIMs/ESOMPs

- Proposals

  - Option 1: It has been clarified throughout the SI and WI definition that NTN covers both VSAT and ESIMs/ESOMPs (which are a type of mobile VSATs operating in FSS spectrum). VSAT and ESIM operates in the entire Ka-band, hence, the frequency bands considered for 3GPP NTN RAN4 work should include spectrum relevant to VSAT and ESIM/ESOMP operation.

  - Option 2: RAN4 to consider defining the entire “Ka Band” as an NTN-NR band in FR2 range for GEO and NGSO based satellite access as proposed in RP-200638 [2].

- Recommended WF

  - TBA

Question: Which option (listed above) do you prefer? Please provide your answer(s) e.g. “Yes” or “No”.

[Note: Companies are encouraged to provide justification for their choices.]

| Company | Comments Option 1 | Comments Option 2 |
| --- | --- | --- |
| Ericsson | We shall follow Regulations anyway | Again this shall be aligned with Regulations anyway, but let’s finalize and focus on FR1 first. |
| Huawei | Before RAN plenary clarify the WID, we don't need to discuss the "Ka band" | No |
| Hughes/EchoStar | Yes | Yes |
| Nokia | Ka-band is not available in RAN4 as of now. Reasons listed in our contribution. | Ka-band is not available in RAN4 as of now. Reasons listed in our contribution. |
| THALES | Yes | Yes |
| Apple | The fact that WI mentions VSAT and ESIM does not automatically mean that certain bands are added to the WI. All WGs will introduce the corresponding NTN functionality, including VSAT devices, in the band agnostic way. | Ka-band can be considered once the core functionality is completed |
| Inmarsat | Yes | Yes |

It seems that companies agree to consider VSAT and ESIM but not necessary agree to consider NTN Ka-band.

Moderator suggests to continue discussion on the GTW session (25th of May 2021) or in the 2nd round discussion of RAN4#99-e.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Please see above |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 4-1-1: NR NTN FR2 Introduction to Specification; | Tentative agreements:- Candidate options:- Recommendations for 2nd round: Moderator suggests to continue discussion on the GTW session (25th of May 2021) or in the 2nd round of RAN4#99-e. |
| Issue 4-2-1: General Aspects on Rel-17 NR NTN FR2 Coexistence Studies | 4 companies Disagree and 4 companies Agree to consider FR2 coexistence studies in Rel-17: - RAN4 will use UL (27-30 GHz) frequency range of Ka band to do co-existence study in Rel-17 NR-NTN WI; - Ka band/FR2 co-existence analysis can be started once the S band/FR1 co-existence analysis principles are stable enough; - Other bands such as Ku & Q/V band will leverage the Ka band/FR2 co-existence study approach in the future. Tentative agreements:- Candidate options:- Recommendations for 2nd round: Moderator suggests to continue discussion on the GTW session (25th of May 2021) or in the 2nd round discussion of RAN4#99-e. |
| Issue 4-2-2: Ka-Band Frequency Range for NTN Deployment Scenario in NTN-TN FR2 Coexistence Analysis | 4 companies Disagree and 4 companies Agree to consider the uplink part of satellite “Ka band”, corresponding with the 27.0 – 30.0 GHz range, which includes both “FSS” and “MSS” denomination, since the provision of VSAT and ESIM services from GSO and NGSO is allowed in FSS as well as MSS denominated spectrum within said range, with different denomination depending on regional regulations, as regulated by ITU-R regulations and recently harmonized on a global basis by WRC Resolutions 156 [8], 169 [9] and 173 [10]. Tentative agreements:- Candidate options:- Recommendations for 2nd round: Moderator suggests to continue discussion on the GTW session (25th of May 2021) or in the 2nd round discussion of RAN4#99-e. |
| Issue 4-3-1: NTN SI & WI Scope with respect to VSAT and ESIMs/ESOMPs; | It seems that companies agree to consider VSAT and ESIM but not necessary agree to consider NTN Ka-band. Tentative agreements:- Candidate options:- Recommendations for 2nd round: Moderator suggests to continue discussion on the GTW session (25th of May 2021) or in the 2nd round discussion of RAN4#99-e. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Recommendations for Tdocs

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Email discussion summary for [99-e][312] NTN_Solutions_Part1 (2nd Round) | Thales, 2nd round discussion |
| #2 | Way Forward on NTN_solutions_Part1 | Thales, WF |

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| Email discussion summary for [99-e][312] NTN_Solutions_Part1 | THALES | R4-2108437 revised to R4-211xxxx |
| Way Forward on NTN_solutions_Part1 | THALES | Document # R4-211xxxx WF [99-e][312] NTN_Solutions_Part1 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Appendix: Companies contribution summary

Contribution summaries are as follows:

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2110813 | Intelsat, Hughes, Inmarsat, ESA, Thales, Fraunhofer | Proposal 1: As part of the Rel-17 NR-NTN WI, a technical study on Ka band for FR2 co-existence analysis will be carried out to identify further technical issues, associated to the deployment of NTN supported by NR (FDD mode) in the satellite Ka band. However, the normative work on Ka band and other bands >10 GHz can be carried out as a part of a release independent work item. For the Rel-17 technical study, - RAN4 will use UL (27-30 GHz) frequency range of Ka band to do co-existence study in Rel-17 NR-NTN WI; - Ka band/FR2 co-existence analysis can be started once the S band/FR1 co-existence analysis principles are stable enough; - Other bands such as Ku & Q/V band will leverage the Ka band/FR2 co-existence study approach in the future. Proposal 2: For NTN coexistence study purposes, RAN should consider the uplink part of satellite “Ka band”, corresponding with the 27.0 – 30.0 GHz range, which includes both “FSS” and “MSS” denomination, since the provision of VSAT and ESIM services from GSO and NGSO is allowed in FSS as well as MSS denominated spectrum within said range, with different denomination depending on regional regulations, as regulated by ITU-R regulations and recently harmonized on a global basis by WRC Resolutions 156 [8], 169 [9] and 173 [10]. Proposal 3: It has been clarified throughout the SI and WI definition that NTN covers both VSAT and ESIMs/ESOMPs (which are a type of mobile VSATs operating in FSS spectrum). Hence, the frequency bands considered for 3GPP NTN RAN4 work should include spectrum relevant to VSAT and ESIM/ESOMP operation. |
| R4-2110614 | ZTE Corporation | Proposal 1: to postpone the SU discussion until there are clear agreement for out-of-band emission requirement and in-band emission requirements defined for NTN; Proposal 2: propose channel raster as 100kHz for NTN L-band and S-band; |
| R4-2110688 | Nokia, Nokia Shanghai Bell | Proposal 1: Postpone NTN NR FR2 band introduction to specification until the devission of the 7-24GHz range have been settled and a FDD band in the FR2 range have been defined. Observation 1: There are no intention not to define FR2 support for NTN. However, from a time management perspective given the timeline for Rel-17 focus should be on FR1 support for NTN. |
| R4-2110413 | Huawei, HiSilicon | Proposal 1: It’s proposed that the default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for NTN S-band is 190MHz. Proposal 2: 100 kHz channel raster can be used for NTN S-band. Observation 1: For Synchronization raster, one solution is to reuse current work frame for NTN system, but for applicable SS raster entries per operating band RAN4 may need to further study it. |
| R4-2111423 | THALES, Hughes/EchoStar, Inmarsat, ESA | Proposal 1: RAN4 shall consider the reference deployment scenario in MSS S-band with channel bandwidths of 5, 10, 15, 20 MHz for SCS=15 kHz configuration. Proposal 2: RAN4 shall consider the reference deployment scenario in MSS S-band with channel bandwidths of 10, 15, and 20 MHz for SCS=30 kHz and 60 kHz configuration. Proposal 3: Is up to NTN operator network design which FRF value shall be further used, independently of the coexistence analysis. Proposal 4: For the purpose of coexistence analysis between adjacent bands, 20 MHz bandwidth size can be considered for simulation purpose. Observation 1: However, in order to simplify the simulations, coexistence analysis could take into account the 30 MHz configuration as an example for NTN deployment in S-band. |
| R4-2109053 | CATT | Proposal 1: It is proposed to develop a separate set of RF specifications for NTN, including BS RF and UE RF. Proposal 2: The frequency range definition should be aligned with NR. Proposal 3: The same set of band coding and signaling design should be used for NTN and NR. The NTN band is numbered in reverse order from the maximum NR band number in each FR. Proposal 4: The common definition for channel bandwidth, transmission bandwidth configuration, minimum guard band, and RB alignment in 38.104 can be reused for NTN system. Proposal 5: The supported channel bandwidth per operating band should be defined based on operator input. Proposal 6: The channel spacing in 38.104 can be reused for NTN. - Exact definition pending channel raster decision. Proposal 7: It is proposed to define the channel raster for NTN bands as SCS based for both FR1 and FR2. - 15kHz is considered for FR1 and 60kHz is considered for FR2 for NTN channel raster - NR-ARFCN can be reused from 38.104. - Δshift is not needed for NTN bands. Proposal 8: It is proposed RAN4 to investigate whether some optimization on the sync raster is needed considering the cell search requirement. |
| R4-2109116 | CATT | Proposal 1: It is proposed to consider wired connection between NTN Gate-way and non-NTN infrastructure gNB function as the typical case. Observation: It seems no problem to develop RF requirements and conduct test based on Port A in Figure 2-2 for FR1. Proposal 2: It is proposed to develop RF requirements and conduct test based on Port A in Figure 2-2 for FR1. |
| R4-2109504 | CMCC | Observation 1: it is suggested to use the interface between gateway and non-NTN gNB as Rx reference point considering non-NTN gNB is the last component in the Rx RF linkage from UE to gNB. Observation 2: RAN4 could define reference point where there is no interface definition in RAN3 with the assumption that the message format received from satellite gateway has been transferred to match NR message format and could be demodulated. Proposal 1: it is suggested to take satellite + gateway as a single black-box entity without any dedicated Rx requirements and take the interface between gateway and non-NTN gNB as Rx reference point to define/test Rx link RF requirements. |
| R4-2111460 | THALES | Proposal 1: The NTN DUT device for testing Rx NTN gNB parameters shall be composed of the RU NTN, Feederlink Emulator, GW, Non-NTN infrastructure gNB and the required signal generators for 5G NR NTN wanted signal, 5G NR (NTN) interfering signal and CW interfering signal. Proposal 2: Based on different combinations and following similar testing procedures as in e.g. TS 38.141, the throughput measurement point at the Non-NTN infrastructure gNB can be used to determine the 5% throughput loss. Proposal 3: The NTN test setup can use a Feederlink Emulator or not. Proposal 4: The NTN test setup can use a NTN Channel Emulator or not, with Doppler or without Doppler. Observation 1: The quality of the radio at the satellite payload side is the most predominant factor of the Rx tests for the gNB side of the NTN service link. |
| R4-2110120 | Ericsson | Proposal 1: RAN4 should handle gateway + satellite as a repeater and specify needed requirements for gateway + satellite in a new NTN repeater specification. |
| R4-2110194 | Xiaomi | Proposal: it is proposed that defining RF requirements for service link shall be considered as a priority. |
| R4-2110615 | ZTE Corporation | Proposal 1: to discuss what’s functionality expected for gNB and what’s kind of requirement to be tested; Proposal 2: for Case A, at least the baseband capability of gNB to serve the service link should be tested, and the corresponding requirement should be defined at the reference point between gateway and gNB; Proposal 3: for Case B/C, to define reference point between gateway and gNB for the purpose of testing baseband capability of NTN system; |
| R4-2110993 | Hughes/EchoStar, Inmarsat, Thales, Fraunhofer | Observation 1: ITU-RR designates primary allocations to the FSS and MSS in the commercial Ka band (17.3-20.2 GHZ and 27.0-30.0 GHz). Observation 2: Satellite broadband access provided across Ka-band are accomplished primarily through the use of VSAT type terminals. Observation 3: Satellite in Ka-band provide broadband access, including Internet connectivity, on platforms in motion via ESIM. Observation 4: Satellite broadband services via VSAT and ESIM operate across the entire Ka band. Proposal 1: RAN4 to consider defining the entire “Ka Band” as an NTN-NR band in FR2 range for GEO and NGSO based satellite access as proposed in RP-200638 [2]. |
| R4-2110118 | Ericsson | Withdrawn (Not Available) |
| R4-2109327 | GLOBALSTAR Inc. | Proposal 1: RAN4 shall also consider the mixed pairing of (L-band) and (S-band) in the related NTN work item. Proposal 2: RAN4 shall also consider the pairing of 1610 - 1618.725 MHz UL (L-band) and 2483.5 – 2500 MHz DL (S-band) in the related NTN work item. Proposal 3: 3GPP should take the relevant FCC and ETSI regulation into account when defining Tx and Rx NTN requirements for the pairing of 1610 - 1618.725 MHz UL and 2483.5 – 2500 MHz DL. |
| R4-2109117 | CATT | Reserved (Not Available) |