3GPP TSG-RAN WG4 Meeting # 99-e 												R4-2108446

Online, May 19-27, 2021

Agenda item:			6.1.7.1, 6.1.7.2, 6.1.7.3

Source:	Qualcomm

Title:	Email discussion summary for [99-e][321]

Document for:	Information

# Introduction

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Agree on the issues pending from the last RAN4 meeting, discuss the final requirements based on the simulation results shared by the companies, review proposed draft CR;

- 2nd round: Keep discussing on issues pending from the 1st round;

# Topic #2: Demodulation Requirements (PDSCH and CQI)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109352 | Apple | Proposal #1: Define requirements for Scenario A and C for 20MHz CBW. |
| R4-2109354 | Apple | Proposal #1: In slot 1 of FFP configure Aperiodic CSI report with aperiodic report slot offset of 6. Proposal #2: Use same downlink transmission scheme as agreed for PDSCH demod requirements. Proposal #3: Re-use the test parameters from CQI reporting tests in static channel for unlicensed carrier where applicable. Proposal #4: Define CQI reporting requirements for NR-U with the same SNR pair as Test 1 for existing CQI reporting tests in static channel. Proposal #5: Define minimum delta of median CQI of 2 for different transmission burst. |
| R4-2109588 | Ericsson | Issue 1-3: Requirement definition method for Scenario A and C. Observation: The maximum bandwidth for PCell is 40MHz for n66 and 20MHz for n25 and n48. Proposal 1: Configure TDD 20MHz for PCell in the test setup. Observation: There is no statement for mandatory UE supported bandwidth. Proposal 2: Define PDSCH requirements for {20, 40, 60, 80} MHz NR-U unlicensed cell. Only test the largest supported bandwidth for both Scenario A and C. Issue 2: CQI report scheduling Observation: Periodic CQI report could not fit in agreed burst transmission model. Observation: The gap between aperiodic CSI report and its reference CSI-RS could be much smaller than periodic CSI report situation. Proposal 3: Taking aperiodic CQI report for NR-U CQI report requirement test. |
| R4-2109591 | Ericsson | Proposal 1: Use aperiodic CQI report to fit for agreed burst transmission structure. Proposal 2: Only define NR-U CQI report requirement for 2Rx. Proposal 3: Consider following SNR configuration for CQI report requirement. Option 1: Low SNR burst set [3, 4] dB, high SNR burst set [9, 10]; Option 2: Low SNR burst set [7, 8] dB, high SNR burst set [13, 14]; |
| R4-2110500 | Huawei | Observation 1: It is optional for a UE to support a specific bandwidth for a given band. Proposal 1: Define the 20MHz for license band and define the requirements for 20MHz, 40MHz, 60MHz and 80MHz for unlicensed band with the following applicability rules: - For scenario A: After selecting the largest NR-U supported CBW by the UE, configure NR PCell with 20 MHz CBW in combination with selected NR-U CBW; - For scenario C: Only test the supported largest channel bandwidth. |
| R4-2110502 | Huawei | Proposal 1: Use CQI Table 2. Proposal 2: Set SNR pair to [8, 9]dB without power level boost for 2RX and Set SNR pair to [5, 6]dB without power level boost for 4RX and set minimum delta CQI for transmission burst with different power level boost to 2 Proposal 3: Use periodic CQI reporting. Proposal 4: Set CQI reporting periodicity and offset to 10 slots/ 9 slots and schedule PUCCH format 2 to transmit CQI information CQI in the first two symbols of last slot in each FFP. Proposal 5: Reuse the codebook configuration of Rel-15 CQI testing. Proposal 6: Change the S2 from {6, 9, 12, 14} to {14}.i.e. Always schedule full slots in the LBT burst transmission for NR-U CQI testing. |
| R4-2110718 | Qualcomm | Proposals 1: For NR-U CQI Performance tests, use CQI Table 2. Proposals 2: For NR-U CQI Performance tests, use CodebookSubsetRestriction 010000. Proposal 3: For NR-U CQI Performance tests, define requirements for 2RX using SNR [8, 9] dB, with the applicability rule that satisfying the requirement for one SNR point is sufficient. Proposal 4: For NR-U CQI Performance tests, define requirements for 4RX using SNR [5, 6] dB, with the applicability rule that satisfying the requirement for one SNR point is sufficient. Proposal 4: For NR-U CQI Performance tests, use the applicability rule to test UEs only for the largest supported number of RX. Proposal 4: Regarding the passing criteria for NR-U CQI Performance tests, the Median CQI delta across reporting based on different power boost should be larger than 2, for both 2RX and 4RX. |
| R4-2110766 | Qualcomm | Observation 1: Previous agreements were to define NR-U PDSCH demodulation requirements for the unlicensed carrier for BW {20,40,60,80} MHz and test the largest supported BW only. Observation 2: For operations in unlicensed bands, support for BW {20, 40, 60, 80} MHz is mandatory. Observation 3: Impact of CBW on the SNR requirement is limited (<1dB difference among proposed CBW options). Proposal 1: In line with current agreements, define and test NR-U UE PDSCH Performances requirements for 80MHz only. Proposal 2: Same as NR-U PDSCH, NR-U UE CQI Performances requirements should be defined and tested for 80MHz only. Proposal 3: For Scenario A, configure the NR PCell with CBW = 20MHz. Proposal 4: According to the proposals in this contribution for defining a single requirement CBW (for both NR licensed CC and NR Unlicensed CC), test applicability rules for largest supported CBW in NR-U PDSCH Tests are not needed and should not be included. Observation 4: The minimum gap between CSI-RS scheduling and Periodic CQI reporting is 8 slots for 30kHz SCS. Observation 5: Periodic CQI reporting would constraint CSI-RS scheduling and CQI Reporting to Slot #0 and Slot #9 respectively, within each 5ms periodicity. Observation 6: The minimum delay between CSI-RS scheduling and Aperiodic CSI reporting is 33 Symbols, which can fit within the 5ms periodicity, leaving the last slot completely idle. Proposal 5: To keep last slot as idle and avoid conflicts with SSB slot, use CSI-RS scheduling with periodicity 10 slots and offset 1 slot and Aperiodic CSI Report type. |
| R4-2110947 | Intel | Observation 1: TD multiplexing of DRS with data can take place only if data has BW equal to 20MHz. Otherwise another LBT for data is required Observation 2: DRS occupies first two slots, which overlaps with candidate positions for CSI-RS transmission. Proposal 1: Define dedicated COT for DRS transmission equal to 1ms (i.e. no TD multiplexing of DRS with data) Proposal 2: Define DRS periodicity equal to 40ms Proposal 3: For periodic CQI reporting define CSI-RS periodicity and offset as 10 and 0 slots respectively Proposal 4: For periodic CQI reporting define CSI-Report periodicity and offset as 10 and 8 slots respectively |
| R4-2109592 | Ericsson | CQI Simulation Results |
| R4-2110502 | Huawei | CQI Simulation Results |

## Open issues summary

The issues listed in this section address topics for discussions related to issue both general and specific for PDSCH performance testing.

### Sub-topic 1-1: DRS Configuration

Issue 1-1-1: Whether to define a dedicated COT for DRS transmission

- Proposals

  - Option 1: No, multiplex DRS and data in TD within the same COT, without multiplexing SSB and data in the same slot (Current WF);

  - Option 2: Yes, define dedicated COT for DRS equal to 1ms, without multiplexing in TD DRS and Data (Intel);

- Recommended WF:

  - TBA;

---------------GTW Note------------

Intel: multiplexing DRS and data with mixed BW is not supported in our view.

QC: We think no need to be aligned BW for DRS and data.

Intel: We need perform another LBT.

Agreement: Option 1.

| Company | Comments |
| --- | --- |
| Qualcomm | We identified two problems raised by Intel in their contribution regarding the current agreements on the DRS window, and it is our view that they do not motivate defining a dedicated COT for DRS. The observations are: - TD multiplexing of DRS with data can take place only if data has BW equal to 20MHz. Otherwise another LBT for data is required; We do not see an impediment in having the NB perform LBT after the end of the transmission of the SSB to continue with the PDSCH transmission as agreed over all the subbands, for tests with CBW larger than 20MHz. For dynamic channel access these two separate transmission does not seem to pose a problem. For semi-static channel access, according to 37.213, Section 4.3, multiple DL transmission bursts within the COT can be transmitted as long as the gNB performs LBT appropriately. A channel occupancy initiated by a gNB and shared with UE(s) satisfies the following: - The gNB shall transmit a DL transmission burst starting at the beginning of the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration Tsi=9us. If the channel is sensed to be busy, the gNB shall not perform any transmission during the current period. - The gNB may transmit a DL transmission burst(s) within the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration Tsi=9us if the gap between the DL transmission burst(s) and any previous transmission burst is more than 16us.. - DRS occupies first two slots, which overlaps with candidate positions for CSI-RS transmission. Unless there are other reasons why this should be avoided, this possibility seems to be contemplated and allowed according to the spec in 37.213, Section 4.0 A discovery burst refers to a DL transmission burst including a set of signal(s) and/or channel(s) confined within a window and associated with a duty cycle. The discovery burst can be any of the following: - Transmission(s) initiated by an eNB that includes a primary synchronization signal (PSS), secondary synchronization signal (SSS) and cell-specific reference signal(s)(CRS) and may include non-zero power CSI reference signals (CSI-RS). - Transmission(s) initiated by a gNB that includes at least an SS/PBCH block consisting of a primary synchronization signal (PSS), secondary synchronization signal (SSS), physical broadcast channel (PBCH) with associated demodulation reference signal (DM-RS) and may also include CORESET for PDCCH scheduling PDSCH with SIB1, and PDSCH carrying SIB1 and/or non-zero power CSI reference signals (CSI-RS).. In conclusion, it’s our view that no modifications need to be done to the current DRS window assumption, so support option 1. |
| Huawei | We agree with QC and support Option 1 |
| Ericsson | We agree with Option 1. |
| Intel | Ok with Option 1 |

Issue 1-1-2: DRS Duration, Periodicity

- Proposals

  - Option 1: 1ms duration, 20ms periodicity (Current WF);

  - Option 2: 1ms duration, 40ms periodicity (Intel);

- Recommended WF:

  - TBA;

| Company | Comments |
| --- | --- |
| Qualcomm | Option 1, keep current DRS agreements. |
| Apple | Option 1. We don’t see strong reason to change the DRS periodicity. |
| Huawei | Option 1 |
| Ericsson | Agree with Option 1. |
| Intel | Agree with Option 1. Option 2 was reasonable only if dedicated COT for DRS would be defined |

### Sub-topic 1-2: Bandwidth configuration

Issue 1-2-1: Bandwidth to be used for PDSCH requirement definition, NR Unlicensed CC (Scenario A and C)

- Proposals

  - Option 1: 20 MHz (Apple);

  - Option 2: 80 MHz (Qualcomm);

  - Option 2: Define requirements for {20, 40, 60, 80} and test the largest supported unlicensed CBW only (Current WF, Ericsson, Huawei);

- Recommended WF:

  - According to the current agreement in the WF and the observation from most of the companies, define requirements for {20, 40, 60, 80} and test the largest supported unlicensed CBW only.

| Company | Comments |
| --- | --- |
| MediaTek | Support the WF. |
| Qualcomm | Support the WF |
| Apple | Option 1. The current WF agreement to define requirements/test for largest supported unlicensed CBW makes sense if we are testing CA requirements. We are configuring for CA in case of scenario A, but we don’t test the licensed carrier. This makes it like any other testcase we have defined since Rel-15 for PDSCH demod. We agreed to only define requirements for 15KHz/10MHz for FSS case, irrespective of what CBWs are supported by the UE. Why should it be different for unlicensed carrier? Hence, we proposed to introduce requirements only for 20MHz CBW which is the LBT BW. |
| Huawei | Support the WF |
| Ericsson | Agree with WF. |

-------GTW Note------

QC: We have only single Test, for NR we have several test cases. Test overhead should not be a concern since only single test can be applied.

Agreements:

PDSCH demod: Define requirements for {20, 40, 60, 80} and test the largest supported unlicensed CBW only

CSI: only 20MHz CHBW will be introduced

Define requirements configuring the NR PCell with CBW=20MHz (TDD) fpr scenario A.

Issue 1-2-2: Bandwidth to be used for PDSCH requirement definition, NR PCell CC (Scenario A)

- Proposals

  - Option 1: 20 MHz (Ericsson, Huawei, Qualcomm);

- Recommended WF:

  - Define requirements configuring the NR Pcell with CBW=20MHz (TDD);

| Company | Comments |
| --- | --- |
| MediaTek | Support the WF. |
| Qualcomm | Support the WF |
| Apple | We support the WF. |
| Huawei | Support the WF |
| Ericsson | Agree with WF. |
| Intel | Agree with WF |

Issue 1-2-3: Bandwidth to be used for CQI requirement definition (Scenario A and C)

- Proposals

  - Option 1: 20 MHz (Huawei, from draftCR);

  - Option 2: Use the same BW and applicability rule as in the proposed WF for PDSCH (Qualcomm);

- Recommended WF:

  - TBA;

| Company | Comments |
| --- | --- |
| MediaTek | Support Option 1. |
| Qualcomm | It is our view that the same discussion we had on PDSCH regarding on CBW support applies here. For this reason, it would make sense to define the CQI reporting test for BW ={20, 40, 60, 80}MHz and use the same applicability rule to test the largest CBW supported only. This should not have a repercussion on the CQI requirements, which should be applied to all CBWs. |
| Apple | We propose to only define requirements for 20MHz case, same as PDSCH demod. |
| Huawei | Support the Option 1 |
| Ericsson | Agree with Option 1. |
| Intel | Agree with Option 1 |

### Sub-topic 1-3: PDSCH Performance Test Requirements

Issue 1-3-1: SNR Requirement definition

- Proposals

  - Option 1: TBA;

- Recommended WF

Based on the Impairment results collected, propose and discuss the PDSCH SNR Requirements

| Company | Comments |
| --- | --- |

### Sub-topic 1-4: Configuration for CQI Performance Tests

Issue 1-4-1: Number of UE RX Antennas to define CQI requirements for

- Proposals

  - Option 1: 2 RX (Ericsson);

  - Option 2: {2, 4} RX (Huawei);

  - Option 3: {2, 4} RX, with the applicability rule to test UEs only for the largest supported number of RX (Qualcomm);

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| MediaTek | Support Option 3 which follows CQI tests in Rel-15. |
| Qualcomm | Support Option 3. |
| Apple | Option 3 is fine, but do we have similar applicability rule for PDSCH demod as well? |
| Huawei | Now we prefer Option 1 since supporting 2RX is mandatory for band n46 and 4RX is optional. We think 2RX is enough |
| Ericsson | We can accept Option 3. NR-U band is higher than 5GHz, and 4Rx might be more typical. It would be better to align with Rel-15/16 applicability rule. There is no optional or mandatory difference in Rx applicability rule definition. In LTE Rel-8, 2Rx is mandatory but the same applicability rule is applied in the later release. Furthermore, we already agreed to define requirement for 2Rx and 4Rx for PDSCH, and CQI test could follow PDSCH. |
| Qualcomm | We share Ericsson’s opinion, in Rel.15 the applicability rule does not distinguish between mandatory and optional capabilities. In conclusion, we propose to - align CQI requirements with the agreed PDSCH requirements and define them for both 2 and 4 RX, and - For both PDSCH and CQI requirements, test only the largest supported number of RX |
| Intel | Agree with Option 3 with the same applicability rule for PDSCH and CQI requirements |

---------------GTW Note---------------

Huawei: For core spec, band n46, n96 ; 4Rx is optional. 2Rx is enough on band n46.

Agreement:

{2, 4} RX, with the applicability rule to test UEs only with applicable rules based on UE declared capability. Further work on the text into specifications.

Issue 1-4-2: Periodic CSI-RS Resource Periodicity/Slot Offset

- Proposals

  - Option 1: 10/1 Slots (Current WF);

  - Option 2: 10/0 Slots (Intel);

- Recommended WF:

  - TBA;

| Company | Comments |
| --- | --- |
| Qualcomm | Support Option 1. |
| Apple | We might need to change it to aperiodic CSI-RS based on discussion in Issue 1-4-3? |
| Huawei | Support Option 1 |
| Intel | Need to consider aperiodic CSI-RS based on the discussion for Issue 1-4-3 |

Issue 1-4-3: CSI Reporting Type and Periodicity and Slot Offset or Aperiodic Report Slot Offset

- Proposals

  - Option 1: Aperiodic (Apple, Ericsson, Qualcomm)

    - Option 1a: Aperiodic Report Slot Offset 7 Slots (Apple);

  - Option 2: Periodic (Huawei, Intel)

    - Periodicity/Offset 10/9 Slots (Huawei);

    - Periodicity/Offset 10/8 Slots (Intel);

- Recommended WF:

  - TBA;

---------------------GTW Note ---------------

Apple: DCI 1-0, and aperiodic CSI –RS resource and reporting type should be used. 
QC: We share similar view as Apple. We don’t think SSB occsication in 2 slot without PDSCH scheduling have big impact on testing.

MTK: Aperiodic CSI report and periodic CSI-RS.

Huawei: Test time will be increased for that case/

Ercisson: If we use aperiodic CSI-RS, and skip PDSCH in CSI-RS slots. Two step approach can be applied.

QC: only impact on 2 slots with SSB. 6% increased. Two step approach will no issue at all.

Apple: Use Downlink length of COT for PDSCH and CQI test cases, that’s the approach used in LAA.

Agreement:

Aperiodic for CSI reporting type

- Not include 2 slot DL transmission duration into COT pattern for CSI test cases

- FFS periodic CSI-RS resource or aperiodic CSI-RS resources

| Company | Comments |
| --- | --- |
| MediaTek | There is a restriction for CQI test in Rel-15 that PDSCH is not scheduled on slots containing CSI-RS or slots which are not full DL. Also, according to specification TS38.213, UE cancels the CSI-RS reception if UE does not detect a DCI format indicating a periodic CRS-RS reception or scheduling a PDSCH reception. For operation with shared spectrum channel access, if a UE is provided csi-RS-ValidationWith-DCI, is not provided CO-DurationsPerCell, and is not provided SlotFormatCombinationsPerCell, and if the UE is configured by higher layers to receive a CSI-RS in a set of symbols of a slot, the UE cancels the CSI-RS reception in the set of symbols of the slot if the UE does not detect a DCI format indicating an aperiodic CSI-RS reception or scheduling a PDSCH reception in the set of symbols of the slot.. If we do follow the restriction, UE will not be able to do CSI-RS validation as no PDCCH for scheduling PDSCH is transmitted. To valid the CRI-RS reception, we may use aperiodic CRI-RS reporting with DCI format 0-1. However, if we do not follow the restriction and allocate PDSCH in the same slot as CRI-RS, we think periodic reporting can be adopted. We think this issue is related to issue 1-4-5 and we need to determine first whether to follow the restriction for CQI test in Rel-15. |
| Qualcomm | The problem raised by MediaTek in the previous comment is valid, but the specification states that UEs are supposed to validate scheduled Periodic CSI-RS when detecting a DCI format aperiodic CSI-RS reception. So reviewing the current agreement in the WF for CSI-RS type and using aperiodic CSI-RS for CSI reporting scheduling would be a viable solution for this issue. As a consequence, aperiodic CSI reporting should be used, so support option 1. |
| Apple | We support option 1a. We brought up this issue in our contribution as well for UE not being validate CSI-RS since PDSCH is not transmitted. Observation #1: For CQI reporting requirements, we don’t transmit CSI-RS and PDSCH in the same slot. The UE will not be able to do CSI-RS validation with periodic CSI-RS transmitted with periodicity 10 slots and offset 1 since no PDCCH scheduling data is transmitted. Our understanding was that configuring PDCCH in the slot with aperiodic CSI report trigger would be sufficient. But if it should be aperiodic CSI-RS, then we would need to have to configure aperiodic CSI-RS as well and only aperiodic reporting will not be a valid test setup. |
| Huawei | We support option 2. From our understanding, compared periodic CQI reporting, aperiodic CQI reporting needs more complicated test procedures and DCI signalling overhead. As our purpose is to verify the UE's behaviour for NR-U CQI measurement rather than CQI type, we should simplify the test procedure while achieving the test purpose. Meanwhile, we propose to schedule PDCCH/PDSCH in slot 1 to indicate UE to receive CSI-RS. Otherwise, there will be no PDSCH scheduling in transmission with SSB |
| Ericsson | According to specification delivered by MTK, it is a double negative sentence. We would translate it to a affirmative sentence. “the UE do the CSI-RS reception in the set of symbols of the slot if the UE does not detect a DCI format indicating an aperiodic CSI-RS reception and scheduling a PDSCH reception in the set of symbols of the slot.” That means aperiodic CSI-RS and PDSCH scheduling should be fulfilled at the same time, otherwise UE will cancel the CSI-RS reception. Is this a correct understanding? If this is the case, we might have to use aperiodic CSI-RS and scheduling PDSCH at the same slot. It will violate the rule used in Rel-15. Otherwise, DCI 2-0 might be needed to inform UE CO-DurationsPerCell and SlotFormatCombinationsPerCell. Then periodic CSI-RS could be used. As addressed in GTW, we don’t need to check UE behaviour of averaging CSI-RS between bursts if aperiodic CSI-RS would be used. The CQI report is limited to the closest CSI-RS resource measurement. @Qualcomm: We have different understanding. Two preconditions: A= ap-CSI-RS not detected, B=PDSCH not detected. Once A or B is true, then UE will cancel CSI-RS reception. Aperiodic CSI-RS DCI Format not detected; PDSCH DCI Format not detected; CSI-RS Reception. Y; Y; Canceled. Y; N; Canceled. N; Y; Canceled. N; N; Not canceled. |
| Qualcomm | We don’t agree with the interpretation of the spec provided by Ericsson. The double negation can be indeed elided to get a clearer understanding, but in their interpretation the change in the phrasing of the condition for validation from ‘either/or’ to ‘and’ seems unjustified. If we look at the potential cases, according to the original spec wording, we see: Aperiodic CSI-RS DCI Format detected; PDSCH DCI Format detected; CSI-RS Reception. Y; N; Not canceled. N; Y; Not canceled. Y; Y; Not canceled. N; N; Canceled. So the original spec can be also written as: [..] the UE validates CSI-RS in the set of symbols of the slot if the UE detects a DCI format indicating an aperiodic CSI-RS reception or scheduling a PDSCH reception in the set of symbols of the slot. @Huawei: as included in your comment, the actual CSI resource configuration type is not the main focus of the test here. Based on this, and taking into account that we fail to see a way in which we can configure periodic CSI-RS and still align with Rel.15 CQI assumptions, it seems evident that configuring aperiodic CSI-RS reception allows us to configure a test in line with Rel.15 CQI performance assumptions that also satisfies CSI-RS validation requirements and should be the option chosen. We also fail to see the added overhead, since DCI information has to be sent to the UE and be processed in both cases, whether to validate periodic or aperiodic CSI-RS. |

Issue 1-4-4: CSI Reporting UL Scheduling Type

- Proposals

  - Option 1: PUCCH Format 2 (Huawei);

- Recommended WF:

  - TBA;

| Company | Comments |
| --- | --- |
| Qualcomm | If aperiodic CSI reporting is chosen in issue 1-4-3, use PUSCH. |
| Apple | It would have to be PUSCH based on aperiodic reporting. |
| Huawei | As we discussed in our contribution, PUCCH format 2 only occupy 2 symbols and it left enough time for idle time. |

Issue 1-4-5: Downlink Transmission Scheme for CQI requirements

- Proposals

  - Option 1: Same Downlink model as agreed for PDSCH requirements (Apple, Current WF);

  - Option 2: Always schedule full slots in the LBT burst transmission for NR-U CQI testing (Huawei);

- Recommended WF:

  - For CQI requirements, used the same downlink model as agreed for PDSCH requirements

| Company | Comments |
| --- | --- |
| MediaTek | There is a restriction for CQI test in Rel-15 that PDSCH is not scheduled on slots containing CSI-RS or slots which are not full DL. If we do agree to follow the restriction, we cannot apply the current LBT burst transmission for PDSCH requirement for CQI testing. According the current WF, the resource for CSI-RS is allocated on slot 1 and there should be no PDSCH on slot 1, which violates the rule for the current downlink model for PDSCH. Besides, for DL Transmission duration larger than 2, the PDSCH allocation for the last slot should be full slot allocation, which also violates the rule for the current downlink model for PDSCH. Hence, similar to issue 1-4-3, we think we need to determine first whether to follow the restriction for CQI test in Rel-15. |
| Qualcomm | On the comment from MediaTek, we do not see why the restriction used in Rel.15 CQI tests cannot be applied on top of the downlink model used for PDSCH tests. A note can be added to the test assumption, that PDSCH is not scheduled on slots that contain CSI-RS or are not full DL. This implies that only when the last slot in the burst is full it will be scheduled with PDSCH. Support option 1, adding the restrictions used in Rel.15 CQI tests. |
| Apple | Perhaps we need to schedule full slots for CQI test. The code rate would change if the number of symbols for PDSCH is different in the slot and we try to keep the same code rate in all slots for CQI tests. Also, we might need to make the minimum DL duration 3 slots. With 2 slot transmission, we would have PDSCH only in slot 1 and if that coincides with SSB transmission, that is also missed and there would be no PDSCH transmission. If we make it minimum of 3 slots we would always have PDSCH transmission when there is no LBT failure. |
| Huawei | We propose to add the note that not scheduling PDSCH for partial slot and needn’t modify the LBT model. |
| Intel | Agree with proposals from Qualcomm and Huawei made in the comments: PDSCH is not scheduled on slots that are not full DL. |

Issue 1-4-6: CQI requirements Simulation Parameters

- Proposals

  - Option 1: Reuse licensed test parameters from CQI reporting test in static channel where applicable (Apple);

- Recommended WF:

  - TBA

| Company | Comments |
| --- | --- |
| Qualcomm | Support Option 1. |
| Apple | We support Option 1 |
| Huawei | Support Option 1 |

Issue 1-4-7: CQI requirements parameters: CQI Table

- Proposals

  - Option 1: Use CQI Table 2 (Huawei, Qualcomm)

- Recommended WF:

  - Use CQI Table 2.

| Company | Comments |
| --- | --- |
| MediaTek | Support the WF. |
| Qualcomm | Support the WF. |
| Apple | We support the WF. |
| Huawei | We support the WF |
| Ericsson | Support WF. |

Issue 1-4-8: CQI requirements parameters: Codebook Configuration

- Proposals

  - Option 1: Reuse from Rel.15 CQI Tests (Huawei);

  - Option 2: CodebookSubsetRestriction = 010000 (Qualcomm, Ericsson)

- Recommended WF:

  - TBA

| Company | Comments |
| --- | --- |
| MediaTek | Reuse CodebookSubsetRestriction = 010000 from Rel-15 CQI tests. |
| Qualcomm | Support Option 2 |
| Apple | Option 1 and 2 are the same. In Rel-15 the CodebookSubsetRestriction was 010000 for CQI tests in AWGN. |
| Huawei | Use CodebookSubSetRestriction=010000 |
| Ericsson | Agree with CodebookSubSetRestriction=010000 |

### Sub-topic 1-5: CQI Performance Tests Requirements

Issue 1-5-1: SNR pair to be used for requirements (not including Transmission Power Level Boost), 2 RX

- Proposals

  - Option 1: [8,9] dB (Huawei, Qualcomm, Apple);

    - Option 1a: Same SNR as Test 1 for existing CQI reporting tests in static channel (Apple);

  - Option 2: [3,4] dB (Ericsson);

  - Option 3: [7,8] dB (Ericsson);

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Qualcomm | Support Option 1. |
| Apple | We support Option 1. |
| Huawei | We support Option 1 |

Issue 1-5-2: SNR pair to be used for requirements (not including Transmission Power Level Boost), 4 RX

- Proposals

  - Option 1: [5,6] dB (Huawei, Qualcomm, Apple);

    - Option 1b: Same SNR as Test 1 for existing CQI reporting tests in static channel (Apple);

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Qualcomm | Support Option 1. |
| Apple | We support option1 |
| Huawei | If requirements for 4RX is defined, we support option 1 |

Issue 1-5-3: Minimum difference between Median CQI reported for different power level boost to be included in the requirements

- Proposals

  - Option 1: 2 (Huawei, Apple, Qualcomm);

- Recommended WF

  - Define the minimum difference between Median CQI reported for different power level boost requirement equal to 2.

| Company | Comments |
| --- | --- |
| Qualcomm | Support the WF. |
| Apple | We support the WF. |
| Huawei | We support WF |

## Companies views’ collection for 1st round

### Open issues

Companies are encouraged to comment in the dedicated comment section below each issue.

### CRs/TPs comments collection

| CR/TP number | Comments collection |  |
| --- | --- | --- |
|  | Company | Comment |
| R4-2110938, MediaTek CR for TS38.101-4, PDSCH requirements for standalone NR-U | Apple | - We need not specify TRS config as its same as that in common parameters. - Need not specify TX EVM - Number of tests is still TBD - Number of additional DMRS: 1 |
|  | Ericsson | Tx EVM is not needed. |
|  | Qualcomm | - Since the ‘number of slots set in a burst’ does not apply to PDSCH only, a dedicated section such as: Downlink Model Parameters; Downlink transmission duration values; {2,4,6,7}. Occupied OFDM symbols in slot other than the last slot; 12. Occupied OFDM symbols in the last slot; {4,7,10,12} (Note 1). - Rename the parameters to align with the draftCR for DL transmission model, according to the table above. @Apple: we agreed to ‘pos1’ in the additional DMRS configuration to avoid issues with shorter slots, right? |
|  | Intel | - We do not operate with the term “burst”. Need to change “number of slots set in burst” to “Downlink transmission duration values”. - In the dedicated section proposed by Qualcomm need to add the parameters “Downlink period” and “pLBT”. - We might need to mention that “last slot” means last slot of the DL transmission. Suggest to change: “Occupied OFDM symbols in slot other than the last slot” to “Occupied OFDM symbols in slot other than the last slot of the downlink duration” and “Occupied OFDM symbols in the last slot” to “Occupied OFDM symbols in the last slot of the downlink duration” |
|  | Huawei | - We agree with QC and Intel to set a separate part for transmission burst model and remove LBT failure to this part. - Agree with Intel’s view for description of “last slot”. - In Table 5.2.3.2.15-3, the title “ Minimum performance for Rank 4” should be changed to “ Minimum performance for Rank 2” to keep align with simulation assumptions - “TX EVM” should be deleted. - Suggest to use “Operation on shared spectrum access” instead of “CCA” to keep align with core spec. - Should we create separate sub-clauses for scenario A and scenario C? The test setup and test applicability rules for scenario C and scenario A are also missing. - For applicability of requirements for different number of RX antenna ports specified in sub-clause 5.1.1.2, may be we need some modifications - For applicability of requirements for mandatory UE features with capability signaling, we suggest to add the UE feature” Supported UL channels for dynamic channel access mode (ul-DynamicChAccess-r16 ) or UL channel access for semi-static channel access mode (ul-Semi-StaticChAccess-r16) or both” - In table 5.2.3.2.15-1, we suggest to remove the “When CSI-RS- ValidationWith-DCI-r16 is configured” to sub-clause 5.1.1.3 with some modifications |
| R4-2109355, Apple Draft CR NRU CQI Scenario A-R16 | Ericsson | Suggest to use terminology like “under CCA” to replace “NR-U”. This is aligned with RRM terminology. And then use “PCell under CCA” for Scenario C and “SCell under CCA” for Scenario A. According to TS38.133 A.9 and A.11 |
|  | Qualcomm | - BW, CSI-RS reporting type, Physical channel for CSI report, Codebook subset restriction should be updated according to the latest agreements. - Shouldn’t be [8,9] SNR dB correspond to 2 different test, and not both Test1? - Downlink Model Parameters section is missing (see comments to the first CR in the table) |
| R4-2109590, Ericsson Draft CR for TS38101-4 introduction of PDSCH demodulation requirements for NR-U Scenario A (catB)_pa1 | Apple | We don’t think the requirements should be specified under interworking requirements. Section 5 would be more appropriate. Ericsson: Thanks for the comment. Section 5 is used for standalone scenario, so we think CA scenario would be better to be defined in another section. |
| R4-2110501, Huawei Draft CR for TS 38.101-4 Introduction of fixed reference channel of NR-U PDSCH | Apple | We should add a note that there is no UL transmission in slot 9 and is idle slot. |
|  | Ericsson | Suggest to use terminology like “under CCA” to replace “NR-U”. This is aligned with RRM terminology. And then use “PCell under CCA” for Scenario C and “SCell under CCA” for Scenario A. According to TS38.133 A.9 and A.11 |
|  | Intel | For Table A.1.2-2b: In Note 3 the range for “i" should be corrected from {0,…,10} to {0,…,9} For Table A.3.2.2.2-18: mixing in one table different options of DL transmission duration with different options of number of symbols in last slot makes the table unreadable. Suggest to split the table into 4 tables – one for each option of DL transmission duration. |
| R4-2110503, Huawei Draft CR for TS 38.101-4 Introduction of NR-U CQI requirements | Apple | The table for test parameters Table 6.1.2-1 is not from latest version of 38.101-4 We shouldn’t introduce new section for CQI reporting for unlicensed carrier. We should add requirements in section 6.2. The requirements seem to cover both scenario A and scenario C. According to CR work split it should only be scenario C. We should have seprate sections for Scenario A and C The wording in the section for requirements in Scenario C should be aligned with Scenario A. Do we need a different measurement channel for unlicensed carrier? The CQI test is still for 1 layer as Rel-15 test. These should be set as configured: timeRestrictionForChannelMeasurements timeRestrictionForInterferenceMeasurements |
|  | Ericsson | Suggest to use terminology like “under CCA” to replace “NR-U”. This is aligned with RRM terminology. And then use “PCell under CCA” for Scenario C and “SCell under CCA” for Scenario A. According to TS38.133 A.9 and A.11 |
|  | Qualcomm | Should the UE feature/capability entry in the table be aligned with draftCR R4-2110938? Ie. ‘Verify PDSCH performance under CCA when CSI-RS-ValidationWith-DCI-r16 IE [17] is configured’; Downlink Model Parameters section is missing (see comments to the first CR in the table) |
|  | Intel | Table 6.1.2-1: for the “K1 value” the range for “i" should be corrected from {0,…,10} to {0,…,9} |
| R4-2110719, Qualcomm DraftCR on NR-U UE Demodulation Downlink Transmission Model | Apple | Suggest using unlicensed carrier or shared spectrum access rather than NR-U in section title and text. |
|  | Ericsson | Suggest to use terminology like “under CCA” to replace “NR-U”. This is aligned with RRM terminology. And then use “PCell under CCA” for Scenario C and “SCell under CCA” for Scenario A. According to TS38.133 A.9 and A.11 Suggest to add full name for COT. It is really good if we can add definition for these terminologies, i.e. CCA, COT, at the beginning of the specification. |
|  | Qualcomm | Change ‘the set of possible duration of the last slot values’ to ‘Occupied OFDM symbols in the last slot’ to align with PDSCH requirements draftCR. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 1-1: DRS Configuration | Agreements: Issue 1-1-1: Whether to define a dedicated COT for DRS transmission No, multiplex DRS and data in TD within the same COT, without multiplexing SSB and data in the same slot (Current WF); Issue 1-1-2: DRS Duration, Periodicity 1ms duration, 20ms periodicity (Current WF); |
| Sub-topic 1-2: Bandwidth configuration | Agreements: Issue 1-2-1: Bandwidth to be used for PDSCH requirement definition, NR Unlicensed CC (Scenario A and C) Define requirements for {20, 40, 60, 80} and test the largest supported unlicensed CBW only Issue 1-2-2: Bandwidth to be used for PDSCH requirement definition, NR PCell CC (Scenario A) Define requirements configuring the NR PCell with CBW=20MHz (TDD) for scenario A. Issue 1-2-3: Bandwidth to be used for CQI requirement definition (Scenario A and C) only 20MHz CHBW will be introduced |
| Sub-topic 1-6: PDSCH Performance Test Requirements | Recommendations for 2nd round: Issue 1-3-1: SNR Requirement definition - Proposals - Based on the impairment results provided in the simulation results collection, the following requirement is proposed for discussion. A margin 0.5 dB (for 16QAM) has been added to the average Impairment results. - Recommended WF - Agree on the proposed requirements. Case; BW (MHz); SCS (KHz); Duplex; RX; Requirement. Test 1-1a; 20; 30; TDD; 2; 14.3. Test 1-1b; 40; 30; TDD; 2; 14.5. Test 1-1c; 60; 30; TDD; 2; 14.7. Test 1-1d; 80; 30; TDD; 2; 15.0. Test 2-1a; 20; 30; TDD; 4; 9.3. Test 2-1b; 40; 30; TDD; 4; 9.3. Test 2-1c; 60; 30; TDD; 4; 9.5. Test 2-1d; 80; 30; TDD; 4; 9.7. Issue 1-3-2: Applicability Rule for PDSCH Tests based on UE declared capabilities - Proposals - Option 1: As in Rel.15 test, UEs are tested only for the largest number of supported RX; - Recommended WF: - Agree to the applicability rule in option 1. |
| Sub-topic 1-4: Configuration for CQI Performance Tests | Agreements: Issue 1-4-1: Number of UE RX Antennas to define CQI requirements for {2, 4} RX, with the applicability rule to test UEs only with applicable rules based on UE declared capability. Further work on the text into specifications. Issue 1-4-3: CSI Reporting Type and Periodicity and Slot Offset or Aperiodic Report Slot Offset Aperiodic for CSI reporting type Issue 1-4-4: CSI Reporting UL Scheduling Type PUSCH, according to Aperiodic CSI Reporting. Issue 1-4-5: Downlink Transmission Scheme for CQI requirements Use the same Downlink Model as agreed for PDSCH, with the following modifications/notes: Do not include 2 slot DL transmission duration into COT pattern for CSI test cases. Do not schedule slots that are not fully allocated to DL. Issue 1-4-6: CQI requirements Simulation Parameters Reuse licensed test parameters from CQI reporting test in static channel where applicable. Issue 1-4-7: CQI requirements parameters: CQI Table Use CQI Table 2. Issue 1-4-8: CQI requirements parameters: Codebook Configuration Use CodebookSubsetRestriction = 010000. Recommendations for 2nd round: Issue 1-4-10: Whether to reuse the Rel.15 CQI performance test setup with no PDSCH scheduling in slots containing CSI-RS - Proposals - Option 1: Yes (Qualcomm); - Option 2: No (Huawei); - Recommended WF: - TBA; Issue 1-4-11: CSI-RS Resource type and Periodicity/Slot Offset Continue the discussion on the feasibility to use periodic CSI-RS resource type in the 2nd round. - Proposals - Option 1: Periodic, 10/1 Slots (Current WF); - Option 2: Periodic, 10/0 Slots (Intel); - Option 3: Aperiodic, 10/1 Slots; - Recommended WF: - Continue discussion in the 2nd round. Issue 1-4-12: Applicability Rule for CQI Tests based on UE declared capabilities - Proposals - Option 1: As in Rel.15 test, UEs are tested only for the largest number of supported RX; - Recommended WF: - Agree to the applicability rule in option 1 Issue 1-4-13: Possible values for the length of the PDSCH allocation in the last slot in the downlink transmission duration for CQI performance Test - Proposals - Option 1: {4, 6, 10, 12}, same as PDSCH Performance test; - Option 2: {12}, always full DL allocation (Apple); - Recommended WF: - TBA; |
| Sub-topic 1-7: CQI Performance Tests Requirements | Agreements: Issue 1-5-2: SNR pair to be used for requirements (not including Transmission Power Level Boost), 4 RX Use SNR [5,6] dB. Issue 1-5-3: Minimum difference between Median CQI reported for different power level boost to be included in the requirements Define the minimum difference between Median CQI reported for different power level boost requirement equal to 2. Recommendations for 2nd round: Issue 1-5-1: SNR pair to be used for requirements (not including Transmission Power Level Boost), 2 RX - Proposals - Option 1: [8,9] dB (Huawei, Qualcomm, Apple); - Option 2: [3,4] dB (Ericsson); - Option 3: [7,8] dB (Ericsson); - Recommended WF - Can Ericsson agree to Option 1? |
| Sub-topic 1-6: Discussion on the draftCRs | Recommendations for 2nd round: Issue 1-6-1: Terminology to use across all CRs: - Proposals - Option 1: To align with RRM, “under CCA” for NR-U, “PCell under CCA” for Scenario C and “SCell under CCA” for Scenario A (Ericsson); - Option 2: To align with core spec, “Operation on shared spectrum access” (Huawei); - Recommended WF - TBA Issue 1-6-2: In R4-2110938, whether to add for the applicability of the requirements for mandatory UE features with capability signaling, the UE feature “Supported UL channels for dynamic channel access mode (ul-DynamicChAccess-r16 ) or UL channel access for semi-static channel access mode (ul-Semi-StaticChAccess-r16) or both”: - Proposals - Yes (Huawei); - Recommended WF - TBA Issue 1-6-3: In R4-2110938, Whether to move the “When CSI-RS- ValidationWith-DCI-r16 is configured” from table 5.2.3.2.15-1 to 5.1.1.3: - Proposals - Yes (Huawei); - Recommended WF - TBA Issue 1-6-4: In which section to introduce the PDSCH requirements for NR-U for Scenario A and Scenario C: - Proposals - Option 1: Section 5, with separate sub-clauses for scenario A and C (Huawei, Apple); - Option 2: Section 5 for scenario C and Section 9 for scenario A (Ericsson); - Recommended WF - TBA Issue 1-6-5: Whether to split the fixed reference channel in 4 tables, one for each DL duration: - Proposals - Option 1: Yes (Intel); - Option 2: No (Huawei, from draftCR); - Recommended WF - TBA Issue 1-6-6: Section in which to add CQI reporting requirements - Proposals - Option 1: 6.2, separately for Scenario A and C (Apple); - Option 2: 6.5 (Huawei, from draftCR); - Recommended WF - TBA Issue 1-6-7: Whether we need a new measurement channel for unlicensed carrier - Proposals - Option 1: No (Apple); - Option 2: Yes (Huawei, from draftCR); - Recommended WF - TBA Issue 1-6-8: Whether “…the last slot” should be replaced by “…the last slot in the downlink transmission duration” in all sentences where this change is applicable - Proposals - Option 1: Yes (Intel, Huawei); - Recommended WF - TBA |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2110938, MediaTek CR for TS38.101-4, PDSCH requirements for standalone NR-U | To be revised according to the comments collected in the first round; |
| R4-2109355, Apple Draft CR NRU CQI Scenario A-R16 | To be revised according to the comments collected in the first round; |
| R4-2109590, Ericsson Draft CR for TS38101-4 introduction of PDSCH demodulation requirements for NR-U Scenario A (catB)_pa1 | To be revised according to the comments collected in the first round; |
| R4-2110501, Huawei Draft CR for TS 38.101-4 Introduction of fixed reference channel of NR-U PDSCH | To be revised according to the comments collected in the first round; |
| R4-2110503, Huawei Draft CR for TS 38.101-4 Introduction of NR-U CQI requirements | To be revised according to the comments collected in the first round; |
| R4-2110719, Qualcomm DraftCR on NR-U UE Demodulation Downlink Transmission Model | To be revised according to the comments collected in the first round; |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #4: PDSCH Simulation Results

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109353 | Apple | PDSCH Simulation Results |
| R4-2109589 | Ericsson | PDSCH Simulation Results |
| R4-2110499 | Huawei | PDSCH Simulation Results |
| R4-2110767 | Qualcomm | PDSCH Simulation Results |
| R4-2110937 | MediaTek | PDSCH Simulation Results |
| R4-2110948 | Intel | PDSCH Simulation Results |

## Open issues summary

### Sub-topic 2-1: Simulation results for alignment

Issue 2-1-1: Simulation results summary

- Recommended WF

Companies are encouraged to fill their results in the summary document which will be shared via mail during the meeting (please see below in Section 5, ‘Existing TDocs’).

## Companies views’ collection for 1st round

### Open issues

Companies are encouraged to comment in the dedicated comment section below each issue.

## Summary for 1st round

### Open issues

### CRs/TPs

## Discussion on 2nd round (if applicable)

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| Big CR for the Introduction of NR-U UE Demodulation Requirements (PDSCH and CQI) | Qualcomm | Big CR to collect all the draftCRs presented in this meeting and be proposed for agreement. |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2109351 | Summary of simulation results for NR-U UE Demod | Apple |  | Companies to fill their PDSCH results |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents