3GPP TSG-RAN WG4 Meeting # 99-e 									    			    R4-2108645

Electronic Meeting, 19th – 27th May, 2021

Agenda item:			9.12.2

Source:	Samsung, CATT

Title:	Simulation assumptions for NTN co-existence study

Document for:	Information

- Introduction

This document captures initial simulation assumptions for the NTN coexistence study in frequency bands around 2GHz.

Remaining issues for further discussion are with [] and highlighted in yellow mark.

- Discussion

Co-existence simulation scenarios

It is proposed to have a phase-by-phase approach to conduct co-existence study considering scenarios.

The proposed scenarios for coexistence study are in the following table.

Table 2.1-1 Scenarios for NTN-NTN/TN co-existence

| FR1: 2GHz |  |  | Set 1 |  |  | Set 22 |  |  | HAPS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | GEO | LEO 600km | LEO 1200km | GEO | LEO 600km | LEO 1200km |  |
| NR / NB-IoT | Rural |  | X | X | X | X | X | X | FFS |
|  | Urban macro |  | X | X | X | X | X | X | FFS |
|  | Dense Urban |  | X | X | X | X | X | X | FFS |
| NTN1 | GEO3 | Set 1 | X | X | X | N/A | N/A | N/A | FFS |
|  | LEO 1200km |  | X | X | X | N/A | N/A | N/A | FFS |
|  | LEO 600km |  | X | X | X | N/A | N/A | N/A | FFS |
|  | GEO | Set 22 | N/A | N/A | N/A | X | X | X | FFS |
|  | LEO 1200km |  | N/A | N/A | N/A | X | X | X | FFS |
|  | LEO 600km |  | N/A | N/A | N/A | X | X | X | FFS |
| Note 1: Start with Earth Fixed beam first, Earth Moving Beams could be further discussed Note 2: Use Set 1 satellite antenna as the starting point for co-existence study. Set 2 might be used if any worst case in associate with Set 2 is found. Note 3: GEO and LEO only operate at adjacent channel. Note 4: Use GEO and LEO@600km when TN is victim. Note 5: Further check the possibility to remove LEO 1200km cases in future RAN4 meetings. |  |  |  |  |  |  |  |  |  |

The aggressor and victim combination is list in Table 2.1-2.

Table 2.1-2 Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL |  | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL |  | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL |  | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL |  | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with Band 34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with Band 34 TDD. | Phase 1 |
| 7 | TN with NTN | TN UL | NTN DL | Applicable for satellite operating in S band, e.g. coexistence with n41 TDD. | Phase 2 |
| 8 | TN with NTN | NTN DL | TN UL | Applicable for satellite operating in S band, e.g. coexistence with n41 TDD. | Phase 1 |
| 9 | NTN with NTN | NTN DL | NTN DL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |
|  |  | NTN UL | NTN UL | LEO-LEO | Phase 2 |
|  |  |  |  | GEO-GEO | Phase 2 |
|  |  |  |  | GEO-LEO@600 or HAPS-HAPS | Phase 2 |

The proposed frequency and bandwidth are listed as table 2.1-3.

Table 2.1-3.  Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- |
| TN Rural | 2 GHz | 20MHz | FDD, TDD | 1 |
| TN Urban macro | 2 GHz | 20MHz | FDD, TDD | 1 |
| TN Dense Urban | 2 GHz | 20MHz | FDD, TDD | 1 |
| GEO | 2 GHz | 5/10/15/20 MHz for FR1 | FDD | 1, 31 |
| LEO | 2 GHz | 5/10/15/20 MHz for FR1 | FDD | 1, 31 |
| HAPS | 2 GHz | TBD | FDD | [1] |
| Note 1: 2 phases will be considered for FRF: FRF=1 in phase 1 for simplification; FRF=3 in phase 2 or it is found FRF=1 is too stringent. |  |  |  |  |

Network layout model

Cellular cell structure is considered for both NTN and TN network layout.

Co-existence between NTN and TN

Coordination System

Referring to TR 38.811 Section 6.3 and Annex A, a 3D global coordinate system is considered (Earth-Centred Earth Fixed) for simulating NTN beams direction and location on the earth surface. It means the NTN beam location, TN randomly dropping location are generated with a set of three parameters (x,y,z).

Simulation Methodology

Following simulation steps can be used for NTN-TN co-existence study.

- Generate aggressor and victim networks.

- [NTN central beam is at satellite nadir, surrounded with 6 co-frequency beams. NTN FRFs higher than 1 need to be considered. Assume one NTN aggressor as default.

- TN center is randomly generated within the NTN central beam on earth surface.

- For following two cases, more TN sites might be needed due to large coverage per beam of NTN node. The number of TN networks needs further discussion.

Option 1: Figure 2.2.1-1 could be used to derive the number. Other options are not precluded. More cases will be discussed and added accordingly.

| No. | Combination | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | TN - NTN | TN DL (TN BS) | NTN UL (NTN satellite) |
| 2 | TN - NTN | TN UL (TN UE) | NTN UL (NTN satellite) |
| 3 | To be added |  |  |

Figure 2.2.1-1 The heterogeneous network layout

- Option 1-1: The heterogeneous network layout with all the sites can be simulated.

- Option 1-2: The heterogeneous network layout with parts of the sites can be simulated. After we get the interference levels of parts of the sites, we can calculate the interference levels of all the sites by scaling factor.

- Option 1-3: Following steps can be used to simplify the simulation

- Randomly generate TN system (19 sites with wraparound) within the coverage of satellite beams.

- Calculate the interference Pi from TN system to satellite. If the maximum number of loops I reached, go to the next step. Otherwise, go back to 1).

- Calculate the total interference:

Where,  is the coverage rate of TN systems, S0 is the coverage area of a TN system and S is the coverage area of satellite beams.

I is a parameter that balances the complexity and accuracy. Lager loop number I could promises better accuracy but costs more calculating resources and time.

- Calculate the performance of NTN UE. In this step, the interference from TN systems is expressed by Ptotal and no more TN sites are generated.

Option 2: For NTN-TN coexistence studies, one TN network (i.e., 19 cells * 3 sectors) is dropped randomly every snapshot with different location inside the NTN cell. TN network centres could be randomly droped among a location grid in order to cover all the locations in the NTN cell. Such location grid could be created by equal sampling points per side of the hexagon. As shown in Figure 2.2.1-2, 11 points per side is used and the distance between successive points is about 12 km.

Figure 2.2.1-2 Possible TN networks locations across one NTN cell

Option 3: Other options are no precluded.

- For other cases, 19-cell with wrap around will be used.

- When NTN cell is not at satellite nadir, following condition should be considered.

| No. | Combination | Aggressor | Victim | Which NTN cell/TN to consider? | Which TN cells to consider? | Additional remark |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | All TNs in NTN cell | All | How many active TNs? |
| 2 | TN with NTN | TN UL | NTN UL | All TNs in NTN cell? | All | How many active TNs? |
| 3 | TN with NTN | NTN DL | TN DL | - NTN cell at Nadir point: - TN randomly placed in the NTN cell. - TN at NTN cell edge if FRF=1. - NTN cell with satellite at low elevation: - TN randomly placed in the NTN cell - TN at NTN cell edge if FRF=1. | All |  |
| 4 | TN with NTN | NTN UL | TN UL | NTN cell at Nadir point: TN randomly placed in this cell | Only the TN cells hosting NTN UE(s) |  |
| 5 | TN with NTN | NTN UL | TN DL | - NTN cell at Nadir point: - TN randomly placed in the NTN cell - TN at NTN cell edge if FRF=1. - NTN cell with satellite at low elevation: - TN randomly placed in the NTN cell - TN at NTN cell edge if FRF=1. | All |  |
| 6 | TN with NTN | TN DL | NTN UL | All TNs in NTN cell | All | How many active TNs? |
| 7 | TN with NTN | TN UL | NTN DL | TBD |  |  |
| 8 | TN with NTN | NTN DL | TN UL | NTN cell at Nadir point: TN randomly placed in this cell | Only the TN cells hosting NTN UE(s) |  |
| 9 | NTN with NTN | NTN DL | NTN DL | TBD | NA |  |
|  |  | NTN UL | NTN UL | TBD | NA |  |
| Further clarification from Ericsson: So far, RAN4 has only considered NTN central beam at Nadir position. But for TN DL as victim, we propose to look also at satellite at low elevation angle, where the NTN central beam won’t be at Nadir position (see R4-21100119 for justification). Note that for the simulations alignment, only NTN central beam at Nadir point could be considered. |  |  |  |  |  |  |

]

- UE associations

- TN UE are generated randomly inside the TN network, make sure enough TN UEs are associated to each TN sectors based on coupling loss.

- Deployment of NTN UE to be further discussed considering following options

[Option 1: NTN UE is randomly generated within the TN area depending on the NTN UE density.

Option 2: Distribute the NTN UEs within the TN network boundaries or centers randomly corresponding to Table 1.

Table 1: NTN UE distribution mapping

| Aggressor | Victim | NTN UE distribution |
| --- | --- | --- |
| TN DL | NTN DL | NTN UEs at TN centers |
| TN UL | NTN UL | NTN UEs at TN boundaries |
| NTN DL | TN DL | NTN UEs at TN boundaries |
| NTN UL | TN UL | NTN UEs at TN centers |
| NTN UL | TN DL | NTN UEs at TN boundaries |
| TN DL | NTN UL | NTN UEs at TN centers |

Option 3: First decide/down-scope the coexistence scenarios (victim and aggressor) and then decide the NTN UE and TN UE distribution

Option 4:  NTN UE should be randomly generated within the NTN area. And when co-locate with TN network, co-ex study only consider those NTN UE dropped within the TN network. The simulation should follow similar Monte-Carlo approach as for TN simulation assumptions. And further discuss following factors:

  - Depending on who is the aggressor, and who is the victim, the simulation approach may be different. For generating interference, RAN4 simulations may consider multiple sources, but for measuring interference, RAN4 simulation may use one single destination in different locations.

  - The simulation assumptions should consider UEs connecting with higher priority to the network providing higher QoS.

  - The simulation assumptions could be further simplified with NTN UEs at the edge of TN network or outside TN network. ]

- Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

- Throughput is computed in the victim systems without considering ACI as below:

- ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

When TN UL is victim, only consider the TN cells hosting NTN UEs (i.e. impact should be checked only for TN UEs inside those TN cells. Here,  “TN cell” means 1 sector and “TN cell hosting NTN UE” means a TN sector where a NTN UEs is located.

- Throughput is computed considering ACI as below:

-	ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

- RF parameters are determined based on the degradation cause by ACI as below:

-	LossACI=1-ThputACIThputSINGLE.

Co-existence between NTN and NTN

[The following 2 cases are considered as candidate options and to be further discussed.

- One satellite carries two neighbour carriers, where the footprints of the 2 carriers are the same and coordinated see figure 2.2-1.

- Two satellites (GEO and LEO) operate on two neighbour carriers but at different height, see figure 2.2-2. The number of LEO satellite and footprints are FFS.

Figure2.1-1 Layout for coexistence between NTN and TN (TBD)

Figure 2.2-2 Layout for coexistence between NTN systems

Figure 2.2-3 Layout for coexistence between NTN systems (different height satellites)

]

Simulation parameters

NTN parameters

Satellite parameters

Two sets of satellite parameters are shown in Table 2.3-2 and Table 2.3-3 according to TR 38.821.

The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 2.3-1 NRB configuration per BandWidth size and SCS

| Configuration FR1 S-band | NRB (5MHz BW) | NRB (10MHz BW) | NRB (15MHz BW) | NRB (20MHz BW) |
| --- | --- | --- | --- | --- |
| SCS 15 kHz | 25 | 52 | 79 | TBA |
| SCS 30 kHz | 11 | 24 | 38 | TBA |

Table 2.3-2 Set-1 satellite parameters for co-existence study

| Satellite orbit |  |  | GEO |  |  |  | LEO-1200 |  |  |  | LEO-600 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km |  |  |  | 1200 km |  |  |  | 600 km |  |  |  |
| Payload characteristics for DL transmissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Satellite EIRP density |  | 2GHz | 59 dBW/MHz |  |  |  | 40 dBW/MHz |  |  |  | 34 dBW/MHz |  |  |  |
| Satellite max TX power in dBm | BW (MHz) |  | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 |
|  | SCS 15kHz |  | 44.53 | 47.71 | 49.53 | TBA | 46.53 | 49.71 | 51.53 | TBA | 40.53 | 43.71 | 45.53 | TBA |
|  | SCS 30kHz |  | 43.98 | 47.37 | 49.36 | TBA | 45.98 | 49.37 | 51.36 | TBA | 39.98 | 43.37 | 45.36 | TBA |
| Satellite Tx max Gain |  |  | 51 dBi |  |  |  | 30 dBi |  |  |  | 30 dBi |  |  |  |
| Channel bandwidth |  |  | 5/10/15/20MHz |  |  |  | 5/10/15/20MHz |  |  |  | 5/10/15/20MHz |  |  |  |
| 3dB beamwidth |  |  | 0.4011 deg |  |  |  | 4.4127 deg |  |  |  | 4.4127 deg |  |  |  |
| Satellite beam diameter |  |  | 250 km |  |  |  | 90 km |  |  |  | 50 m |  |  |  |
| Payload characteristics for UL transmissions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| G/T |  | 2 GHz | 19 dB K-1 |  |  |  | 1.1 dB K-1 |  |  |  | 1.1 dB K-1 |  |  |  |
| Satellite Rx max Gain |  |  | 51 dBi |  |  |  | 30 dBi |  |  |  | 30 dBi |  |  |  |

Table 2.3-2 Set-2 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | 2GHz | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi | 24 |
| Channel bandwidth |  | 5/10/15/20MHz | 5/10/15/20MHz | 5/10/15/20MHz |
| 3dB beamwidth |  | 0.7353 deg | 8.8320 deg | 8.8320 deg |
| Satellite beam diameter |  | 450 km | 190 km | 90 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| G/T | 2 GHz | 14 dB K-1 | -4.9 dB K-1 | -4.9 dB K-1 |
| Satellite Rx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |

Table 2.3-3 Other parameters for NTN

| Parameters | NTN | Remark |
| --- | --- | --- |
| Carrier frequency | 2GHz |  |
| The number of active UE (UL) | 3 | Same with TN |
| The number of active UE (DL) | 1 | Same with TN |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | See Session 2.6.2 |  |
| NTN satellite Noise figure in dB | See Table 2.3-3-1 |  |
| Handover margin | [0]/[3]dB |  |

Table 2.3-3-1 NTN satellite Noise figure in dB

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 19 | 1.1 | 1.1 |
| G_Rx (dBi) | 51 | 30 | 30 |
| NF (dB) | 7.4 | 4.3 | 4.3 |

UE parameters

UE parameters are shown in Table 2.3-4

Table 2.3-4 UE characteristics for system level simulations

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Rx Antenna gain | 0 dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 9 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | 0 dBi per element |

HAPS parameters

Refer to R4-2108646.

TN parameters

Table 2.3-5 Simulation assumptions of TN respectively based on NB-IoT and NR

|  | NB-IoT standalone | NR |  | NR |
| --- | --- | --- | --- | --- |
|  |  | Option 1 (R4-2106476 CATT) | Option 2 (R4-2105045 Samsung) | Agreed values |
| Carrier frequency in GHz | 2 | 2 | 2 | 2 |
| Size of each nominal channel BW in MHz | 0.2 | 20 | 20 | 20 |
| Transmission bandwidth in MHz | 0.18 | 9 |  | N/A |
| Environment | Urban macro Sub-urban Rural | Urban macro Sub-urban Rural | Deployment scenario related, check Table 2.3-6. | Deployment scenario related, check Table 2.3-6. |
| Network layout | 19-sites [57 sectors] with wrap-around | 19-sites [57 sectors] with wrap-around | 19-sites 57 sectors with wrap-around | 19-sites 57 sectors with wrap-around |
| Inter-site distance in meter | 500 for 2GHz band for UMA | 500 for 2GHz band for UMA | Deployment scenario related, check Table 2.3-6 | Deployment scenario related, check Table 2.3-6 |
| System loading and activity | Full buffer 100% | Full buffer 100% | Full buffer 100% | FFS |
| Network location | FFS | FFS | TN as victim: Randomly generated in NTN central beam | FFS |
| DL subcarrier spacing | 15kHz | 15kHz | 15kHz | 15kHz |
| UL | See RP-152284 | OFDMA | OFDMA | OFDMA |
| DL power control | No | No | No | No |
| UL power control | 36.942 section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB | 36.942 section 5.1.1.6 (set=1) | 36.942 Section 9.1 | 36.942 |
| Frequency reuse | 1 | 1 | 1 | 1 |
| Number of scheduled UE per cell (DL) | 1 | 1 | 1 | 1 |
| Number of scheduled UE per cell (UL) | 3 for multi-tone (60kHz per UE), 12 for 15kHz single-tone, 48 for 3.75kHz single-tone | 3 | 3 | 3 |
| UE antenna height in meter | 1.5 | 1.5 | Deployment scenario related, check Table 2.3.6 | 1.5m |
| UE TX power in dBm | -40 to 23 | -40 to 23 | -40 to 23 | -40 to 23 |
| UE antenna gain in dBi | 0 | 0 | 0 | 0 |
| Building penetration loss | 45.820 Annex D.1 | n/a | In Pathloss model, TR 38.901 | In pathloss model, TR 38.901 |
| Cell selection margin in dB | 3 | 3 | 3 | 3 |
| BS-MS min couple loss in dB | 70 | 70 | Proposed ‘Minimum BS-UE distance in meter’ instead of MCL. Deployment scenario related, check Table 2.3-6. | 70 for outdoor scenario in Table 2.1-1. |
| BS noise figure in dB | 5 | 5 | 5 | 5 |
| UE noise figure in dB | 9 | 9 | 9 | 9 |
| BS-UE path-loss model | TR36.942 macro urban | TR38.803 | TR 38.901 | TR 38.901 |
| Standard deviation of BS-UE log-normal shadow fading in dB | 10 | 10 | Deployment scenario related, referring to TR 38.901. | Deployment scenario related, referring to TR 38.901. |
| Shadowing correlation | Inter-cell 0.5 intra-cell 1 | Inter-cell 0.5 intra-cell 1 |  | Inter-cell 0.5 intra-cell 1 |
| Link-level performance model |  | section 2.9 |  | See Section 2.9 Throughtput-SINR mapping |
| UE distribution |  |  | Uniform | Uniform |
| Evaluation metrics | SINR vs ACS (as victim) | SINR and throughput loss vs standalone NB-IoT ACLR (as victim); | Throughput loss, referring to TR 38.803 section 5.2.7 | See Section 2.9 Throughtput or SNR loss criteria |

Table 2.3-6 Deployment-related parameters of TN (2 GHz) (Used with ‘Option-2’ above, R4-2105045 Samsung)

|  | Urban Macro | Suburban Macro | Rural Macro | Remarks |
| --- | --- | --- | --- | --- |
| Cell radius in meters | 500 | 1000 | 5000 | ITU-R Report M.2292 |
| BS Antenna height in meters | 25 | 30 | 30 |  |
| UE Parameters |  |  |  |  |
| UE Outdoor/indoor | 100% Outdoor |  |  |  |
| UE height in meter | 1.5 | 1.5 | 1.5 | 3GPP LS to ITU-R WP5D RP-200559 and ITU-R WP5D [IMT_Parameters] |
| Minimum BS-UE distance in meter | 35 | 35 | 35 |  |

Table 2.3-7 ACLR/ACS for TN  (2GHz)

|  |  | NR | NB-IOT |
| --- | --- | --- | --- |
| BS | ACLR | 45 dB | 40 dB |
|  | ACS | 45 dB | 45 dB |
| UE | ACLR | 30dB (ACLR1) 43dB (ACLR2) | 37 |
|  | ACS | 33 | 28 |

Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling of satellite could be referred to section 6.4.1 in TR 38.821.

Satellite antenna pattern

The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered

1

where J1(x) is the Bessel function of the first kind and first order with argument x, a is the radius of the antenna's circular aperture, k = 2f/c is the wave number, f is the frequency of operation, c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam. Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency.

The normalized gain pattern for a = 10 c/f (aperture radius of 10 wavelengths) is shown in Figure 2.4.1-1.

Figure 2.4.1-1: Satellite antenna gain pattern for aperture radius 10 wavelengths, a=10 c/f

Satellite and UE beam forming pattern

The following table is agreed for the beam layout definition for a single satellite simulation in S-Band.

Table 2.4.1-1: Beam layout definition for single satellite simulation

| Beam layout definition | Baseline: Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. Only the 3dB beam width parameters should be used. The beam diameter and beam spacing values can be computed directly from the 3 dB beam width assumptions and should be considered as informative. |
| --- | --- |
| Number of beams | Baseline: 7-beam layout (i.e. 6 co-frequency beams surrounding the central beam) |
| UV plane illustration (extracted from [19]) |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after: The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern: ABS = sqrt(3) x sin(HPBW/2 [rad]) |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam center is considered at nadir point |
| Frequency re-use factor | Option 1: 1 Option 2: 3 Option 3: 2 if polarization re-use is enabled |
| Polarization re-use | Option 1: Disable Option 2: Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UE distribution | The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers. |
| UE configuration | S-band: Handheld |
| UE orientation | Handheld: Random |
| UE attachment | RSRP |
| NOTE 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. NOTE 2: For the calibration purpose, the ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude). The atmospheric absorptions loss shall be considered. |  |

TN BS and UE antenna and beam forming pattern modelling

BS antenna

For AAS antenna, it refers to TR 38.921 pattern.

For non-AAS antenna, the parameter in Table 2.4.3-1 is used for 2GHz BS antenna pattern in the NTN system simulation.

Table 2.4.3-1 FR1 BS antenna pattern for 2GHz

| Parameter for BS | Values |
| --- | --- |
| Antenna vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=10°,SLAV=20dB |
| Antenna horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=20dB |
| Combining method for 3D antenna pattern (dB) |  |
| Maximum directional gain of an antenna GE,max | 17 dBi |

Only Non-AAS antenna can be used for NB-IoT.

UE antenna

For UE antennas, an omni-directional radiation pattern with antenna gain 0dBi is assumed.

Propagation model

Propagation model between NTN and UE

Propagation model between NTN and UE could be referred to section 6.6 in TR 38.811.

Propagation model between TN BS and UE

Propagation model between TN BS and UE could be referred to section 7.4 in TR 38.901.

Propagation model between NTN BS and TN BS

Propagation model between NTN BS and TN BS should reference to TS 38.811 which is used for DL-UL cross link interference for S band.

Transmission power control model

TN UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 could be applied for TN with following parameters.

Where, Pmax = 23dBm, Rmin = TBD dB, CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y,

where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

NTN UL TPC

For calibration purpose, reuse the same TN TPC for NTN with SNR target 15dB.

[For the coexistence study,

Option 1: Adopt the same TPC model of TN for NTN UL scenarios but needs to revise CLx-ile to align with UE UL power control parameters used in TR38.821. The CLx-ile value should be adapted for rural, dense urban and indoor scenarios.

Option 2: The CLx-ile value should be adapted for rural, dense urban and indoor scenarios.]

DL TPC

For downlink scenario, no power control scheme is applied.

Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

Where,

RX_PWR is the received power

TX_PWR is the transmitted power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

Performance metric

For NR,

The average throughput loss and 5%-ile throughput loss should be less than 5%.

For NB-IOT,

The average throughput loss and SNR loss, 5%-ile throughput loss and SNR loss should be according to 36.802

For NTN,

The average throughput loss and 5%-ile throughput loss should be less than 5%.

Throughput ~ SNR mapping

Adopt Section 5.2.7 of TR 38.803 as the SINR-Throughput performance metrics, but , SNIRMIN, and SNIRMAX need to be further studied and decided for NR NTN.

[Parameters below can be a starting point for α, SNIRMIN, and SNIRMAX, but other options are not precluded.

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | [0.6] | [0.4] | Represents implementation losses |
| SNIRMIN, dB | [-10] | [-10] | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | [30] | [22] | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

It’s infeasible to achieve 30dB/22dB DL/UL maximum SNIR for NR NTN, so the following parameters need to be further studied and RAN4 need to check them with RAN1.]

- Conclusion

It is proposed to use the simulation assumptions in this paper as the starting point for NTN co-existence study.

- Reference

[1] R4-2108700_Summary_313_2nd round

Annex 1. Interference Chart

Coexistence scenarios for NTN-TN scenarios are shown in Figure A.1, A.2 and A.3 below.

Figure A.1: S-band NTN-TN adjacent band coexistence scenarios with TN in FDD mode

Figure A.2: S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (e.g. n34)

Figure A.3: S-band NTN-TN adjacent band coexistence scenarios with TN in TDD mode (e.g. n41)