3GPP TSG-RAN WG4 Meeting#99      	R4-2108914

E-meeting, 19th – 27th  May, 2021

Title:	TP to TR 38.851 Applicability of CBM IBM for different CA configurations

Source:	Nokia, Nokia Shanghai Bell

Agenda item:	9.4.2.1.1

Document for:	Approval

# 1	Introduction

This TP captures the relevant agreements from [1]…[4] into TR 38.851 section 4.1 Applicability of CBM/IBM for different CA configurations.

# 2	Discussion

It is proposed to add these agreements in to the TR.

- IBM UE capability is applicable for all CA configurations [1]

- On “frequency group” [1]

- “frequency group” term shall not be defined in specification [1]

- If either CBM or IBM is concluded as infeasible for certain band combinations, it is reasonable to clearly state in the spec that only the requirements of feasible BM apply to these band combinations. If both CBM and IBM are concluded as feasible for certain band combinations, IBM/CBM is up to UE’s capability. [1]

- Typical inter-band CA deployment between bands in the same frequency group cannot be limited to co-located deployments [2]

- It is agreed that RAN4 will not label CBM or IBM as a default BM method for any band combination. Used beam management is based on UE capability. This issue is not discussed anymore in RAN4 [3]

- There are no deployment restrictions (Non-co-located/co-located) for network to configure inter-band DL CA for CBM UEs. [4]

- UE RF requirements for CBM shall be derived based on co-located deployment scenario only. [4]

# 3	Conclusion

In this contribution we are proposing to add relevant agreements on the applicability of CBM/IBM beam management for different CA configurations. TP is in the end of the contribution.

# 4 	References

[1] R4-2016915

[2] R4-2007813

[3] R4-2103111

[4] R4-2103399

******************************** TP to TR 38.851 **************************************

# 4	Inter-band DL CA

## 4.1	Applicability of CBM/IBM for different CA configurations

Following agreements have been reached concerning applicability of CBM/IBM for different CA configurations.

  - IBM UE capability is applicable for all CA configurations R4-2016915

  - On “frequency group” R4-2016915

  - “frequency group” term shall not be defined in specification

  - If either CBM or IBM is concluded as infeasible for certain band combinations, it is reasonable to clearly state in the spec that only the requirements of feasible BM apply to these band combinations. If both CBM and IBM are concluded as feasible for certain band combinations, IBM/CBM is up to UE’s capability. R4-2016915

  - Typical inter-band CA deployment between bands in the same frequency group cannot be limited to co-located deployments R4-2007813

  - It is agreed that RAN4 will not label CBM or IBM as a default BM method for any band combination. Used beam management is based on UE capability. This issue is not discussed anymore in RAN4 R4-2103111

  - There are no deployment restrictions (Non-co-located/co-located) for network to configure inter-band DL CA for CBM UEs. R4-2103399

  - UE RF requirements for CBM shall be derived based on co-located deployment scenario only. R4-2103399

******************************** End of TP **************************************