3GPP TSG-WG RAN4 Meeting #99-e	R4-2108990

# Electronic meeting, 19th – 27th May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5754 | rev | - | Current version: | 13.20.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for updates related to LTE band 24 in 36.101 (Rel-13) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_B24_mod-Core |  |  |  |  |  | Date: |  |  | 2021-5-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At RAN#91e, Band 24 requirements were updated with square brackets as placeholder for additional A-MPR regions and was assigned a NS value that conflicts with V2X |  |  |  |  |  |  |  |  |
| Summary of change: |  | Finalization of A-MPR regions and values, assignment of different NS value to avoid conflict with one assigned for V2X bands and update to the notes as agreed to in WF tdoc R4-2105371, update tolerance for MOP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 24 requirements in the specification will not comply with the updated regulations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.2.4, 6.6.3.3.35, Annex G2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2.2	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth for non CA configuration unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.2-1: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | ±2 |  |  |
| 2 |  |  |  |  | 23 | ±22 |  |  |
| 3 |  |  |  |  | 23 | ±22 |  |  |
| 4 |  |  |  |  | 23 | ±2 |  |  |
| 5 |  |  |  |  | 23 | ±2 |  |  |
| 6 |  |  |  |  | 23 | ±2 |  |  |
| 7 |  |  |  |  | 23 | ±22 |  |  |
| 8 |  |  |  |  | 23 | ±22 |  |  |
| 9 |  |  |  |  | 23 | ±2 |  |  |
| 10 |  |  |  |  | 23 | ±2 |  |  |
| 11 |  |  |  |  | 23 | ±2 |  |  |
| 12 |  |  |  |  | 23 | ±22 |  |  |
| 13 |  |  |  |  | 23 | ±2 |  |  |
| 14 | 31 | +2/-3 |  |  | 23 | ±2 |  |  |
| 17 |  |  |  |  | 23 | ±2 |  |  |
| 18 |  |  |  |  | 23 | ±25 |  |  |
| 19 |  |  |  |  | 23 | ±2 |  |  |
| 20 |  |  |  |  | 23 | ±22 |  |  |
| 21 |  |  |  |  | 23 | ±2 |  |  |
| 22 |  |  |  |  | 23 | +2/-3.52 |  |  |
| 23 |  |  |  |  | 236 | ±26 |  |  |
| 24 |  |  |  |  | 23 | +2/-32 |  |  |
| 25 |  |  |  |  | 23 | ±22 |  |  |
| 26 |  |  |  |  | 23 | ±22 |  |  |
| 27 |  |  |  |  | 23 | ±2 |  |  |
| 28 |  |  |  |  | 23 | +2/-2.5 |  |  |
| 30 |  |  |  |  | 23 | ±2 |  |  |
| 31 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | ±2 |  |  |
| 34 |  |  |  |  | 23 | ±2 |  |  |
| 35 |  |  |  |  | 23 | ±2 |  |  |
| 36 |  |  |  |  | 23 | ±2 |  |  |
| 37 |  |  |  |  | 23 | ±2 |  |  |
| 38 |  |  |  |  | 23 | ±2 |  |  |
| 39 |  |  |  |  | 23 | ±2 |  |  |
| 40 |  |  |  |  | 23 | ±2 |  |  |
| 41 |  |  |  |  | 23 | ±22 |  |  |
| 42 |  |  |  |  | 23 | +2/-3 |  |  |
| 43 |  |  |  |  | 23 | +2/-3 |  |  |
| 44 |  |  |  |  | 23 | +2/[-3] |  |  |
| 45 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | ±2 |  |  |
| 66 |  |  |  |  | 23 | ±2 |  |  |
| 68 |  |  |  |  | 23 | ±2 |  |  |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. NOTE 4: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 5: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB for transmission bandwidths confined within 815 MHz and 818 MHz. NOTE 6: When NS_20 is signalled, the total output power within 2000-2005 MHz shall be limited to 7 dBm. |  |  |  |  |  |  |  |  |

The default power class for an operating band is Power Class 3 unless otherwise stated.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 6.2.4	UE maximum output power with additional requirements

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2.2-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 1 and 3 the specific requirements and identified subclauses are specified in Table 6.2.4-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2.4.-1 to 6.2.4-15 are in addition to the allowed MPR requirements specified in subclause 6.2.3.

Table 6.2.4-1: Additional Maximum Power Reduction (A-MPR)

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.6.2.1.1 | Table 5.5-1 | 1.4, 3, 5, 10, 15, 20 | Table 5.6-1 | N/A |
| NS_03 | 6.6.2.2.1 | 2, 4,10, 23, 25, 35, 36, 66 | 3 | >5 | ≤ 1 |
|  |  |  | 5 | >6 | ≤ 1 |
|  |  |  | 10 | >6 | ≤ 1 |
|  |  |  | 15 | >8 | ≤ 1 |
|  |  |  | 20 | >10 | ≤ 1 |
| NS_04 | 6.6.2.2.2, 6.6.3.3.19 | 41 | 5, 10, 15, 20 | Table 6.2.4-4 |  |
| NS_05 | 6.6.3.3.1 | 1 | 10,15,20 | ≥ 50 (NOTE1) | ≤ 1 (NOTE1) |
|  |  |  | 15, 20 | Table 6.2.4-18 (NOTE2) |  |
|  |  | 65 (NOTE 3) | 10,15,20 | ≥ 50 | ≤ 1 (NOTE 1) |
|  |  |  | 15,20 | Table 6.2.4-18 (NOTE 2) |  |
| NS_06 | 6.6.2.2.3 | 12, 13, 14, 17 | 1.4, 3, 5, 10 | Table 5.6-1 | N/A |
| NS_07 | 6.6.2.2.3 6.6.3.3.2 | 13 | 10 | Table 6.2.4-2 |  |
| NS_08 | 6.6.3.3.3 | 19 | 10, 15 | > 44 | ≤ 3 |
| NS_09 | 6.6.3.3.4 | 21 | 10, 15 | > 40 | ≤ 1 |
|  |  |  |  | > 55 | ≤ 2 |
| NS_10 |  | 20 | 15, 20 | Table 6.2.4-3 |  |
| NS_11 | 6.6.2.2.1 6.6.3.3.13 | 23 | 1.4, 3, 5, 10, 15, 20 | Table 6.2.4-5 |  |
| NS_12 | 6.6.3.3.5 | 26 | 1.4, 3, 5, 10, 15 | Table 6.2.4-6 |  |
| NS_13 | 6.6.3.3.6 | 26 | 5 | Table 6.2.4-7 |  |
| NS_14 | 6.6.3.3.7 | 26 | 10, 15 | Table 6.2.4-8 |  |
| NS_15 | 6.6.3.3.8 | 26 | 1.4, 3, 5, 10, 15 | Table 6.2.4-9 Table 6.2.4-10 |  |
| NS_16 | 6.6.3.3.9 | 27 | 3, 5, 10 | Table 6.2.4-11, Table 6.2.4-12, Table 6.2.4-13 |  |
| NS_17 | 6.6.3.3.10 | 28 | 5, 10 | Table 5.6-1 | N/A |
| NS_18 | 6.6.3.3.11 | 28 | 5 | ≥ 2 | ≤ 1 |
|  |  |  | 10, 15, 20 | ≥ 1 | ≤ 4 |
| NS_19 | 6.6.3.3.12 | 44 | 10, 15, 20 | Table 6.2.4-14 |  |
| NS_20 | 6.2.2 6.6.2.2.1 6.6.3.3.14 | 23 | 5, 10, 15, 20 | Table 6.2.4-15 |  |
| NS_21 | 6.6.2.2.1 6.6.3.3.15 | 30 | 5, 10 | Table 6.2.4-16 |  |
| NS_22 | 6.6.3.3.16 | 42, 43 | 5, 10, 15, 20 | Table 6.2.4-17 |  |
| NS_23 | 6.6.3.3.17 | 42, 43 | 5, 10, 15, 20 | N/A |  |
| NS_24 | 6.6.3.3.20 | 65 (NOTE 4) | 5, 10, 15, 20 | Table 6.2.4-19 |  |
| NS_25 | 6.6.3.3.21 | 65 (NOTE 4) | 5, 10, 15, 20 | Table 6.2.4-20 |  |
| NS_26 | 6.6.3.3.22 | 68 | 10, 15 | Table 6.2.4-21 |  |
| ... |  |  |  |  |  |
| NS_32 | - | - | - | - | - |
| NS_36 | 6.6.3.3.28 | 68 | 5, 10, 15 | Table 6.2.4-27 |  |
| .. |  |  |  |  |  |
| NS_56 | 6.6.3.3.35 | 24 | 5, 10 | Table 6.2.4-34a |  |
| NOTE 1 Applicable when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is larger than or equal to the upper edge of PHS band (1915.7 MHz) + 4 MHz + the channel BW assigned, where channel BW is as defined in subclause 5.6. A-MPR for operations below this frequency is not covered in this version of specifications except for the channel assignments in NOTE2 as the emissions requirement in 6.6.3.3.1 may not be met. For 10MHz channel bandwidth whose carrier frequency is larger than or equal to 1945 MHz or 15 MHz channel bandwidth whose carrier frequency is larger than or equal to 1947.5 MHz, no A-MPR applies. NOTE 2 Applicable when carrier frequency is 1932.5 MHz for 15MHz channel bandwidth or 1930 MHz for 20MHz channel bandwidth case. NOTE 3: Applicable when the E-UTRA carrier is within 1920-1980 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980MHz. |  |  |  |  |  |

Table 6.2.4-2: A-MPR for "NS_07"

| Parameters | Region A |  | Region B |  | Region C |
| --- | --- | --- | --- | --- | --- |
| RBstart | 0 - 12 |  | 13 – 18 | 19 – 42 | 43 – 49 |
| LCRB [RBs] | 6-8 | 1 to 5 and 9-50 | ≥8 | ≥18 | ≤2 |
| A-MPR [dB] | ≤ 8 | ≤ 12 | ≤ 12 | ≤ 6 | ≤ 3 |
| NOTE 1; RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2; LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping between two regions, notes 1 and 2 apply on a per slot basis. NOTE 4; For intra-subframe frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for both slots in the subframe. |  |  |  |  |  |

<Unchanged sections omitted>

Table 6.2.4-34a: A-MPR for "NS_56"

| Channel bandwidth confined to 1627.5- 1637.5MHz |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | Carrier centre frequency (FC) (MHz) | Parameters |  |  | Region A | Region B | Region C | Region D | Region E | Region F | Region G |
| 5 MHz | 1630.0, 1630.3 | RBstart |  | ≤ 8 |  | ≤ 8 | N/A | N/A | N/A | N/A | N/A |
|  |  | LCRB [RBs] |  | ≤ 8 |  | > 8 | N/A | N/A | N/A | N/A | N/A |
|  |  | A-MPR [dB] |  | 8 |  | 2 | N/A | N/A | N/A | N/A | N/A |
|  | 1635.0 |  | No A-MPR needed |  |  |  |  |  |  |  |  |  |
|  | 1649.0 |  |  |  |  |  |  |  |  |  |  |  |
|  | 1654.0 |  |  |  |  |  |  |  |  |  |  |  |
| 10 MHz | 1632.5 | RBstart |  | ≤ 5 |  | ≤ 18 | ≤ 18 | ≥ 35 | ≥ 35 | ≥ 40 | ≥ 40 |
|  |  | LCRB [RBs] |  | ≤ 5 |  | ≤ 12 | > 12 | ≤ 7 | > 7 | ≤ 7 | > 7 |
|  |  | A-MPR [dB] |  | 7 |  | 5 | 7 | 4 | 2 | 5 | 3 |
|  | 1651.5 |  | No A-MPR needed |  |  |  |  |  |  |  |  |  |

<Unchanged sections omitted>

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 6.6.3.3.35	Minimum requirement (network signalled value “NS_56”)

When "NS_56" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.35-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.35-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10MHz |  |  |
| 1541 ≤ f ≤ 1559 | -102 | 2kHz | Averaged over any 2 millisecond active transmission interval |
| 1559≤ f ≤ 1608 | -85 | 700Hz |  |
| 1608≤ f ≤ 1610 | -85 +5/2 (f-1608) | 700Hz |  |
| 1610≤ f ≤ 1625 | -80+ 66/15 (f-1610) | 700Hz |  |
| 1541 ≤ f ≤ 1608 | -75 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1608≤ f ≤ 1610 | -75 + 5/2 (f-1608) | 1MHz |  |
| 1610≤ f ≤ 1627.5 | -70+ 57/17.5 (f-1610) | 1MHz |  |
| 1627.5 | -37 | 4kHz |  |
| 1638.5 ≤f ≤ 1645.5 | -28 | 4kHz |  |
| 1657.5 ≤f ≤ 1660.5 | -28 | 4kHz |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

# G.2		Typical receiver sensitivity performance (QPSK)

<Unchanged sections omitted>

Table G.2-3: Network Signalling Value for reference sensitivity

| E-UTRA Band | Network Signalling value |
| --- | --- |
| 2 | NS_03 |
| 4 | NS_03 |
| 10 | NS_03 |
| 12 | NS_06 |
| 13 | NS_06 |
| 14 | NS_06 |
| 17 | NS_06 |
| 19 | NS_08 |
| 21 | NS_09 |
| 23 | NS_03 |
| 24 | NS_56 |
| 30 | NS_21 |
| 35 | NS_03 |
| 36 | NS_03 |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------