3GPP TSG-RAN WG4 Meeting # 99-e	R4-2109358

# Electronic Meeting, May 19-27, 2021

Agenda item:	9.11.2.1

Source:	Apple Inc.

Title:	On PDSCH requirements in Intercell Interference scenarios

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#98bis-e PDSCH demodulation requirements for intercell interference with MMSE-IRC receiver was discussed and way forward [1] was agreed. In this contribution we present initial simulation results and our views on open issues related to test parameters.

# 2. Discussion

## Simulation Results

For the simulation assumptions in Table 1 below, we provide simulation results. As a baseline we consider 1 interfering cell. DIP based interference is chosen and the DIP is chosen based on requirements for PDSCH demodulation in TM9 in LTE.

Table 1: Simulation Parameters for PDSCH demodulation for ICI

| Parameter | Cell1 | Interferer Cell |
| --- | --- | --- |
| Duplex mode | FDD |  |
| DIP | N/A | -1.73dB |
| Active DL BWP index | 1 | N/A |
| CBW/SCS | 10 MHz/ 15 KHz |  |
| RB allocation | Full bandwidth |  |
| MIMO layer | Rank 1 | Rank 1,2 70% Rank 1 30% Rank 2 |
| Antenna Config and Correlation | 2x2, ULA Low |  |
| Channel | TDLA30-10 TDLC300-100 |  |
| PDSCH configuration |  |  |
| Mapping type | Type A | Type A |
| Starting symbol (S) | 2 | 2 |
| Length (L) | 12 | 12 |
| PDSCH DMRS configuration | Type 1 1+1 Port: 0 | Type 1 1+1 Port: 1 |
| Precoding Granularity | 2 PRB | 2 PRB |
| MCS | 4 (QPSK) 13 (16QAM) | 13 (16QAM) |
| HARQ Parameters |  |  |
| Number of HARQ Processes | 4-FDD 8-TDD | N/A |
| Maximum HARQ transmission | 4 | N/A |
| CSI-RS for Tracking |  |  |
| First subcarrier index in the PRB used for CSI-RS | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the PRB used for CSI-RS | l0 = [ 6 10] | (1) l0 = [6 10] for fully overlap (2) l0 = [5 9] for non- overlap (3) l0 = [6 10] for fully overlap w/ ZP CSI-RS for no interference |
| CSI-RS periodicity | 20ms | 20ms |
| CSI-RS offset | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |

One of the open issues related to test parameters was the TRS configuration. We provide results with 3 options for TRS:

Option 1: TRS/CSI-RS colliding with TRS/CSI-RS interference

Option 2: TRS/CSI-RS colliding with data interference

Option 3: TRS/CSI-RS with interference free

Figure 1: Performance in ICI for QPSK on target cell

Figure 2: Performance in ICI for 16QAM on target cell

For the baseline simulation assumptions, we don’t observe a significant delta in performance for different TRS configurations.

Observation #1: No significant performance delta is observed with no TRS interference, TRS interference with TRS or PDSCH.

In actual deployment scenarios it would be unreasonable to assume that there would be no interference on TRS. Also, we don’t see significant impact when TRS sees interference from either TRS or PDSCH from the interfering cell. It is practical to consider TRS of serving cell colliding with PDSCH with interference. Hence, we propose to define requirements with non-colliding TRS between target and interfering cell and with PDSCH from interference overlapped with TRS of serving cell.

Proposal #1: Define requirements with non-colliding TRS between target and interfering cell and with PDSCH from interference overlapped with TRS of serving cell.

## Test Scope for Inter-cell interference

Some of the open issues related to requirements in ICI:

- Network type for FDD

- Number of interfering cells

- Additional CBW for FDD and TDD

- SSB Configuration

- TRS Configuration

- Interference Model

For ICI scenarios the interference and target cells might not be synchronized but considering synchronized network would have the worse impact on UE performance. We don’t see the purpose of considering asynchronous network fo ICI scenarios.

Proposal #2: Only consider synchronized network for ICI requirements.

For the purpose of requirements definition using link level simulations, the number of interfering cells only affects the final SIR and SINR as experienced by the UE, other than that we don’t think any additional functionality in UE processing is verified by introducing more than 1 interfering cell. Hence, we propose to consider 1 interfering cell for PDSCH requirements with 1 interfering cell.

Proposal #3: Introduce requirements for intercell interference with 1 interferer cell

Another open issue for test scope was whether additional CBWs should be considered for ICI requirements. It is common understanding that performance doesn’t change significantly with different CBW. Hence, we don’t see the necessity to consider additional CBW and increase test definition efforts.

Proposal #4: Do not introduce requirements for ICI with additional BWs for TDD and FDD.

In [1] it was agreed to use DIP approach for interference profile definition and re-use LTE MMSE-IRC interference profiles. The interference profile adopted should be practical and applicable to NR deployments. If further study is needed to evaluate interference profiles for NR, RAN4 should further discuss and plan for such study. It would not be meaningful to define requirements with un-realistic interference profiles for NR. Perhaps using LTE NAICS interference profile as a starting point would also be a good idea since the NAICS interference profiles were defined more recently compared to DIP based profiles for LTE MMSE-IRC.

Proposal #5 RAN4 further discusses if interference profiles for NR need further study.

# 3. Conclusion

In this contribution we present initial simulation results and our views on open issues related to test parameters for PDSCH demodulation requirements for intercell interference with MMSE-IRC receiver. Our observations and proposals are captured below:

Observation #1: No significant performance delta is observed with no TRS interference, TRS interference with TRS or PDSCH.

Proposal #1: Define requirements with non-colliding TRS between target and interfering cell and with PDSCH from interference overlapped with TRS of serving cell.

Proposal #2: Only consider synchronized network for ICI requirements.

Proposal #3: Introduce requirements for intercell interference with 1 interferer cell.

Proposal #4: Do not introduce requirements for ICI with additional BWs for TDD and FDD.

Proposal #5 RAN4 further discusses if interference profiles for NR need further study.

# Reference

- R4-2106116, “WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, Intel.