3GPP TSG-RAN WG4 Meeting #99-e	R4-2109370

19 – 28 May 2021

Source:	Qualcomm Incorporated

Title:	Calculation of ΔRIB,V2X for V2X_n79A-n47A and V2X_n79A-47A

Agenda item:	8.26.2

Work Item:	          NR_LTE_V2X_PC5_combos-Core

Document for:	Approval

# 1.	Introduction

In RAN4#98bis-e new inter-band V2X combinations V2X_n79A-n47A and V2X_n79A-47A were introduced [1,2]. In this paper we would like to present the justification for the required ΔRIB,V2X for these combinations.

# 2. 	Discussion

In [1,2] the inter-band combinations V2X_n79A-n47A and V2X_n79A-47A were introduced. Though the frequency separation between these band is larger than 800MHz analysis has shown that there is extensive PA noise coupling from the TX of n79 into the RX of n47/B47 and TX of n47/B47 into the RX of band n79. The TX noise coupling into the RX bands causes RX sensitivity degradation. Below the PA noise and associated TX to RX filtering budget for each band is detailed using the performance of commercially available components and practically achievable isolation numbers.

Table 1: TX to RX Noise budget

|  | n79PA-->CV2x n47 Rx | CV2x n47/B47 PA-->n79 Rx | n79PA-->CV2x B47 Rx |
| --- | --- | --- | --- |
| BandWidth(MHz) | 10 | 40 | 10 |
| 3GPP Sensitivity(dBm) | -92.5 | -89.6 | -90.4 |
| Interfering PA Noise dBm/Hz | -112 | -112 | -112 |
| Tx-Rx Antenna Isolation | -20 | -20 | -20 |
| Tx Filter Rejection+ Triplexer Rej | -30 | -33 | -30 |
| Total PA noise (dBm) | -92 | -89 | -92 |
| 3GPP sensitivity+ PA noise (dBm) | -89.2 | -86.3 | -88.1 |
| De-Sense(dB) | 3.3 | 3.3 | 2.3 |

As can be seen from the above table the PA noise from n79 and n47/B47 leads to approximately 2.3 dB to 3.3 dB of desensitization of the respective RX bands. Therefore, it is proposed that the following ΔRIB,V2X values be adopted for V2X_n79A-n47A and V2X_n79A-47A:

Table 2 ΔRIB,V2X for V2X_n79A-47A

| V2X inter-band con-current band Combination | E-UTRA or NR Band | ΔRIB,V2X [dB] |
| --- | --- | --- |
| V2X_n79-47 | n79 | 3.3 |
|  | 47 | 2.3 |

Table 3 ΔRIB,V2X for V2X_n79A-n47A

| V2X inter-band con-current band Combination | E-UTRA or NR Band | ΔRIB,V2X [dB] |
| --- | --- | --- |
| V2X_n79-n47 | n79 | 3.3 |
|  | n47 | 3.3 |

We also note that these ΔRIB,V2X are optional and designs that have better performance do not need to adopt them. We make the following proposals:

Proposal 1: The ΔRIB,V2X given in table 2 be adopted for V2X_n79A-47A

Proposal 2: The ΔRIB,V2X given in table 3 be adopted for V2X_n79A-n47A

Also, it is our understanding that support for all V2X band combinations are optional and support for any that are made release independent to an earlier release is also optional in the earlier release. Therefore, we make the following proposals:

Proposal 3: Support of all V2X band combinations is optional

Proposal 4: Support of all V2X band combinations that are made release independent to an earlier release is optional in the earlier release

# Conclusion

We present the analysis for ΔRIB,V2X for inter-band combinations V2X_n79A-47A and V2X_n79A-n47A and make the following proposals. Also, proposals on the optionality of supporting inter-band V2X combinations are outlined.

Proposal 1: The ΔRIB,V2X given in table 2 be adopted for V2X_n79A-47A

Table 2 ΔRIB,V2X for V2X_n79A-47A

| V2X inter-band con-current band Combination | E-UTRA or NR Band | ΔRIB,V2X [dB] |
| --- | --- | --- |
| V2X_n79-47 | n79 | 3.3 |
|  | 47 | 2.3 |

Proposal 2: The ΔRIB,V2X given in table 3 be adopted for V2X_n79A-n47A

Table 3 ΔRIB,V2X for V2X_n79A-n47A

| V2X inter-band con-current band Combination | E-UTRA or NR Band | ΔRIB,V2X [dB] |
| --- | --- | --- |
| V2X_n79-n47 | n79 | 3.3 |
|  | n47 | 3.3 |

Proposal 3: Support of all V2X band combinations is optional

Proposal 4: Support of all V2X band combinations that are made release independent to an earlier release is optional in the earlier release

# References

[1] R4-2105367, CR for TS 38.101-1, Introduce new band combination of V2X_n79A-n47A, RAN4#98-bis, Apr. 12-20 2021

[2] R4-2105368, CR for TS 38.101-3, Introduce new band combination of V2X_n79A-47A, RAN4#98-bis, Apr. 12-20 2021