3GPP TSG-RAN WG4 Meeting # 99-e	R4-2109386

Electronic Meeting, 19 – 27 May 2021

Agenda Item:	11.9.2.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on BS RF requirements for support of 16QAM in NB-IoT

Document for:	Approval

# 1.	Introduction

The work item for additional enhancements for NB-IoT and LTE-MTC was approved at TSG RAN#88-e [1]. One of the objectives of this work item is to support 16QAM in NB-IoT unicast in UL and DL. BS RF requirements for support of 16QAM in NB-IoT were discussed at TSG RAN4#98-bis-e and a WF was approved [2].

This contribution provides further proposals on BS RF requirements for support 16-QAM in NB-IoT unicast in UL and DL according to the approved WF and the agreements in RAN1 [3].

# 2.	Discussion

2.1	NB-IoT RB power dynamic range for in-band or guard band operation or NB-IoT operation in NR in-band

It is stated in [2]:

- NB-IoT RB power dynamic range for in-band or guard band operation or NB-IoT operation in NR in-band

  - RAN4 will decide whether to use the same QPSK requirements for 16QAM.

To enhance NB-IoT coverage, the +6 dB power dynamic range is currently required for one NB-IoT QPSK RB for both E-UTRA in-band and guard band operation modes, except for guard band operation with E-UTRA 5 MHz channel bandwidth signal where BS manufacturer shall declare the NB-IoT dynamic range power it could support (as specified in TS 36.104 [4]). For NB-IoT operation in NR in-band, the +6 or +3 dB power dynamic range is currently required for one NB-IoT QPSK RB depending on the NB-IoT RB frequency position within the BS channel bandwidth, as shown in table 1 below (copied from Table 6.3.4.2-1 in TS 38.104 [5]).

Table 1: NB-IoT RB power dynamic range for NB-IoT operation in NR in-band

| BS channel bandwidth (MHz) | NB-IoT RB frequency position | NB-IoT RB power dynamic range (dB) |
| --- | --- | --- |
| 5, 10 | Any | +6 |
| 15 | Within center 77*180kHz+15kHz at each edge | +6 |
|  | Other | +3 |
| 20 | Within center 102*180kHz+15kHz at each edge | +6 |
|  | Other | +3 |
| 25, 30, 40, 50, 60, 70, 80, 90, 100 | Within center 90% of BS channel bandwidth | +6 |
|  | Other | +3 |

Considering NB-IoT coverage remains important for 16QAM, it is proposed to apply the NB-IoT RB current power dynamic range for QPSK RB transmission in NB-IoT to 16QAM RB for in-band or guard band operation or NB-IoT operation in NR in-band.

Proposal 1: The current NB-IoT RB power dynamic range for QPSK RB transmission in NB-IoT should apply to 16QAM RB for in-band or guard band operation or NB-IoT operation in NR in-band.

2.2	Error Vector Magnitude

It is stated in [2]:

- EVM limit for 16QAM DL

  - RAN4 will decide whether to define 12.5% as the limit

The EVM limit is currently specified in [4] as 17.5% for NB-IoT QPSK transmission, which is the same as E-UTRA QPSK transmission. Considering 16QAM has a higher target SNR than QPSK, it is proposed to specify a tighter EVM limit of 12.5% for NB-IoT 16QAM transmission, which is the same as E-UTRA 16QAM transmission.

Proposal 2: An EVM limit of 12.5% should be specified for NB-IoT 16QAM transmission.

2.3	Fixed Reference Channels for Receiver Characteristics

It is stated in [2]:

- 16QAM FRC for BS Rx Characteristics

  - RAN4 will decide whether new 16QAM FRC is needed for NB-IoT BS RF tests.

For E-UTRA, uplink FRCs using 16QAM are used for the receiver dynamic range requirement [4] because the higher required SNR comparing to QPSK would present a more stringent condition for the receiver dynamic range. Similarly, new uplink FRC using 16QAM would need to be defined for receiver dynamic range requirement of NB-IoT BS supporting 16QAM.

The UL TBS agreed in RAN1 for 16QAM in NB-IoT uplink is provided in table 2 below.

Table 2: TBS indices are introduced for 16QAM in NB-IoT uplink

| I_TBS | I_RU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

It has also been agreed in RAN1 that repetition is not used for 16QAM in NB-IoT uplink, where it can be used for 3, 6 and 12 subcarriers NPUSCH format 1. Considering the agreements in RAN1, and the expected more commonly use of 12 subcarriers for 16QAM in NB-IoT uplink, it is proposed to define the new uplink FRC for 16QAM as shown in table 3 below.

Table 3: FRC parameters for 16QAM in NB-IoT uplink

| Reference channel | Ax-y |
| --- | --- |
| Sub carrier spacing (kHz) | 15 |
| Number of tones | 12 |
| Modulation | 16-QAM |
| Diversity | No |
| Frequency offset | 0 |
| I_TBS / I_RU | 19 / 3 |
| Payload size (bits) | 1736 |
| Allocated resource units | 4 |
| Transport block CRC (bits) | 24 |
| Coding rate (target) | 3/4 |
| Coding Rate | 0.76 |
| Code block CRC size (bits) | 0 |
| Number of code blocks – C | 1 |
| Total symbols per resource unit | 144 |
| Total number of bits per resource unit | 576 |
| Tx time (ms) | 4 |
| Frequency offset | 0 |
| Channel estimation length (ms) Note 1 | 4 |
| Note 1: Channel estimation lengths are included in the table for information only. |  |

Proposal 3: To define the new uplink FRC for 16QAM as shown in table 3 above.

Preliminary link-level simulation has been performed using the FRC parameters in table 3 above, and the simulation results for SNIR required at 70% and 95% of maximum throughput are provided in Figure 1 below.

Figure 1: SNIR required at 70% and 95% of maximum throughput for 16QAM in NB-IoT uplink

# 3.	Conclusion

This contribution has provided further proposals on BS RF requirements for support 16-QAM in NB-IoT unicast in UL and DL according to the approved WF and the agreements in RAN1. They are summarized as follows.

Proposal 1: The current NB-IoT RB power dynamic range for QPSK RB transmission in NB-IoT should apply to 16QAM RB for in-band or guard band operation or NB-IoT operation in NR in-band.

Proposal 2: An EVM limit of 12.5% should be specified for NB-IoT 16QAM transmission.

Proposal 3: To define the new uplink FRC for 16QAM as shown in table 3 above.

# References

[1]	RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon.

[2]	R4-2105432, “WF on BS RF requirements for R17 NB-IoT 16QAM”, Huawei, HiSilicon, Ericsson.

[3]	RP-210534, “Status report for WI: Additional enhancements for NB-IoT and LTE-MTC”, Huawei.

[4]	3GPP TS 36.104: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.

[5]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”.