3GPP TSG-RAN WG4 Meeting # 99-e	R4-2109389

Electronic Meeting, 19 – 27 May 2021

Agenda Item:	11.1.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	TP to TR 37.880: Conclusion on feasibility studies for High-power UE Vs Public Safety operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71

Document for:	Approval

# 1.	Introduction

The study item on High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71 was approved at TSG RAN#88-e [1]. The purpose of this study item is to study RF requirements that are applicable for high power UE operation in LTE band 12 and band 5, and in NR band n71 for fixed wireless and vehicle-mounted use cases, in ITU Region 2.

Various studies have been performed during the period of this study item, and corresponding findings have been recorded into TR 37.880 [2]. This contribution provides the conclusion of this study item from the recorded findings and a text proposal for approval to include the conclusion into TR 37.880.

# 2.	Discussion

Various studies have been performed within this study item on coexistence between HPUE and adjacent channel services in the same geographical area, HPUE operation impacts on BS and UE performance, HPUE hardware requirements and implementation feasibility, as well as the required changes in the RAN4 specifications for HPUE requirements.

Coexistence studies between HPUE in Band 12, Band 5, and Band n71 and adjacent channel services in the same geographical area have been performed by analysis and simulation. No fundamental coexistence issue has been found between HPUE in Band 12, Band 5, and Band n71 and adjacent channel LTE, NR, MTC, and NB-IoT standalone operation in the same geographical area, and the impacts on NB-IoT in-band or guard band operation within the operator’s own spectrum can be handled using mitigation techniques like improved filtering or internal gap between the NB-IoT and LTE subcarriers. On the other hand, the simulation methodology and parameters for the system simulation for coexistence studies between HPUE in Band 5 and adjacent channel public safety operation in the same geographical area should be further discussed and agreed during the WI phase.

HPUE operation impacts on Band 12, Band 5, and Band n71 BS and UE performance have been studied in terms of UE transmitter harmonics and UE self-dense, as well as BS receiver blocking. No need has been identified for any additional UE requirements or relaxations due to uplink harmonics, nor any change in the BS receiver blocking requirements. On the other hand, UE REFSENS exception due to self-dense may be needed, and this should be further discussed and agreed during the WI phase.

HPUE hardware and software requirements in Band 12, Band 5, and Band n71 as well as implementation feasibility have been studied, and no major implementation issue has been found, given the target form factors for fixed-wireless/vehicle-mounted use cases.

The required changes in the RAN4 specifications for HPUE requirements in Band 12, Band 5, and Band n71 have been identified, including MOP, MPR, A-MPR, and ACLR. These requirements should be further discussed and agreed during the WI phase.

# 3.	Conclusion

This contribution has provided the conclusion if this study item from the recorded findings in TR 37.880. The text proposal to include the conclusion into TR 37.880 is provided below for approval.

# 4.	Text proposal

<Start of text proposal>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

3GPP	3rd Generation Partnership Project

ACLR	Adjacent Channel Leakage Ratio

A-MPR	Additional Maximum Power Reduction

BS	Base Station

BW	Bandwidth

CL	Coupling Loss

DL	Downlink

HPUE	High Power User Equipment

ITU	International Telecommunication Union

LTE	Long Term Evolution

MOP	Maximum Output Power

MPR	Allowed Maximum Power Reduction

MTC	Machine-Type Communications

NB-IoT	Narrowband – Internet of Things

NB	Narrowband

NR	New Radio

OOBE	Out Of Band Emissions

PA	Power Amplifier

PC	Power Class

PRB	Physical Resource Block

RAN4	Radio Access Network Working Group 4

REFSENS	Reference Sensitivity

RF	Radio Frequency

Rx	Receiver

SW	Software

TR	Technical Report

Tx	Transmitter

UE	User Equipment

<Next change>

## 9	Conclusion

Various studies have been performed within this study item on coexistence between HPUE and adjacent channel services in the same geographical area, HPUE operation impacts on BS and UE performance, HPUE hardware requirements and implementation feasibility, as well as the required changes in the RAN4 specifications for HPUE requirements.

Coexistence studies between HPUE in Band 12, Band 5, and Band n71 and adjacent channel services in the same geographical area have been performed by analysis and simulation. No fundamental coexistence issue has been found between HPUE in Band 12, Band 5, and Band n71 and adjacent channel LTE, NR, MTC, and NB-IoT standalone operation in the same geographical area, and the impacts on NB-IoT in-band or guard band operation within the operator’s own spectrum can be handled using mitigation techniques like improved filtering or internal gap between the NB-IoT and LTE subcarriers. On the other hand, the simulation methodology and parameters for the system simulation for coexistence studies between HPUE in Band 5 and adjacent channel public safety operation in the same geographical area should be further discussed and agreed during the WI phase.

HPUE operation impacts on Band 12, Band 5, and Band n71 BS and UE performance have been studied in terms of UE transmitter harmonics and UE self-dense, as well as BS receiver blocking. No need has been identified for any additional UE requirements or relaxations due to uplink harmonics, nor any change in the BS receiver blocking requirements. On the other hand, UE REFSENS exception due to self-dense may be needed, and this should be further discussed and agreed during the WI phase.

HPUE hardware and software requirements in Band 12, Band 5, and Band n71 as well as implementation feasibility have been studied, and no major implementation issue has been found, given the target form factors for fixed-wireless/vehicle-mounted use cases.

The required changes in the RAN4 specifications for HPUE requirements in Band 12, Band 5, and Band n71 have been identified, including MOP, MPR, A-MPR, and ACLR. These requirements should be further discussed and agreed during the WI phase.

<End of text proposal>

# References

[1]	RP-201261, “New SID on high-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71”, U.S. Cellular.

[2]	R4-2111007, “TR 37.880 V1.1.1: High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71”, Nokia, Nokia Shanghai Bell.