3GPP TSG-RAN WG4 Meeting #99e	R4-2109418

Electronic Meeting, 19th - 27th May, 2021

Source:	ZTE Corporation

Title:	A unified equation for specifying REFSENS for both TDD and FDD bands

Agenda Item:	8.27.2

Document for:	Discussion

# Introduction

In RAN4#98-bis-e, a WF was approved on splitting and simplifying UE REFSENS table for the sake of readability and more new channel bandwidth potentially added in the future [1] with two issues to be confirmed:

- FR1 REFSENS table in TS 38.101-1 Table 7.3.2-1 is modified based on the split table format proposed in [1] starting from Rel-17. - Add a note in REFSENS table for TDD bands to indicate that “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. - Companies are encouraged to verify the equation-based REFSENS formula for TDD bands in [1]. - Companies are also encouraged to further simplify the REFSENS table for FDD/SDL bands potentially with equation-based REFSENS representation.

- Verify the proposed equation-based REFSENS formula for TDD bands is correct

- Check if similar method could apply to the REFSENS table for FDD/SDL bands

In this contribution, we propose a unified formula applicable to both FDD and TDD bands, and subsequently resulting in a single simplified table.

# Discussion

During the verification process, we found that there is an unified formula applicable for all FDD and TDD bands for REFSENS values:

REFSENS=-100+10∙log10NRB∙SCSkHz∙124500+∆R

Where

- NRB is the maximum transmission bandwidth configuration for the concerned channel bandwidth and SCS.

- the REFSENS adjustment ∆R is a band-specific parameter as shown in below table:

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| REFSENS (dBm) = 10*log10(N_RB*12*SCS(kHz)/4500)+∆R-100 |  |  |  |  |
| Operating Band | SCS kHz | Channel bandwidth (MHz) | ∆R | Duplex Mode |
| n1 | 15 | 5,10,15,20,25,30,40,50 | 0 | FDD |  |
|  | 30 | 10,15,20,25,30,40,50 |  |  |  |
|  | 60 | 10,15,20,25,30,40,50 |  |  |  |
| n2 | 15 | 5,10,15,20 | 2 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
|  | 60 | 10,15,20 |  |  |  |
| n3 | 15 | 5,10,15,20,25,30,40 | 3 | FDD |  |
|  | 30 | 10,15,20,25,30,40 |  |  |  |
|  | 60 | 10,15,20,25,30,40 |  |  |  |
| n5 | 15 | 5,10,15,20 | 2 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
|  | 60 |  |  |  |  |
| n71 | 15 | 5,10,15,20,25,30,40,50 | 2 | FDD |  |
|  | 30 | 10,15,20,25,30,40,50 |  |  |  |
|  | 60 | 10,15,20,25,30,40,50 |  |  |  |
| n8 | 15 | 5,10,15,20 | 3 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
|  | 60 |  |  |  |  |
| n12 | 15 | 5,10,15 | 3 | FDD |  |
|  | 30 | 10,15 |  |  |  |
|  | 60 |  |  |  |  |
| n14 | 15 | 5,10 | 3 | FDD |  |
|  | 30 | 10 |  |  |  |
|  | 60 |  |  |  |  |
| n18 | 15 | 5,10,15 | 0 | FDD |  |
|  | 30 | 10,15 |  |  |  |
|  | 60 |  |  |  |  |
| n20 | 15 | 5,10,15,20 | 3 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
|  | 60 |  |  |  |  |
| n25 | 15 | 5,10,15,20,25,30,40 | 3.5 | FDD |  |
|  | 30 | 10,15,20,25,30,40 |  |  |  |
|  | 60 | 10,15,20,25,30,40 |  |  |  |
| n26 | 15 | 5,10,15,20 | 2.44 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
| n28 | 15 | 5,10,15,20,30 | 1.5 | FDD |  |
|  | 30 | 10,15,20,30 |  |  |  |
|  | 60 |  |  |  |  |
| n29x | 15 | 5,10 | 3 | SDL |  |
|  | 30 | 10 |  |  |  |
|  | 60 |  |  |  |  |
| n30 | 15 | 5,10 | 1 | FDD |  |
|  | 30 | 10 |  |  |  |
|  | 60 |  |  |  |  |
| n34 | 15 | 5,10,15 | 0 | TDD |  |
|  | 30 | 10,15 |  |  |  |
|  | 60 | 10,15 |  |  |  |
| n381 | 15 | 5,10,15,20,25,30,40 | 0 | TDD |  |
|  | 30 | 10,15,20,25,30,40 |  |  |  |
|  | 60 | 10,15,20,25,30,40 |  |  |  |
| n39 | 15 | 5,10,15,20,25,30,40 | 0 | TDD |  |
|  | 30 | 10,15,20,25,30,40 |  |  |  |
|  | 60 | 10,15,20,25,30,40 |  |  |  |
| n40 | 15 | 5,10,15,20,25,30,40,50 | 0 | TDD |  |
|  | 30 | 10,15,20,25,30,40,50,60,80 |  |  |  |
|  | 60 | 10,15,20,25,30,40,50,60,80 |  |  |  |
| n411 | 15 | 10,15,20,30,40,50 | 2 | TDD |  |
|  | 30 | 10,15,20,30,40,50,60,80,90,10 |  |  |  |
|  | 60 | 10,15,20,30,40,50,60,80,90,10 |  |  |  |
| n481 | 15 | 5, 10, 15, 20, 40, 505 | 1 | TDD |  |
|  | 30 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 |  |  |  |
|  | 60 | 10, 15, 20, 40, 505, 605, 705, 805, 905, 1005 |  |  |  |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | 0 | TDD |  |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 |  |  |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 |  |  |  |
| n51 | 15 | 5 | 0 | TDD |  |
|  | 30 |  |  |  |  |
|  | 60 |  |  |  |  |
| n53 | 15 | 5, 10 | 0 | TDD |  |
|  | 30 | 10 |  |  |  |
|  | 60 | 10 |  |  |  |
| n65 | 15 | 5,10,15,20,50 | 0.5 | FDD |  |
|  | 30 | 10,15,20,50 |  |  |  |
|  | 60 | 10,15,20,50 |  |  |  |
| n66 | 15 | 5,10,15,20,25,30,40 | 0.5 | FDD |  |
|  | 30 | 10,15,20,25,30,40 |  |  |  |
|  | 60 | 10,15,20,25,30,40 |  |  |  |
| n70 | 15 | 5,10,15,20,25 | 0 | FDD |  |
|  | 30 | 10,15,20,25 |  |  |  |
|  | 60 | 10,15,20,25 |  |  |  |
| n71 | 15 | 5,10,15,20 | 2.8 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
|  | 60 |  |  |  |  |
| n74 | 15 | 5,10,15,20 | 0.47 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
|  | 60 | 10,15,20 |  |  |  |
| n757 | 15 | 5,10,15,20,25,30,40,50 | 0 | SDL |  |
|  | 30 | 10,15,20,25,30,40,50 |  |  |  |
|  | 60 | 10,15,20,25,30,40,50 |  |  |  |
| n767 | 15 | 5 | 0 | SDL |  |
|  | 30 |  |  |  |  |
|  | 60 |  |  |  |  |
| n771,4 | 15 | 10, 15, 20, 25, 30, 40, 50 | 1.5 | TDD |  |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| n781 | 15 | 10, 15, 20, 25, 30, 40, 50 | 1 | TDD |  |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |  |
| n791 | 15 | 40, 50 | 1 | TDD |  |
|  | 30 | 40, 50, 60, 80, 100 |  |  |  |
|  | 60 | 40, 50, 60, 80, 100 |  |  |  |
| n91 | 15 | 5 | 0 | FDD |  |
|  | 30 |  |  |  |  |
|  | 60 |  |  |  |  |
| n92 | 15 | 5,10,15,20 | 0 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
|  | 60 |  |  |  |  |
| n93 | 15 | 5 | 0 | FDD |  |
|  | 30 |  |  |  |  |
|  | 60 |  |  |  |  |
| n94 | 15 | 5,10,15,20 | 0 | FDD |  |
|  | 30 | 10,15,20 |  |  |  |
|  | 60 |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. |  |  |  |  |  |
| NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |  |
| NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. |  |  |  |  |  |
| NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. |  |  |  |  |  |
| NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. |  |  |  |  |  |
| NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. |  |  |  |  |  |
| NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. |  |  |  |  |  |
| NOTE 8: NRB in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: The REFSENS value is rounded to the nearest number down to one decimal point. |  |  |  |  |  |

Compared with the initial proposal in [1], the method proposed in this contribution is applicable to all TDD bands and FDD bands based on a unified equation, thus there is no need to split the REFSENS table into two tables.

Proposal 1: Specify REFSENS based on a unified equation shown above.

Accordingly, the draft CR based on the proposal is shown in our companion paper [2].

Proposal 2: RAN4 endorse the draft CR based on the unified equation for specifying REFSENS.

# Conclusion

In this paper, we have the following proposals for specifying REFSENS:

Proposal 1: Specify REFSENS based on a unified equation shown above.

Proposal 2: RAN4 endorse the draft CR based on the unified equation for specifying REFSENS.

# Reference

R4-2105365, WF on REFSENS Table Split and Simplification, Apple

R4-210xxxx, draft CR for specifying REFSENS based on a unified equation method, ZTE