3GPP RAN WG4 Meeting #99e		R4-2109429

Electronic meeting, 19 – 27th May 2021

Agenda item:	8.1.1

Source:	Apple, Facebook, Hewlett Packard Enterprise, Skyworks Solutions Inc.

Title:	Band plan for lower 6GHz NR unlicensed operation in EU/CEPT

WI/SI:	NR_6GHz_unlic_EU

Release:	Rel-17

Document for:	Decision

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  While 5GHz is a well-known band for the unlicensed operation, 6GHz is a relative new band usage of which was approved recently in different regulatory regions. Referring to the latest 6GHz regulatory status update [2], FCC approved license-exempt usage in the frequency range of 5925-7125MHz, which has been already reflected by the corresponding 3GPP band n96. Several Administrations, such as South Korea and Brazil, have also opened up the whole range to the license-exempt operation. The regulatory framework for the EU/CEPT countries was completed in November covering the 5945-6425MHz frequency range.

Since the 6GHz regulatory framework for the CEPT countries is completed, 3GPP RAN#90 meeting decided to resume the corresponding WI [3] that aims at adding 3GPP support for the 6GHz band in that regulatory region. During RAN4#98bis meeting, several contributions were made elaborating further on whether 3GPP can leverage the existing band n96 or should add a new band [4]-[8]. RAN WG4 did not conclude on this matter and instead two options were captured in the final summary document. In this discussion paper we present our further technical analysis of the overall 6GHz ecosystem and how to support the 6GHz band in the EU/CEPT regulatory domain.

# 2	6GHz band

As already mentioned in the Introduction part, while several Administrations, such as US FCC and Korean MIST, have allocated the full range of 5925-7125MHz to the license-exempt operation, EU/CEPT countries will use a smaller portion of the 6GHz band for the unlicensed operation. As can be seen from the Figure 1 below, EU/CEPT 6GHz band roughly corresponds to the US U-NII-5 sub-band with the only difference that it starts at 5945MHz.  It also worth noting that the regulatory parameters of the EU/CEPT 6GHz band are close to the corresponding frequency range allocated by the Korean government for the similar low power indoor (LPI) and very low power (VLP) indoor/outdoor operation (refer to Table 2-1 below for more technical details).

Figure 2-1: Overview of the 6GHz unlicensed band in different regulatory regions.

Table 2-1: Summary of the CEPT 6GHz band regulatory requirements.

| Country | Permissible operation | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- |
| EU/CEPT | LPI: Restricted for indoor use only (including trains where metal coated windows are fitted and aircraft). Outdoor use (including in road vehicles) is not permitted. | 5945 - 6425MHz | 23dBm | 10dBm/MHz | -22 dBm/MHz (below 5935MHz) |
|  | VLP: Indoors and outdoors. Use on drones is prohibited. The VLP device is a portable device. |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) | -45 dBm/MHz (below 5935MHz); see NOTE1 |
|  | NOTE1: ECC will study the appropriateness of this level of OOBE by 31/12/2024. In absence of the justified evidence, a value of -37 dBm/MHz will be adopted from 1 January 2025 |  |  |  |  |

One of the first questions for introduction of the unlicensed operation on the 6GHz band in the EU/CEPT countries is the band plan for the 3GPP specifications. As presented in Figure 1, there already exists 3GPP band n96 covering the whole frequency range from 5925MHz to 7125MHz. And even though band n96 covers the whole frequency range of 5.875-7.125GHz (effectively supporting the 6GHz band in countries, such US, Korea, Brazil, etc), there is no technical obstacle for using the same band for a sub-range of frequencies in EU/CEPT. In fact, this is quite a common practice for 3GPP when an existing larger band is used to support a smaller band whereupon a larger and a smaller band can be even in different geographical regions. One of the most recent examples is band n77 being leveraged for the C-band in US. The fact that CEPT countries have a smaller frequency range should not cause any discrepancies because it is eventually up to the base station configuration on which frequencies NR is configured.

Observation 1: 	There is no technical issue with supporting EU/CEPT 5.945-6.425GHz frequency range with a larger band.

During the RAN4#98bis meeting, several EU/CEPT operators expressed a concern on leveraging existing band n96 due to the possibility that WRC-23 might identify some parts of the 6 GHz band for IMT in ITU Region 1. It was claimed that having a separate/new band in 3GPP for 5.945-6.425GHz will simplify adoption of IMT bands (if the latter has special protection requirements). Firstly, our view is that we cannot predict outcome of the regulatory discussions and thus 3GPP can only devise specifications following existing and officially approved regulatory decisions. In that sense CEPT has completed the regulatory framework for 5.945-6.425GHz, which has no out-of-band emission limits included into the ECC Decision to protect frequencies above 6425 MHz. Should CEPT decide to implement additional systems, including, but not limited to, the potential IMT identification above 6425 MHz, 3GPP has the corresponding framework with NS values that can activate the new requirements, if needed. Thus, leveraging band n96 for supporting the 6 GHz band in EU/CEPT countries will not create any technical obstacles for supporting new requirements.

Observation 2:	The ECC Decision does not have any OOBE limits to protect frequencies above 6425MHz.

Observation 3:	Should EU/CEPT release new regulatory requirements for the 5.945-6.425GHz frequency range, they can be supported with the corresponding new NS values for band n96.

From the UE implementation perspective same hardware components can be used to support band 6GHz, or parts thereof, with the only difference that in EU/CEPT a UE will be configured on different channels following what the NR system information broadcasts. Referring to Annex A, the regulatory parameters and usage of the 5.925-6.425GHz frequency range are very similar (if not identical) across all the major Administrations, and thus there is no technical obstacle in supporting this range with the same 3GPP band. As can be seen, EU/CEPT regulatory rules define LPI and VLP modes for the 5.945-6.425GHz range; and Korean and Brazil rules generally follow the same principle with the only difference that Brazil extends usage of two modes for the whole frequency range up to 7.125GHz. In that sense tailoring UE implementation to the EU/CEPT rules does not bring any benefit, while supporting the EU/CEPT 6GHz band with the existing band n96 will decrease time-to-market fostering band 6GHz usage in different regions and incentivising investments.

Proposal:	Leverage existing band n96 to support license-exempt usage of the 6GHz band in EU/CEPT countries.

# 3	Conclusions

In this discussion paper we have presented our initial views on the 6GHz unlicensed operation in the EU/CEPT countries and how the CEPT regulatory requirements can be supported in the 3GPP specifications. Our technical understanding is that existing 3GPP band n96 can be leveraged to support the 6GHz band in CEPT countries, for which the corresponding NS flag(s) will be added to differentiate between e.g. FCC and CEPT requirements.

Proposal:	Leverage existing band n96 to support license-exempt usage of the 6GHz band in EU/CEPT countries.

# 3	References

RP-1912926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

3GPP TR 37.890, "Feasibility Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations", v0.10.0

RP‐202592, "WID on introduction of lower 6GHz NR unlicensed operation for Europe", Nokia, Nokia Shanghai Bell

R4-2104882, "Band plan for lower 6GHz NR unlicensed operation for Europe", Apple Inc.

R4-2100546, "NRU in 6GHz EU spectrum: Band Definition and Related Emission Requirements", Skyworks Solutions Inc.

R4-2101965, "Discussion on Europe unlicensed 6GHz for NR-U", ZTE Corporation

R4-2107197, "On system parameters for the lower 6GHz NR unlicensed operation", Nokia, Nokia Shanghai Bell

R4-2102416, "UE requirements for EU NR-U 6 GHz band", Qualcomm Inc.

R4-2104884, "Supporting the 6GHz band in other countries/regions", Apple Inc.

Annex A: Summary of the 6GHz band regulatory parameters

| Country | Permissible operation | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- |
| US | Standard power: Outdoor with AFC. | 5925 - 6425MHz 6525 - 6875MHz | 36dBm (AP) 30dBm (CL) | 23dBm/MHz (AP) 17dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  | LPI: The access point devices cannot be weather resistant and shall have integrated antennas (prohibiting connection to other antennas). Operations in moving vehicles (cars, trains, aircraft) and drones are prohibited, with one exception in the U-NII-5 band for large passenger aircraft operating over 10,000 feet. | 5925 - 7125MHz | 30dBm (AP) 24dBm (CL) | 5 dBm/MHz (AP) -1 dBm/MHz (CL) |  |
| EU/CEPT | LPI: Restricted for indoor use only (including trains where metal coated windows are fitted and aircraft). Outdoor use (including in road vehicles) is not permitted. | 5945 - 6425MHz | 23dBm | 10dBm/MHz | -22 dBm/MHz (below 5935MHz) |
|  | VLP: Indoors and outdoors. Use on drones is prohibited. The VLP device is a portable device. |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) | -45 dBm/MHz (below 5935MHz); see NOTE1 |
|  | NOTE1: ECC will study the appropriateness of this level of OOBE by 31/12/2024. In absence of the justified evidence, a value of -37 dBm/MHz will be adopted from 1 January 2025 |  |  |  |  |

| Country | Permissible operation | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- |
| South Korea | LPI: A device shall use power supply from a wired connection, and a client battery powered device must communicate through the device connected to a wired power supply. This frequency range cannot be used for indoor automotive or airplane deployments. Usage inside moving vehicles is prohibited | 5925 - 7125MHz | 24dBm | 2dBm/MHz | -27 dBm/MHz (outside operational range) |
|  | VLP: Very low power indoor and outdoor usage. | 5925 - 6425MHz | 14dBm | 1dBm/MHz | -34 dBm/MHz (outside operational range) |
| Brazil | LPI: The AP equipment must be used only in an indoor environment. It must be powered directly by the electric power network, and battery power is not allowed. Their physical structures cannot be protected against bad weather. The operation is prohibited on oil extraction platforms, cars, trains, boats and aircraft, with the exception of operation in the 5.925-6.425 GHz band inside large aircraft flying above 3,048m. Client equipment must operate under the control of an indoor AP. | 5925 - 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  | VLP: Very low power indoor and outdoor usage |  | 17 dBm | -5 dBm/MHz |  |

| Country | Permissible operation | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- |
| UK | LPI: The Low Power Indoor apparatus may only be used within a building, onboard an aircraft or in any other enclosed space with attenuation characteristics at least as strong as those of either a building or an aircraft. | 5925 - 6425MHz | 24dBm | 11dBm/MHz | In accordance with directive 2014/53/EC |
|  | VLP: Very low power indoor and outdoor usage. |  | 14dBm |  |  |