3GPP TSG RAN WG4 Meeting #99-e	R4-2109463

Electronic Meeting, 19th – 27th May, 2021

Agenda item:	8.17.2

Source:	SoftBank Corp., Samsung, KDDI

Title:	TP update for TR 37.717-21-11: EN-DC_1-11_n41

Document for:	Approval

1	Introduction

TP for DC_1-11_n41 was approved in RAN4#98-bis-e [1]. This TP is to update its co-existence studies and REFSENS requirements.

2	Reference

[1]  R4-2105327  TP for TR 37.717-21-11: DC_1-11_n41, Samsung, KDDI

Annex.   Proposed text for TR37.717-21-11:

[Unchanged Parts Skipped]

5.x	DC_1-11_n41

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-11A_n41A | DC_1A_n41A DC_11A_n41A |

### 5.x.2	Co-existence studies

Based on co-existence studies of DC_1A-11A_n41A with 2UL, it can get that:

- IMD4 by dual uplink DC_1_n41 will fall into the own Rx band of Band 11

- IMD4 by dual uplink DC_11_n41 will fall into the own Rx band of Band 1

### 5.x.3	∆TIB and ∆RIB values

For DC_1 A-11A_n41A, the TIB,c and RIB,c values are proposed in the tables below.

Table 5.x.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11_n41 | 1 | 0.5 |
|  | 11 | 0.3 |
|  | n41 | 0.5 |

Table 5.x.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-11_n41 | 1 | 0 |
|  | 11 | 0 |
|  | n41 | 0 |

### 5.x.4	REFSENS requirements

Based the analysis on 5.x.2 above, MSD values due to IMD are shown below:

Table 5.x.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_1A-11A_n41A | 11 | 1442 | 5 | 25 | 1490 | N/A | FDD | N/A |
|  | n41 | 2520 | 10 | 50 | 2520 | N/A | TDD | N/A |
|  | 1 | 1966 | 5 | 25 | 2156 | 10.2 | FDD | IMD4 |
| DC_1A-11A_n41A | 1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | 11 | 1442 | 5 | 25 | 1490 | 10.6 | FDD | IMD4 |

[Unchanged Parts Skipped]