3GPP TSG-RAN WG4 Meeting # 99-e      		R4-2109491

Electronic Meeting, May. 19-27, 2021

Agenda Item:	9.11.3.1

Source: 	CMCC

Title: 	Discussion on BS demodulation enhancement for FR1 256QAM

Document for:	Discussion

- Introduction

In RAN4#98bis-e meeting, we discussed the test parameters of BS demodulation for FR1 256QAM, and a way forward was agreed in [1]. In this meeting, we will further discuss the remaining issues about test parameters.

- Discussion

  - Test parameters

Number of Tx

Agreement in RAN4#98bis-e meeting:

- Option 1: Only 1Tx

- Option 2: Both 1Tx and 2Tx

Number of Rx

Agreement in RAN4#98bis-e meeting:

- Option 1: 2/8 Rx

- Option 2: 2/4/8 Rx

Number of layers

Agreement in RAN4#98bis-e meeting:

- Option 1: Only 1 layer

- Option 2: Both of 1 and 2 layers

Applicability rule for different antenna configurations

Agreement in RAN4#98bis-e meeting:

- Option 1: Reuse the existing test applicability rule defined in clause 8.1.2.0 of TS38.141-1

- Other options

For above four issues, we propose to define 1Tx (1 layer) / 2Tx (2 layers), 2/4/8 Rx test cases to cover most of typical cases, and reuse the existing test applicability rule defined in clause 8.1.2.0 of TS 38.141-1 to reduce the test burden.

Proposal 1: For Tx, use both 1Tx and 2Tx

Proposal 2: For Rx, use 2Rx, 4Rx and 8Rx

Proposal 3: Use 1 layer for 1Tx and 2 layers for 2Tx.

Proposal 4: For the antenna configuration applicability rule, reuse the existing test applicability rule defined in clause 8.1.2.0 of TS 38.141-1.

SCS and bandwidth

Agreement in RAN4#98bis-e meeting:

- 15kHz SCS

  - Option 1: 5MHz and 10MHz

  - Option 2: 5MHz, 10MHz and 20MHz

- 30kHz SCS

  - Option 1: 10MHz and 40MHz

  - Option 2: 10MHz, 20MHz, 40MHz and 100MHz

Applicability rule for different SCS and CBW

Agreement in RAN4#98bis-e meeting:

- Option 1: Reuse the existing applicability rules defined in clause 8.1.2.1.1 and 8.1.2.1.2 of TS38.141-1

- Other options

For BW configurations, we propose to reuse the R15 64QAM configuration, which is Option 2 for both 15 and 30kHz SCS. In order to reduce the test burden, we propose to reuse the existing applicability rules defined in clause 8.1.2.1.1 and 8.1.2.1.2 of TS38.141-1.

Proposal 5: For 15kHz SCS, define 5MHz, 10MHz and 20MHz bandwidth configuration test cases.

Proposal 6: For 30kHz SCS, define 10MHz, 20MHz, 40MHz and 100MHz bandwidth configuration test cases.

Proposal 7: Reuse the existing applicability rules defined in clause 8.1.2.1.1 and 8.1.2.1.2 of TS38.141-1

- Conclusion

In this contribution, we discuss the test parameters for FR1 256QAM PUSCH demodulation requirements and provide our proposals. The proposals are:

Proposal 1: For Tx, use both 1Tx and 2Tx

Proposal 2: For Rx, use 2Rx, 4Rx and 8Rx

Proposal 3: Use 1 layer for 1Tx and 2 layers for 2Tx.

Proposal 4: For the antenna configuration applicability rule, reuse the existing test applicability rule defined in clause 8.1.2.0 of TS 38.141-1.

Proposal 5: For 15kHz SCS, define 5MHz, 10MHz and 20MHz bandwidth configuration test cases.

Proposal 6: For 30kHz SCS, define 10MHz, 20MHz, 40MHz and 100MHz bandwidth configuration test cases.

Proposal 7: Reuse the existing applicability rules defined in clause 8.1.2.1.1 and 8.1.2.1.2 of TS38.141-1

- Reference

[1] R4-2106120, Way forward for PUSCH 256QAM performance requirements, Huawei.

[2] RP-202000, New WID: Further enhancement on NR demodulation performance, China telecom.