3GPP TSG-RAN WG4 Meeting # 99-e	R4-2109634

Electronic Meeting, May. 19-27, 2021

Agenda Item: 9.7.2.1.2

Source: MediaTek Inc.

Title: Discussion on Rel-17 HST in FR1 for intra-frequency measurement

Document for: Discussion

- Introduction

In last meeting, some issues for the measurement in high speed train (HST) are identified and concluded, e.g., intra-frequency measurement for deactivated SCell. On the other hand, one remaining issue, i.e., Kp factor, is still under discussion without conclusion. Our views on these open issues are provided in this paper.

- Discussion

2.1 intra-frequency measurement without measurement gap

For intra-frequency measurement, one remaining issue about Kp factor is discussed without conclusion in last meeting. The purpose of Kp factor is that UE can measure the neighbour cells on SMTC outside measurement gaps when SMTC is partially overlapping with measurement gaps, where Kp = 1/(1- (SMTC period /MGRP)).

On the other hand, Kp is agreed to keep for activated SCell measurement for R17 HST in last meeting.

Agreement in RAN4 #98b-e - PSS/SSS detection delay, time index detection delay, and measurement period for activated SCell - Confirm the agreements in last meeting that PCell enhancement in Rel-16 HST are re-used to specify the requirements on PSS/SSS detection, time index detection and measurement period for activated Scell (including both intra-frequency measurement without MG and intra-frequency measurement with MG), i.e. the M2 and Y defined in Rel-16 HST are reused for activated Scell - Kp is kept for activated Scell measurement

For convenience of reference, the intra-frequency measurement for activated SCell in R17 HST is provided as follows. (Note that the requirement for intra-frequency measurement for activated SCell has been agreed to reuse the R16 HST requirement for PCell)

The measurement period of PCell for intra-frequency measurements without gaps in clause 9.2.5 in TS 38.133 Table 9.2.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1) DRX cycle; TPSS/SSS_sync_intra. No DRX; max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra. DRX cycle≤ 320ms; max( 600ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(5 x Kp) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1.. Table 9.2.5.1-3: Time period for time index detection (FR1) DRX cycle; TSSB_time_index_intra. No DRX; max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; Ceil(3 x Kp) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps(FR1) DRX cycle; T SSB_measurement_period_intra. No DRX; max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil( 5 x Kp ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Table 9.2.5.2-5: T SSB_measurement_period_intra When highSpeedMeasFlag-r16 is configured (Frequency range FR1) DRX cycle; T SSB_measurement_period_intra. No DRX Note 2; max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 160ms; max(200ms, ceil(5 x M2 Note 2 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. 160ms < DRX cycle≤ 320ms; ceil(4 x M2 Note 2 x Kp) x max(SMTC period,DRX cycle). DRX cycle>320ms; ceil( Y Note 3 x Kp ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 3: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms.

As shown in the above tables, the Kp factor is applied for activated SCell in HST and non-HST. However, it is observed that, in some cases, the measurement delay on deactivated SCell without scaling Kp is smaller than that of activated cells. In our understanding, the deactivated SCell should not be tighten than activated SCell. Thus, we propose to add Kp factor for deactivated SCell in HST and non-HST.

Proposal 1: Scaling factor Kp should be introduced in PSS/SSS detection, time index detection and measurement requirement for deactivated SCells in HST and non-HST.

- Summary

In this paper, the discussion of R17 HST in FR1 is provided. We have the following proposal:

Proposal 1: Scaling factor Kp should be introduced in PSS/SSS detection, time index detection and measurement requirement for deactivated SCells in HST and non-HST.