3GPP TSG-RAN WG4 Meeting # 99-e	R4-2109965

# Electronic Meeting, 19th – 27th May, 2021

Source: 	LG Electronics

Title: 	MPR results for NR PC2 intra-band non-contiguous CA UE according to RF architecture

Agenda Item:	9.3.2.5

Document for:	Approval

Introduction

In the RAN4#98BIS e-meeting, a way forward on simulation assumptions [1] for PC2 non-contiguous UL CA MPR/A-MPR was approved as follow.

RF architectures under discussion and MPR assumptions for intra-band UL NC CA are summarized as below PC2 non-contiguous UL CA architecture options and MPR assumption. Arch; description; MPR evaluation assumption. #1; 2x26dBm PA + 2LO with 100MHz BW; - PA calibration for 20MHz QPSK DFT-s-OFDM 100RB0 waveform based on 4dB post PA losses and 1dB MPR: - 26dBm/antenna: 29dBm at 31dB ACLR - 23dBm/antenna: 26dBm at 30dB ACLR - Equal PSD and Equal back-off power split - For 2PA architecture, 10dB antenna isolation is assumed - For 2PA architecture, Emission requirements are checked by summing the power of the two transmit paths - Waveform: both CP-OFDM and DFT-s-OFDM can be evaluated - CBW configuration: - 20MHz channel 15kHz SCS and 40MHz channel 15kHz SCS with a gap of 20MHz (100MHz class and in gap ACLR) - 40MHz channel 15kHz SCS and 40MHz channel 15kHz SCS with a gap of 120MHz (200MHz class) - 100MHz channel 30kHz SCS and 100MHz channel 30kHz SCS with a gap of 400MHz (600MHz class for n77(2A)) - RB allocation: - Worst case back-off IMD3 at -13dBm/MHz and -30dBm/MHz for 1RB+1RB at 15kHz and 30kHz SCS for MPR with 31dBc ACLR - The RB locations adopt for MPR evaluation in [2][8][9] can be referenced.. #2; 1x26dBm PA + 1LO with 200MHz BW. #3; 2x23dBm PA + 1LO with 200MHz BW. #4; 1x23dBm+1x26dBm + 2LO with 100MHz BW. - Companies to provide more MPR values to decide on the net power gain of PC2 in comparison with PC3 - If some architecture is beneficial in supporting UL MIMO, the same or different set of requirements can be considered - To further check if in-gap exceptions (for both PC3 and PC2) required by some architectures are allowed by regulations - Open issues - Issue 1: PA swap time for 1x23dBm+1x26dBm + 2LO with 100MHz BW - Proposals - Option 1: define new swap time specifically for this architecture - Option 2: swap time is 0us - Option 3: 0us or 35us or 140us - Issue 2: For 1x26dBm PA + 1LO with 200MHz BW and 2x23dBm PA + 1LO with 200MHz BW, how to handle in-gap requirement when LO or image fall inside? - Proposals - Option 1: Reuse in-gap exception under some conditions(e.g. Sync) as defined for PC3 - Option 2: use MPR to meet in-gap emission requirement - Option 3: other - Agreement: - Based on architecture #4, companies are encouraged to provide analysis on PA swap time UE can reach, and the impact of the swap time is evaluated on the remaining benefit versus PC3 for the different proposed swap time for PC2 intra-band UL NC CA. - Based on architecture #2 and #3, RAN4 to further check on in-gap requirements.

In this contribution, we provide initial MPR simulations results and expected swap time for PC2 intra-band UL non-contiguous CA UE.

Discussion

The UE architecture perspective, the #4 RF architecture with 1x23dBm+1x26dBm PAs can support UL non-contiguous CA with the separate class III as shown in Table 5.3A.5.2 in TS38.101-1.

Table 5.3A.5-2: NR intra-band non-contiguous UL CA frequency separation classes

| NR NC UL CA frequency separation class | Maximum allowed frequency separation |
| --- | --- |
| I | 100 MHz |
| II | 200 MHz |
| III | [600MHz] |

The #4 architecture also support UL-MIMO in single carrier compare to architecture #2 and #3. Furthermore the required MPR level is less than 2x23dBm PA architecture based on our simulation results. The MPR results for 26dBm PA in [2] is quite optimistic, RAN4 need to consider low cost UE to derive the MPR for PC2 intra-band CA UE. So the common 26dBm PA is baseline to derive MPR requirements for PC2 intra-band NC-CA UE.

Observation1: The #4 RF architecture with 1x23dBm+1x26dBm PAs can support UL non-contiguous CA with the separate class III. Also support UL-MIMO in single carrier compare to architecture #2 and #3.

MPR results for #4 (1x23dBm +1x26dBm) PA architecture

Firstly, we measured the required MPR results for intra-band NR non-contiguous RB allocation for 2PA #4 RF architecture (23dBm +26dBm with 2LOs). The measured MPR results are provided in Table 1 and Table 2 as follow

Table 1: Measured MPR results (23dBm + 26dBm, 2LOs) with -30dBm/MHz based on waveforms, modulation order and RB allocations

| NR CC1 20 MHz |  | NR CC2 40 MHz |  | DFT-S-OFDM | CP-OFDM | B (allocated RB portion in all aggregated CBW) |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | Allocated RB | SCS | allocated RB | MPR | MPR |  |
| 15KHz | 001R000 | 15KHz | 001R215 | 15.5 | 15.5 | 0.36 |
| 15KHz | 002R104 | 15KHz | 001R000 | 15.0 | 15.0 | 0.54 |
| 15KHz | 005R000 | 15KHz | 001R215 | 13.8 | 14.3 | 1.08 |
| 15KHz | 008R000 | 15KHz | 002R214 | 12.7 | 13.8 | 1.80 |
| 15KHz | 008R000 | 15KHz | 003R213 | 12.5 | 13.8 | 1.98 |
| 15KHz | 010R000 | 15KHz | 003R213 | 12.3 | 13.3 | 2.34 |
| 15KHz | 008R000 | 15KHz | 007R209 | 12.3 | 13.1 | 2.70 |
| 15KHz | 015R091 | 15KHz | 001R215 | 12.0 | 13.0 | 2.88 |
| 15KHz | 015R091 | 15KHz | 004R000 | 11.6 | 12.2 | 3.42 |
| 15KHz | 015R091 | 15KHz | 006R000 | 10.2 | 10.6 | 3.78 |
| 15KHz | 015R091 | 15KHz | 010R000 | 10.0 | 10.3 | 4.50 |
| 15KHz | 015R000 | 15KHz | 015R000 | 9.5 | 10.0 | 5.40 |
| 15KHz | 030R076 | 15KHz | 002R000 | 8.6 | 9.0 | 5.76 |
| 15KHz | 020R000 | 15KHz | 020R196 | 8.6 | 8.8 | 7.20 |
| 15KHz | 053R000 | 15KHz | 001R215 | 5.5 | 6.0 | 9.72 |
| 15KHz | 045R061 | 15KHz | 020R000 | 5.0 | 5.0 | 11.70 |
| 15KHz | 060R046 | 15KHz | 020R000 | 4.6 | 4.7 | 14.40 |
| 15KHz | 090R016 | 15KHz | 030R000 | 5.2 | 5.5 | 21.60 |
| 15KHz | 106R000 | 15KHz | 050R000 | 5.5 | 5.9 | 28.08 |
| 15KHz | 053R000 | 15KHz | 108R000 | 5.3 | 5.7 | 28.98 |
| 15kHz | 106R000 | 15kHz | 150R000 | 6.0 | 6.5 | 46.08 |
| 15KHz | 106R000 | 15KHz | 216R000 | 5.5 | 5.7 | 57.96 |
| NR CC1 40 MHz |  | NR CC2 40 MHz |  | DFT-S-OFDM | CP-OFDM | B (allocated RB portion in all aggregated CBW) |
| SCS | Allocated RB | SCS | Allocated RB | MPR | MPR |  |
| 15KHz | 001R000 | 15KHz | 001R215 | 15.5 | 15.5 | 0.36 |
| 15KHz | 001R000 | 15KHz | 001R000 | 15.0 | 15.2 | 0.36 |
| 15KHz | 002R214 | 15KHz | 001R000 | 14.2 | 14.7 | 0.54 |
| 15KHz | 005R000 | 15KHz | 001R215 | 14.0 | 14.5 | 1.08 |
| 15KHz | 008R000 | 15KHz | 002R214 | 13.5 | 13.8 | 1.80 |
| 15KHz | 008R000 | 15KHz | 003R213 | 13.0 | 13.0 | 1.98 |
| 15KHz | 010R000 | 15KHz | 003R213 | 12.4 | 12.6 | 2.34 |
| 15KHz | 015R201 | 15KHz | 001R000 | 11.9 | 11.9 | 2.88 |
| 15KHz | 015R201 | 15KHz | 004R000 | 11.5 | 11.8 | 3.42 |
| 15KHz | 015R201 | 15KHz | 006R000 | 11.2 | 11.2 | 3.78 |
| 15KHz | 015R201 | 15KHz | 010R000 | 10.7 | 10.8 | 4.50 |
| 15KHz | 015R000 | 15KHz | 015R000 | 10.0 | 10.0 | 5.40 |
| 15KHz | 030R186 | 15KHz | 002R000 | 8.8 | 9.0 | 5.76 |
| 15KHz | 020R000 | 15KHz | 020R196 | 8.8 | 8.8 | 7.20 |
| 15KHz | 030R000 | 15KHz | 024R192 | 7.8 | 8.0 | 9.72 |
| 15KHz | 045R171 | 15KHz | 020R000 | 5.5 | 5.8 | 11.70 |
| 15KHz | 070R146 | 15KHz | 030R000 | 4.5 | 4.5 | 18.00 |
| 15KHz | 108R000 | 15KHz | 001R000 | 4.4 | 4.5 | 19.62 |
| 15KHz | 108R000 | 15KHz | 108R000 | 4.0 | 4.0 | 38.88 |
| 15KHz | 150R066 | 15KHz | 070R000 | 3.0 | 3.3 | 39.60 |
| 15KHz | 150R000 | 15KHz | 150R066 | 3.0 | 3.0 | 54.00 |
| 15kHz | 216R000 | 15kHz | 216R000 | 3.0 | 3.0 | 77.76 |
| NR CC1 100 MHz |  | NR CC2 100 MHz |  | DFT-S-OFDM | CP-OFDM | B (allocated RB portion in all aggregated CBW) |
| SCS | Allocated RB | SCS | Allocated RB | MPR | MPR |  |
| 30KHz | 001R000 | 30KHz | 001R272 | 15.5 | 15.5 | 0.72 |
| 30KHz | 001R000 | 30KHz | 001R000 | 15.2 | 15.3 | 0.72 |
| 30KHz | 002R271 | 30KHz | 001R000 | 14.3 | 14.7 | 1.08 |
| 30KHz | 005R000 | 30KHz | 002R271 | 13.2 | 13.4 | 2.52 |
| 30KHz | 008R000 | 30KHz | 003R270 | 12.0 | 12.0 | 3.96 |
| 30KHz | 010R000 | 30KHz | 003R270 | 10.5 | 10.8 | 4.68 |
| 30KHz | 015R258 | 30KHz | 001R000 | 9.7 | 10.0 | 5.76 |
| 30KHz | 015R258 | 30KHz | 004R000 | 9.0 | 9.0 | 6.84 |
| 30KHz | 015R258 | 30KHz | 006R000 | 8.7 | 8.7 | 7.56 |
| 30KHz | 015R258 | 30KHz | 010R000 | 7.4 | 7.5 | 9.00 |
| 30KHz | 015R258 | 30KHz | 013R000 | 6.0 | 6.4 | 10.08 |
| 30KHz | 015R000 | 30KHz | 015R000 | 5.8 | 6.0 | 10.80 |
| 30KHz | 030R243 | 30KHz | 002R000 | 5.5 | 5.5 | 11.52 |
| 30KHz | 020R253 | 30KHz | 015R000 | 5.3 | 5.4 | 12.60 |
| 30KHz | 030R243 | 30KHz | 008R000 | 5.3 | 5.3 | 13.68 |
| 30KHz | 020R000 | 30KHz | 020R253 | 4.8 | 5.0 | 14.40 |
| 30KHz | 045R228 | 30KHz | 020R000 | 4.1 | 4.2 | 23.40 |
| 30KHz | 100R173 | 30KHz | 030R000 | 3.0 | 3.0 | 46.80 |
| 30KHz | 136R000 | 30KHz | 001R000 | 5.2 | 5.2 | 49.32 |
| 30KHz | 160R113 | 30KHz | 050R000 | 2.5 | 2.5 | 75.60 |
| 30KHz | 136R000 | 30KHz | 136R000 | 3.4 | 3.7 | 97.92 |
| 30KHz | 220R053 | 30KHz | 100R000 | 2.5 | 2.5 | 115.20 |
| 30KHz | 250R023 | 30KHz | 150R000 | 2.5 | 2.5 | 144.00 |
| 30KHz | 273R000 | 30KHz | 273R000 | 2.5 | 2.5 | 196.56 |

Table 2: Measured MPR results (23dBm + 26dBm, 2LOs) with -13dBm/MHz based on waveforms, modulation order and RB allocations

| NR CC1 20 MHz |  | NR CC2 40 MHz |  | DFT-S-OFDM | CP-OFDM | B (allocated RB portion in all aggregated CBW) |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | Allocated RB | SCS | allocated RB | MPR | MPR |  |
| 15KHz | 001R000 | 15KHz | 001R000 | 8.0 | 8.5 | 0.36 |
| 15KHz | 002R104 | 15KHz | 001R000 | 8.0 | 8.2 | 0.54 |
| 15KHz | 005R000 | 15KHz | 001R215 | 7.4 | 7.5 | 1.08 |
| 15KHz | 008R098 | 15KHz | 002R214 | 5.0 | 5.0 | 1.80 |
| 15KHz | 008R098 | 15KHz | 003R213 | 4.8 | 4.9 | 1.98 |
| 15KHz | 010R096 | 15KHz | 003R213 | 4.4 | 4.6 | 2.34 |
| 15KHz | 008R000 | 15KHz | 007R209 | 4.4 | 4.5 | 2.70 |
| 15KHz | 015R091 | 15KHz | 001R215 | 4.2 | 4.4 | 2.88 |
| 15KHz | 015R091 | 15KHz | 004R000 | 6.1 | 6.2 | 3.42 |
| 15KHz | 015R091 | 15KHz | 006R000 | 5.0 | 5.3 | 3.78 |
| 15KHz | 015R091 | 15KHz | 010R000 | 4.4 | 4.5 | 4.50 |
| 15KHz | 015R000 | 15KHz | 015R000 | 4.2 | 4.3 | 5.40 |
| 15KHz | 030R076 | 15KHz | 002R000 | 6.2 | 6.5 | 5.76 |
| 15KHz | 020R000 | 15KHz | 020R196 | 4.4 | 4.4 | 7.20 |
| 15KHz | 053R000 | 15KHz | 001R215 | 4.0 | 4.0 | 9.72 |
| 15KHz | 045R061 | 15KHz | 020R000 | 4.0 | 4.2 | 11.70 |
| 15KHz | 060R046 | 15KHz | 020R000 | 4.0 | 4.1 | 14.40 |
| 15KHz | 090R016 | 15KHz | 030R000 | 4.0 | 4.2 | 21.60 |
| 15KHz | 106R000 | 15KHz | 050R000 | 4.0 | 4.0 | 28.08 |
| 15KHz | 053R000 | 15KHz | 108R000 | 4.3 | 4.5 | 28.98 |
| 15kHz | 106R000 | 15kHz | 150R000 | 4.3 | 4.5 | 46.08 |
| 15KHz | 106R000 | 15KHz | 216R000 | 4.2 | 4.2 | 57.96 |
| NR CC1 40 MHz |  | NR CC2 40 MHz |  | DFT-S-OFDM | CP-OFDM | B (allocated RB portion in all aggregated CBW) |
| SCS | Allocated RB | SCS | Allocated RB | MPR | MPR |  |
| 15KHz | 001R215 | 15KHz | 001R000 | 6.3 | 6.5 | 0.36 |
| 15KHz | 001R000 | 15KHz | 001R000 | 5.5 | 5.8 | 0.36 |
| 15KHz | 002R214 | 15KHz | 001R000 | 5.3 | 5.5 | 0.54 |
| 15KHz | 005R000 | 15KHz | 001R215 | 5.2 | 5.5 | 1.08 |
| 15KHz | 008R000 | 15KHz | 002R214 | 5.0 | 5.2 | 1.80 |
| 15KHz | 008R000 | 15KHz | 003R213 | 4.5 | 4.8 | 1.98 |
| 15KHz | 010R000 | 15KHz | 003R213 | 4.5 | 4.8 | 2.34 |
| 15KHz | 015R201 | 15KHz | 001R000 | 4.3 | 4.5 | 2.88 |
| 15KHz | 015R201 | 15KHz | 004R000 | 4.2 | 4.3 | 3.42 |
| 15KHz | 015R201 | 15KHz | 006R000 | 3.8 | 4.0 | 3.78 |
| 15KHz | 015R201 | 15KHz | 010R000 | 3.8 | 4.1 | 4.50 |
| 15KHz | 015R000 | 15KHz | 015R000 | 4.0 | 4.5 | 5.40 |
| 15KHz | 030R186 | 15KHz | 002R000 | 3.7 | 3.8 | 5.76 |
| 15KHz | 020R000 | 15KHz | 020R196 | 3.6 | 3.8 | 7.20 |
| 15KHz | 030R000 | 15KHz | 024R192 | 3.6 | 3.6 | 9.72 |
| 15KHz | 045R171 | 15KHz | 020R000 | 2.5 | 2.8 | 11.70 |
| 15KHz | 070R146 | 15KHz | 030R000 | 2.6 | 2.7 | 18.00 |
| 15KHz | 108R000 | 15KHz | 001R000 | 2.6 | 3.0 | 19.62 |
| 15KHz | 108R000 | 15KHz | 108R000 | 2.8 | 3.0 | 38.88 |
| 15KHz | 150R066 | 15KHz | 070R000 | 2.6 | 2.7 | 39.60 |
| 15KHz | 150R000 | 15KHz | 150R066 | 2.3 | 2.5 | 54.00 |
| 15kHz | 216R000 | 15kHz | 216R000 | 2.3 | 2.5 | 77.76 |
| NR CC1 100 MHz |  | NR CC2 100 MHz |  | DFT-S-OFDM | CP-OFDM | B (allocated RB portion in all aggregated CBW) |
| SCS | Allocated RB | SCS | Allocated RB | MPR | MPR |  |
| 30KHz | 001R000 | 30KHz | 001R272 | 6.4 | 6.7 | 0.72 |
| 30KHz | 001R000 | 30KHz | 001R000 | 6.0 | 6.0 | 0.72 |
| 30KHz | 002R271 | 30KHz | 001R000 | 5.7 | 5.8 | 1.08 |
| 30KHz | 005R000 | 30KHz | 002R271 | 5.5 | 5.5 | 2.52 |
| 30KHz | 008R000 | 30KHz | 003R270 | 5.3 | 5.4 | 3.96 |
| 30KHz | 010R000 | 30KHz | 003R270 | 5.0 | 5.2 | 4.68 |
| 30KHz | 015R258 | 30KHz | 001R000 | 5.0 | 5.0 | 5.76 |
| 30KHz | 015R258 | 30KHz | 004R000 | 3.8 | 4.0 | 6.84 |
| 30KHz | 015R258 | 30KHz | 006R000 | 3.8 | 4.0 | 7.56 |
| 30KHz | 015R258 | 30KHz | 010R000 | 3.5 | 3.5 | 9.00 |
| 30KHz | 015R258 | 30KHz | 013R000 | 3.3 | 3.5 | 10.08 |
| 30KHz | 015R000 | 30KHz | 015R000 | 3.3 | 3.3 | 10.80 |
| 30KHz | 030R243 | 30KHz | 002R000 | 4.0 | 4.0 | 11.52 |
| 30KHz | 020R253 | 30KHz | 015R000 | 3.2 | 3.3 | 12.60 |
| 30KHz | 030R243 | 30KHz | 008R000 | 2.8 | 2.8 | 13.68 |
| 30KHz | 020R000 | 30KHz | 020R253 | 2.7 | 2.8 | 14.40 |
| 30KHz | 045R228 | 30KHz | 020R000 | 2.9 | 2.9 | 23.40 |
| 30KHz | 100R173 | 30KHz | 030R000 | 2.8 | 2.7 | 46.80 |
| 30KHz | 136R000 | 30KHz | 001R000 | 2.8 | 2.9 | 49.32 |
| 30KHz | 160R113 | 30KHz | 050R000 | 2.0 | 2.0 | 75.60 |
| 30KHz | 136R000 | 30KHz | 136R000 | 2.3 | 2.5 | 97.92 |
| 30KHz | 220R053 | 30KHz | 100R000 | 1.7 | 1.7 | 115.20 |
| 30KHz | 250R023 | 30KHz | 150R000 | 1.7 | 1.7 | 144.00 |
| 30KHz | 273R000 | 30KHz | 273R000 | 1.7 | 1.7 | 196.56 |

MPR results for #1 (2x26dBm) PA architecture

Secondly, we measured the required MPR results intra-band NR non-contiguous RB allocation for 2PA #1 RF architecture (26dBm +26dBm with 2LOs) in above section 1. The measured MPR results are provided in Table 3 and Table 4 as follow

Table 3: Measured MPR results (2x26dBm, 2LO) with -30dBm/MHz based on waveforms, modulation order and RB allocations

| NR CC1 20 MHz |  | NR CC2 40 MHz |  | DFT-S-OFDM | CP-OFDM | B (allocated RB portion in all aggregated CBW) |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | Allocated RB | SCS | allocated RB | MPR | MPR |  |
| 15KHz | 001R000 | 15KHz | 001R215 | 14.4 | 14.4 | 0.36 |
| 15KHz | 002R104 | 15KHz | 001R000 | 14.0 | 14.2 | 0.54 |
| 15KHz | 005R000 | 15KHz | 001R215 | 12.9 | 13.0 | 1.08 |
| 15KHz | 008R000 | 15KHz | 002R214 | 12.3 | 12.5 | 1.80 |
| 15KHz | 008R000 | 15KHz | 003R213 | 12.3 | 12.4 | 1.98 |
| 15KHz | 010R000 | 15KHz | 003R213 | 12.0 | 12.0 | 2.34 |
| 15KHz | 008R000 | 15KHz | 007R209 | 11.5 | 11.8 | 2.70 |
| 15KHz | 015R091 | 15KHz | 001R215 | 11.1 | 11.3 | 2.88 |
| 15KHz | 015R091 | 15KHz | 004R000 | 10.4 | 10.7 | 3.42 |
| 15KHz | 015R091 | 15KHz | 006R000 | 10.1 | 10.3 | 3.78 |
| 15KHz | 015R091 | 15KHz | 010R000 | 10.0 | 10.0 | 4.50 |
| 15KHz | 015R000 | 15KHz | 015R000 | 9.1 | 9.4 | 5.40 |
| 15KHz | 030R076 | 15KHz | 002R000 | 8.5 | 8.5 | 5.76 |
| 15KHz | 020R000 | 15KHz | 020R196 | 8.8 | 8.8 | 7.20 |
| 15KHz | 053R000 | 15KHz | 001R215 | 5.3 | 5.5 | 9.72 |
| 15KHz | 045R061 | 15KHz | 020R000 | 5.0 | 5.0 | 11.70 |
| 15KHz | 060R046 | 15KHz | 020R000 | 4.7 | 4.7 | 14.40 |
| 15KHz | 090R016 | 15KHz | 030R000 | 5.5 | 5.5 | 21.60 |
| 15KHz | 106R000 | 15KHz | 050R000 | 5.7 | 5.9 | 28.08 |
| 15KHz | 053R000 | 15KHz | 108R000 | 5.7 | 5.7 | 28.98 |
| 15kHz | 106R000 | 15kHz | 150R000 | 6.3 | 6.5 | 46.08 |
| 15KHz | 106R000 | 15KHz | 216R000 | 5.5 | 5.7 | 57.96 |
| NR CC1 40 MHz |  | NR CC2 40 MHz |  | DFT-S-OFDM | CP-OFDM | B (allocated RB portion in all aggregated CBW) |
| SCS | Allocated RB | SCS | Allocated RB | MPR | MPR |  |
| 15KHz | 001R000 | 15KHz | 001R215 | 14.5 | 14.6 | 0.36 |
| 15KHz | 001R000 | 15KHz | 001R000 | 14.3 | 14.4 | 0.36 |
| 15KHz | 002R214 | 15KHz | 001R000 | 13.6 | 13.7 | 0.54 |
| 15KHz | 005R000 | 15KHz | 001R215 | 13.8 | 13.8 | 1.08 |
| 15KHz | 008R000 | 15KHz | 002R214 | 13.0 | 13.0 | 1.80 |
| 15KHz | 008R000 | 15KHz | 003R213 | 12.0 | 12.2 | 1.98 |
| 15KHz | 010R000 | 15KHz | 003R213 | 12.0 | 12.0 | 2.34 |
| 15KHz | 015R201 | 15KHz | 001R000 | 11.0 | 11.1 | 2.88 |
| 15KHz | 015R201 | 15KHz | 004R000 | 10.5 | 10.8 | 3.42 |
| 15KHz | 015R201 | 15KHz | 006R000 | 10.2 | 10.3 | 3.78 |
| 15KHz | 015R201 | 15KHz | 010R000 | 10.0 | 10.1 | 4.50 |
| 15KHz | 015R000 | 15KHz | 015R000 | 8.8 | 8.8 | 5.40 |
| 15KHz | 030R186 | 15KHz | 002R000 | 8.0 | 8.0 | 5.76 |
| 15KHz | 020R000 | 15KHz | 020R196 | 7.7 | 7.8 | 7.20 |
| 15KHz | 030R000 | 15KHz | 024R192 | 7.5 | 7.5 | 9.72 |
| 15KHz | 045R171 | 15KHz | 020R000 | 4.9 | 4.8 | 11.70 |
| 15KHz | 070R146 | 15KHz | 030R000 | 4.1 | 4.0 | 18.00 |
| 15KHz | 108R000 | 15KHz | 001R000 | 4.0 | 4.0 | 19.62 |
| 15KHz | 108R000 | 15KHz | 108R000 | 3.4 | 3.4 | 38.88 |
| 15KHz | 150R066 | 15KHz | 070R000 | 2.6 | 2.7 | 39.60 |
| 15KHz | 150R000 | 15KHz | 150R066 | 2.4 | 2.4 | 54.00 |
| 15kHz | 216R000 | 15kHz | 216R000 | 3.0 | 3.0 | 77.76 |
| NR CC1 100 MHz |  | NR CC2 100 MHz |  | DFT-S-OFDM | CP-OFDM | B (allocated RB portion in all aggregated CBW) |
| SCS | Allocated RB | SCS | Allocated RB | MPR | MPR |  |
| 30KHz | 001R000 | 30KHz | 001R272 | 15.0 | 15.0 | 0.72 |
| 30KHz | 001R000 | 30KHz | 001R000 | 14.4 | 14.6 | 0.72 |
| 30KHz | 002R271 | 30KHz | 001R000 | 14.2 | 14.0 | 1.08 |
| 30KHz | 005R000 | 30KHz | 002R271 | 12.8 | 12.8 | 2.52 |
| 30KHz | 008R000 | 30KHz | 003R270 | 11.1 | 11.2 | 3.96 |
| 30KHz | 010R000 | 30KHz | 003R270 | 10.0 | 10.1 | 4.68 |
| 30KHz | 015R258 | 30KHz | 001R000 | 9.0 | 9.1 | 5.76 |
| 30KHz | 015R258 | 30KHz | 004R000 | 8.1 | 8.3 | 6.84 |
| 30KHz | 015R258 | 30KHz | 006R000 | 8.2 | 8.2 | 7.56 |
| 30KHz | 015R258 | 30KHz | 010R000 | 7.0 | 7.0 | 9.00 |
| 30KHz | 015R258 | 30KHz | 013R000 | 5.6 | 5.6 | 10.08 |
| 30KHz | 015R000 | 30KHz | 015R000 | 5.2 | 5.3 | 10.80 |
| 30KHz | 030R243 | 30KHz | 002R000 | 5.2 | 5.2 | 11.52 |
| 30KHz | 020R253 | 30KHz | 015R000 | 5.0 | 5.0 | 12.60 |
| 30KHz | 030R243 | 30KHz | 008R000 | 4.8 | 4.8 | 13.68 |
| 30KHz | 020R000 | 30KHz | 020R253 | 4.4 | 4.4 | 14.40 |
| 30KHz | 045R228 | 30KHz | 020R000 | 3.4 | 3.5 | 23.40 |
| 30KHz | 100R173 | 30KHz | 030R000 | 2.7 | 2.5 | 46.80 |
| 30KHz | 136R000 | 30KHz | 001R000 | 4.4 | 4.5 | 49.32 |
| 30KHz | 160R113 | 30KHz | 050R000 | 2.0 | 1.7 | 75.60 |
| 30KHz | 136R000 | 30KHz | 136R000 | 3.0 | 3.0 | 97.92 |
| 30KHz | 220R053 | 30KHz | 100R000 | 1.5 | 1.5 | 115.20 |
| 30KHz | 250R023 | 30KHz | 150R000 | 1.5 | 1.5 | 144.00 |
| 30KHz | 273R000 | 30KHz | 273R000 | 1.5 | 1.5 | 196.56 |

Table 4: Measured MPR results (26dBm + 26dBm, 2LOs) with -13dBm/MHz based on waveforms, modulation order and RB allocations

| NR CC1 20 MHz |  | NR CC2 40 MHz |  | DFT-S-OFDM | CP-OFDM | B (allocated RB portion in all aggregated CBW) |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | Allocated RB | SCS | allocated RB | MPR | MPR |  |
| 15KHz | 001R000 | 15KHz | 001R000 | 7.0 | 7.5 | 0.36 |
| 15KHz | 002R104 | 15KHz | 001R000 | 6.8 | 7.2 | 0.54 |
| 15KHz | 005R000 | 15KHz | 001R215 | 6.4 | 6.7 | 1.08 |
| 15KHz | 008R098 | 15KHz | 002R214 | 4.3 | 4.5 | 1.80 |
| 15KHz | 008R098 | 15KHz | 003R213 | 4.0 | 4.3 | 1.98 |
| 15KHz | 010R096 | 15KHz | 003R213 | 3.5 | 4.0 | 2.34 |
| 15KHz | 008R000 | 15KHz | 007R209 | 3.5 | 4.0 | 2.70 |
| 15KHz | 015R091 | 15KHz | 001R215 | 3.5 | 3.7 | 2.88 |
| 15KHz | 015R091 | 15KHz | 004R000 | 5.1 | 5.5 | 3.42 |
| 15KHz | 015R091 | 15KHz | 006R000 | 4.0 | 4.6 | 3.78 |
| 15KHz | 015R091 | 15KHz | 010R000 | 3.5 | 4.0 | 4.50 |
| 15KHz | 015R000 | 15KHz | 015R000 | 3.3 | 3.7 | 5.40 |
| 15KHz | 030R076 | 15KHz | 002R000 | 5.5 | 6.0 | 5.76 |
| 15KHz | 020R000 | 15KHz | 020R196 | 3.4 | 3.8 | 7.20 |
| 15KHz | 053R000 | 15KHz | 001R215 | 3.1 | 3.5 | 9.72 |
| 15KHz | 045R061 | 15KHz | 020R000 | 3.1 | 3.5 | 11.70 |
| 15KHz | 060R046 | 15KHz | 020R000 | 3.3 | 3.6 | 14.40 |
| 15KHz | 090R016 | 15KHz | 030R000 | 3.3 | 3.8 | 21.60 |
| 15KHz | 106R000 | 15KHz | 050R000 | 3.5 | 3.5 | 28.08 |
| 15KHz | 053R000 | 15KHz | 108R000 | 3.3 | 3.8 | 28.98 |
| 15kHz | 106R000 | 15kHz | 150R000 | 3.5 | 4.0 | 46.08 |
| 15KHz | 106R000 | 15KHz | 216R000 | 3.5 | 3.5 | 57.96 |
| NR CC1 40 MHz |  | NR CC2 40 MHz |  | DFT-S-OFDM | CP-OFDM | B (allocated RB portion in all aggregated CBW) |
| SCS | Allocated RB | SCS | Allocated RB | MPR | MPR |  |
| 15KHz | 001R215 | 15KHz | 001R000 | 5.3 | 5.5 | 0.36 |
| 15KHz | 001R000 | 15KHz | 001R000 | 5.0 | 5.0 | 0.36 |
| 15KHz | 002R214 | 15KHz | 001R000 | 5.0 | 5.2 | 0.54 |
| 15KHz | 005R000 | 15KHz | 001R215 | 5.0 | 5.2 | 1.08 |
| 15KHz | 008R000 | 15KHz | 002R214 | 4.5 | 4.8 | 1.80 |
| 15KHz | 008R000 | 15KHz | 003R213 | 4.2 | 4.3 | 1.98 |
| 15KHz | 010R000 | 15KHz | 003R213 | 4.0 | 4.3 | 2.34 |
| 15KHz | 015R201 | 15KHz | 001R000 | 3.5 | 3.8 | 2.88 |
| 15KHz | 015R201 | 15KHz | 004R000 | 3.6 | 3.7 | 3.42 |
| 15KHz | 015R201 | 15KHz | 006R000 | 3.3 | 3.4 | 3.78 |
| 15KHz | 015R201 | 15KHz | 010R000 | 3.5 | 3.7 | 4.50 |
| 15KHz | 015R000 | 15KHz | 015R000 | 3.5 | 4.0 | 5.40 |
| 15KHz | 030R186 | 15KHz | 002R000 | 3.2 | 3.2 | 5.76 |
| 15KHz | 020R000 | 15KHz | 020R196 | 3.0 | 3.2 | 7.20 |
| 15KHz | 030R000 | 15KHz | 024R192 | 3.0 | 3.2 | 9.72 |
| 15KHz | 045R171 | 15KHz | 020R000 | 2.0 | 2.2 | 11.70 |
| 15KHz | 070R146 | 15KHz | 030R000 | 2.0 | 2.2 | 18.00 |
| 15KHz | 108R000 | 15KHz | 001R000 | 2.0 | 2.7 | 19.62 |
| 15KHz | 108R000 | 15KHz | 108R000 | 2.2 | 2.7 | 38.88 |
| 15KHz | 150R066 | 15KHz | 070R000 | 2.2 | 2.3 | 39.60 |
| 15KHz | 150R000 | 15KHz | 150R066 | 2.0 | 2.0 | 54.00 |
| 15kHz | 216R000 | 15kHz | 216R000 | 2.0 | 2.0 | 77.76 |
| NR CC1 100 MHz |  | NR CC2 100 MHz |  | DFT-S-OFDM | CP-OFDM | B (allocated RB portion in all aggregated CBW) |
| SCS | Allocated RB | SCS | Allocated RB | MPR | MPR |  |
| 30KHz | 001R000 | 30KHz | 001R272 | 5.4 | 5.7 | 0.72 |
| 30KHz | 001R000 | 30KHz | 001R000 | 5.0 | 5.2 | 0.72 |
| 30KHz | 002R271 | 30KHz | 001R000 | 5.0 | 5.2 | 1.08 |
| 30KHz | 005R000 | 30KHz | 002R271 | 4.7 | 5.0 | 2.52 |
| 30KHz | 008R000 | 30KHz | 003R270 | 4.7 | 4.8 | 3.96 |
| 30KHz | 010R000 | 30KHz | 003R270 | 4.4 | 4.5 | 4.68 |
| 30KHz | 015R258 | 30KHz | 001R000 | 4.2 | 4.2 | 5.76 |
| 30KHz | 015R258 | 30KHz | 004R000 | 3.3 | 3.3 | 6.84 |
| 30KHz | 015R258 | 30KHz | 006R000 | 3.0 | 3.2 | 7.56 |
| 30KHz | 015R258 | 30KHz | 010R000 | 3.0 | 3.0 | 9.00 |
| 30KHz | 015R258 | 30KHz | 013R000 | 3.0 | 3.0 | 10.08 |
| 30KHz | 015R000 | 30KHz | 015R000 | 2.8 | 2.7 | 10.80 |
| 30KHz | 030R243 | 30KHz | 002R000 | 3.4 | 3.5 | 11.52 |
| 30KHz | 020R253 | 30KHz | 015R000 | 2.7 | 2.8 | 12.60 |
| 30KHz | 030R243 | 30KHz | 008R000 | 2.5 | 2.5 | 13.68 |
| 30KHz | 020R000 | 30KHz | 020R253 | 2.5 | 2.5 | 14.40 |
| 30KHz | 045R228 | 30KHz | 020R000 | 2.5 | 2.5 | 23.40 |
| 30KHz | 100R173 | 30KHz | 030R000 | 2.0 | 2.3 | 46.80 |
| 30KHz | 136R000 | 30KHz | 001R000 | 2.3 | 2.5 | 49.32 |
| 30KHz | 160R113 | 30KHz | 050R000 | 1.4 | 1.4 | 75.60 |
| 30KHz | 136R000 | 30KHz | 136R000 | 2.2 | 2.3 | 97.92 |
| 30KHz | 220R053 | 30KHz | 100R000 | 1.0 | 1.0 | 115.20 |
| 30KHz | 250R023 | 30KHz | 150R000 | 1.0 | 1.0 | 144.00 |
| 30KHz | 273R000 | 30KHz | 273R000 | 1.0 | 1.0 | 196.56 |

Based on the above MPR results in above sections. We can see the delta MPR value is not quite different as shown in and the Figure 2.1 between #1 PA architecture and #4 PA architecture for PC2 NR intra-band non-contiguous RB allocation.

Figure 2.1. Comparison MPR levels according to PA architecture

Obeservation2: Based on MPR results, the delta MPR value is not quite different (up to 1.5dB) between #1 PA architecture and #4 PA architecture.

Proposal

Proposal 1: Based on the MPR results, we propose following MPR requirements for PC2 NR intra-band non-contiguous CA UE

MPRIM3 to meet -30dBm/MHz

Based on simulation results and PC3 MPR requirements in TS38.101-1, we propose as follow

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as: MPR=MA Where MA is defined as follows MA = 16; 0 ≤ B < 1.08 15.0; 1.08 ≤ B < 2.88 14.0; 2.88 ≤ B < 5.40 12.0; 5.40 ≤ B < 9.72 10.5; 9.72 ≤ B < 16.38 9.0; 16.38 ≤ B Where: B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

MPRIM5 to meet -13dBm/MHz

For the MPR requirements with -13dBm/MHz, RAN4 can reuse the PC3 MPR requirements for PC2 intra-band non-contiguous CA as follow

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as: MPR=MA Where MA is defined as follows MA = 9 ; 0 ≤ B < 0.54 8 ; 0.54 ≤ B < 1.08 7 ; 1.08 ≤ B < 2.16 6.5 ; 2.16 ≤ B < 3.24 5.5 ; 3.24 ≤ B < 5.4 4 ; 5.4 ≤ B Where: B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

Other open issues

For the #4 architecture, RAN4 discussed whether or not need the PA swapping time from one CC to two CC as follow

- Issue 1: PA swap time for 1x23dBm+1x26dBm + 2LO with 100MHz BW

  - Proposals

    - Option 1: define new swap time specifically for this architecture

    - Option 2: swap time is 0us

    - Option 3: 0us or 35us or 140us

In our understanding, the PA swapping time does not need for intra-band non-contiguous CA since each PA per CC always operated for intra-band non-contiguous CA operation.

Maybe, the swapping time is needed when CA UE fallback to single CC or the UE support UL-MIMO without CA operation.

Therefore, we don’t need to define the PA swapping time for intra-band NC-CA UE.

If our understanding is not correct, then we can further discuss the detail operating scenarios and UE behavior.

Observation3: The PA swapping time does not need for intra-band non-contiguous CA since each PA per CC operate for intra-band non-contiguous CA operation.

For the in-gap emission requirements, RAN4 proposed 3 candidate options as follow

- Issue 2: For 1x26dBm PA + 1LO with 200MHz BW and 2x23dBm PA + 1LO with 200MHz BW, how to handle in-gap requirement when LO or image fall inside?

  - Proposals

    - Option 1: Reuse in-gap exception under some conditions(e.g. Sync) as defined for PC3

    - Option 2: use MPR to meet in-gap emission requirement

    - Option 3: other

In these options, we prefer to option1 as same PC3 intra-band non-contiguous CA UE.

Conclusion

In this contribution, we provide some observations and propose MPR requirements for PC2 NR intra-band non-contiguous CA UE according to RF architecture as follow

Observation1: The #4 RF architecture with 1x23dBm+1x26dBm PAs can support UL non-contiguous CA with the separate class III. Also support UL-MIMO in single carrier compare to architecture #2 and #3.

Obeservation2: Based on MPR results, the delta MPR value is not quite different (up to 1.5dB) between #1 PA architecture and #4 PA architecture.

Observation3: The PA swapping time does not need for intra-band non-contiguous CA since each PA per CC operate for intra-band non-contiguous CA operation.

Proposal 1: Based on the MPR results, we propose following MPR requirements for PC2 NR intra-band non-contiguous CA UE

- -	MPRIM3 to meet -30dBm/MHz

Based on simulation results and PC3 MPR requirements in TS38.101-1, we propose as follow

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as: MPR=MA Where MA is defined as follows MA = 16; 0 ≤ B < 1.08 15.0; 1.08 ≤ B < 2.88 14.0; 2.88 ≤ B < 5.40 12.0; 5.40 ≤ B < 9.72 10.5; 9.72 ≤ B < 16.38 9.0; 16.38 ≤ B Where: B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

- -	MPRIM5 to meet -13dBm/MHz

For the MPR requirements with -13dBm/MHz, RAN4 can reuse the PC3 MPR requirements for PC2 intra-band non-contiguous CA as follow

MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as: MPR=MA Where MA is defined as follows MA = 9 ; 0 ≤ B < 0.54 8 ; 0.54 ≤ B < 1.08 7 ; 1.08 ≤ B < 2.16 6.5 ; 2.16 ≤ B < 3.24 5.5 ; 3.24 ≤ B < 5.4 4 ; 5.4 ≤ B Where: B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

- References

[1]	R4-2105389 “WF on RF architecture options handling for PC2 intra-band UL NC CA,” Huawei

[2] R4-2104819, “Discussion on Transmitter Architecture for PC2 UL NC CA”, Skyworks, RAN4 #98bis-e

[3] R4-2107282, “Discussion on possible reference architecture for nc ul ca”, Qualcomm, RAN4 #98bis-e

[4] R4-2106366, “On PC2 intra-band non-contiguous NR CA”, ZTE, RAN4 #98bis-e

[5] R4-2106542, “Discussion on HP UE for TDD intra-band UL CA”, Xiaomi, RAN4 #98bis-e

[6] TS38.101-1