3GPP TSG-RAN WG4 Meeting #99-e														R4-2110173

Online, 19 – 27 May, 2021

Source:	Intel Corporation

Title:	On frequency range definition for NR 52.6 – 71 GHz

Agenda item:	9.15.7

Document for:	Discussion

# Background

In RAN#90-e, the definition and notations of the frequency range 52.6 – 71 GHz were discussed. Email discussion summarized as follows:

- Introduce FR3: Nokia, Charter Communications, Samsung, OPPO, InterDigital, Lenovo, Motorola Mobility, Qualcomm, Apple, Sony, AT&T (11)​

- Reuse FR2: CATT, Ericsson (2)​

- Leave it to RAN4 decision: Intel, FUTUREWEI, ZTE, LG Electronics, Huawei, HiSilicon (6)​

- Moderator’s proposal: Considering the RAN4’s workload, RAN plenary makes decision to introduce FR3 for frequency range 52.6-71GHz to save time in RAN

As the outcome of the discussion, the latest approved WID [1] captured the plenary guidance as follows:

“Note 5: RAN plenary will decide whether new FR (e.g. FR3) shall be defined for the frequency range from 52.6GHz-71GHz or the existing FR2 shall be extended to cover frequency range from 52.6GHz-71GHz.”

In RAN#91-e, there were extensive discussions without conclusion, and plenary tasked each working group to discuss and recommend the decision to the upcoming June plenary RAN #92e meeting.

“RAN1, RAN2 and RAN4 are asked to provide its analysis or recommendation to RAN#92E (June) on how to introduce the 52.6-71GHz frequency range”

In RAN4 #98-bis-e further discussion took place. No formal conclusions on the FR definition were made and the following options were discussed:

- Option 1: Extend FR2 to cover 60 GHz range

- Option 2: Introduce 60 GHz as FR2a

- Option 3: Rename the existing FR2 as FR2a and introduce 60 GHz as FR2b

- Option 4: FR3

Meantime, most of companies wanted to minimize spec work and it was agreed not to introduce a new spec for 60 GHz (i.e., include 60 GHz in the existing UE and BS specs). To facilitate further discussion in RAN plenary RAN4 needs to agree on the common understanding and inform RAN during the RAN4 #99e meeting.

In this paper we present our views on the frequency range definition on NR 52.6 – 71 GHz.

# Discussion

3GPP has spent multiple meetings (two plenary meetings and two RAN4 meetings) to discuss this issue without conclusion. Based on what we understand, currently the following options are largely being discussed:

- Option 1: Introduce a new frequency range, i.e., FR3

- Option 2: Extend FR2 to cover the new frequency range

The decision on the new FR notation mostly depends on the convenience of the specification introduction and it is recommended to strive to minimize the RAN4 specification impacts. In RAN #98-bis-e it was agreed not to introduce a new spec for 60 GHz (i.e., include 60 GHz in the existing UE and BS specs).

Observation #1: RAN4 agreed not to introduce a new spec for 60 GHz (i.e., include 60 GHz in the existing UE and BS specs).

Even in case the new specs are not introduced RAN4 still needs to decide on how to capture the new FR in the existing specifications. While we think the Option 1 is straightforward, it may create much work burden to RAN4. Option 2 seems to be a simpler way to resolve the issue. However, care should be taken as there are new system parameters which were not introduced in FR2 discussion and direct reuse of the existing FR2 requirements may not be always an option. Therefore, we think that RAN4 should come up with a flexible solution which would take the best from Option 1 and 2 and allow flexible specification implementation. Below, we provide a preliminary assessment of new FR introduction on RAN4 specs.

## UE RF spec (TS 38.101-2/3)

The FR2 UE RF requirements are defined in TS 38.101-2 and TS 38.101-3 (interworking requirements). In general, RAN4 needs to go through the full set of requirements and make a decision whether the specific requirements can be reused or need to be adjusted for the new frequency range. Although, it is expected that several FR2 requirements can be reused (e.g. max TRP/EIRP), at least the following RF requirements may need to be revisited/reevaluated for the new frequency range:

- Operating bands

- System parameters (Channel bandwidth, Spectrum utilization, Channel/Sync raster)

- CA configurations

- Tx power (min peak EIRP, spherical coverage, multi-band relaxation, MPR, A-MPR)

- Beam correspondence

- Output power dynamics (min output power, transmit off power, transmit ON/OFF masks, power control)

- ACLR

- Spurious emission band UE co-existence

- Signal quality (carrier leakage, IBE)

- OOBE

- Reference sensitivity (REFSENS, EIS spherical coverage)

- ACS

- In-band blocking

It can be expected that some of the requirements above will be introduced in a band-specific and/or power class specific manner (i.e. do not require definition of a new FR). However, some requirements are defined in a generic manner and cannot be introduced in band/PC specific manner. Therefore, a new notation to differentiate the new FR is required. Meantime, we think the existing FR2 RF specification structure can be extended to support the new FR and no new UE RF specification would be required for NR 52.6-71 GHz frequency range.

Observation #2: Multiple UE RF requirements may need to be defined for the new FR:

- Some FR2 UE RF requirements can be reused for the new FR

- Some UE RF requirements for the new FR can be introduced in a band-specific or in a power class specific manner and do not require a new notation for the FR

- Some UE RF requirements for the new FR cannot be introduced in a band-specific or in a UE power class specific manner and a new notation to differentiate the new FR is required for UE RF specification.

## RRM spec (TS 38.133)

Many of the RRM requirements defined in TS 38.133 are differentiated between FR1 and FR2. In addition, a number of FR2 requirements are defined in a UE power class specific. At least the following requirements are currently defined in FR-specific manner:

- Measurement delay requirements (RX beam sweeping related)

- MTRD/MTTD requirements

- Timing (NTAoffset, MTTD/MRTD)

- RLM/BFD/CBD

- Scheduling availability/restrictions

- MG patterns applicability

- Interruption requirements

- Measurement accuracy requirements

- Others

Currently the RAN4 work on the definition of RRM requirements has not started yet. We expect that it may not be possible to reuse all the existing FR2 requirements and new requirements need to be introduced. Therefore, a new notation to differentiate the frequency range can be helpful.

Observation #3: Multiple RRM requirements are defined in a FR-specific manner. Some of the existing FR2 requirements cannot be reused for the new FR and a new notation to differentiate the frequency range is required for the RRM specification.

## UE Demodulation spec (TS 38.101-4)

The UE demodulation requirements in TS 38.101-4 for FR2 are defined in separate clauses (Clause 7 and Clause 8) related to the radiated performance requirements. Although it is expected that radiated testing shall be applied for new FR testing, the existing requirements are applicable for FR2 frequency bands and their applicability for the higher frequencies needs further assessment. In addition, RAN1 is discussing potential L1 enhancements to support the new FR definition (e.g. multi-slot transmission, PTRS) and it is not clear whether the existing L1 design will be directly applicable to the new range. Therefore, it is also helpful to have sufficient flexibility to introduce a specific set of requirements for NR > 52GHz and a new notation can be helpful.

Observation #4: It is FFS whether existing UE demodulation test cases can be reused for the new FR. Introduction of the new notation for the new FR can be beneficial to differentiate the test cases if it is needed.

## BS RF/Demod specs (TS 38.104)

Similar to the UE RF and Demod specs selected new requirements may need to be defined for the BS RF/Demod. In addition, further discussion is needed whether a new BS type will be introduced for the new FR. In case a new BS type is defined then the specification can be easily extended to cover new FR specific requirements. Otherwise, a dedicated notation for the new FR can be helpful to differentiate the requirements.

Observation #5: Selected new BS RF/Demod requirements may need to be defined for the new FR and a new notation for the FR or new BS types may be required to simplify specification implementation.

## Conclusion

The general set of RAN4 RF/RRM/Demod requirements can be categorized as

1) FR agnostic

2) FR specific

3) Band specific

4) UE Power Class specific

Band specific and UE PC specific requirements can be modified without introduction of the new FR and can be handled simply via introduction of the new bands and UE power classes. Meantime, in case certain changes need to be defined for the FR-specific requirements, then a new notation to differentiate the new FR shall be considered. Introduction of the new FR3 notation may have a large impact on the specification and will not allow simple reuse of the existing requirements. Therefore, we propose to introduce a new notation to denote the new FR as a subset of the existing FR2.

During RAN4 #98-bis-e it was also noted that RAN5 is already using designations FR2a, FR2b, FR2c to subdivide current FR2 ranges: a) 23.45-32.125GHz, b) 32.125-40.8GHz, c) 40.8GHz-44.3GHz. Therefore, to it may be undesirable to have overlap with the RAN5 notations.

Considering the RAN5 notations, the following approaches can be envisioned:

- Option 1: Use FR2 notation to designate the full 24.25 – 71 GHz range

  - Use a new FR2-1 notation for 24.25 – 52.6 GHz range

  - Use a new FR2-2 notation for 52.6 – 71 GHz range

- Option 2: Use FR2 notation for 24.25 – 52.6 GHz range and a new FR2-2 notation for 52.6 – 71 GHz range

## Draft LS to RAN

Given that RAN plenary tasked the RAN1, RAN2 and RAN4 to provide analysis or recommendation to RAN#92E (June) on how to introduce the 52.6-71GHz frequency range, we propose to send the LS to RAN plenary summarizing the feedback. The draft LS is shown below.

Proposal #2: Send LS to RANP to inform on the frequency range definition impact on the RAN4 specification structure and requirements and provide recommendations on the FR notation definition.

Title: LS on frequency range definition for NR operation in 52.6 to 71GHz Response to: Release: Rel-17 Work Item: NR_ext_to_71GHz Source: RAN WG4 To: RAN Cc: RAN1, RAN2 Contact Person: #### Name: Jiwoo Kim E-mail Address: jiwoo.kim@intel.com 1. Overall Description: RAN4 discussed the frequency range definition for NR operation in 52.6 to 71GHz. The following observations were made with respect to the frequency range definition impact on RAN4 specifications and requirements definition: - Introduction of new FR3 will create much work burden to RAN4 in terms of specification update - RAN4 will define BS and UE RF, RRM and demodulation requirements for the new frequency range. The detailed requirements are under discussion, while it is RAN4 understanding that frequency range definition may have impact on the requirements definition and specification structure. RAN4 has identified the following: - Some RAN4 FR2 requirements can be reused for the new frequency range - Some RAN4 requirements can be introduced in a band-specific or in a power class specific manner and do not require a new notation for the FR - Some RAN4 requirements for the new frequency range may not be introduced in a band-specific or in a UE power class specific manner and a new notation to differentiate the new range can be helpful. The following agreements were made on the RAN4 specification structure: - No new specification will be defined for NR operation in 52.6 to 71GHz (i.e. include 52.6 to 71GHz operation in the existing RAN4 specifications) RAN4 recommends RAN to introduce a new notation for the 52.6 – 71 GHz frequency range as follows: - Option 1: Use FR2 notation to designate the full 24.25 – 71 GHz range - Use a new FR2-1 notation for 24.25 – 52.6 GHz range - Use a new FR2-2 notation for 52.6 – 71 GHz range - Option 2: Use FR2 notation for 24.25 – 52.6 GHz range and a new FR2-2 notation for 52.6 – 71 GHz range 2. Actions: To RAN: RAN4 recommends RAN to consider the information above for the final decision on the frequency range definition for NR operation in 52.6 to 71GHz. 3. Date of Next RAN4 Meetings: TSG-RAN WG4 Meeting #100-e 23 - 27 Aug 2021 Online

# Summary

In this paper we present our views on the frequency range definition on NR 52.6 – 71 GHz. In summary, we make the following proposals.

Proposal #1: RAN4 to recommend RANP to introduce a new notation for the 52.6 – 71 GHz frequency range

- Option 1: Use FR2 notation to designate the full 24.25 – 71 GHz range

  - Use a new FR2-1 notation for 24.25 – 52.6 GHz range

  - Use a new FR2-2 notation for 52.6 – 71 GHz range

- Option 2: Use FR2 notation for 24.25 – 52.6 GHz range and a new FR2-2 notation for 52.6 – 71 GHz range

Proposal #2: Send LS to RANP to inform on the frequency range definition impact on the RAN4 specification structure and requirements and provide recommendations on the FR notation definition.

# Reference

[1]	RP-202925, Revised WID: Extending current NR operation to 71 GHz, RAN#90-e, Qualcomm, Intel