3GPP TSG-RAN WG4 Meeting #99-e                                   R4-2110203

Electronic Meeting, May. 19-27, 2021

Agenda Item:	6.2.4.2.2

Source:	MediaTek Inc.

Title:	Simulation results for NR V2X PSCCH test case

Document for:		Discussion

- Introduction

In last RAN4 e-meeting, the simulation assumptions of PSCCH were agreed [1]. In the contribution, we will provide initial simulation results of PSCCH based on simulation assumption.

- Discussion

Simulation assumptions

The simulation assumptions are showed as following:

Table 1 Simulation assumption for PSCCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | CCH_Test1 |
| Synchronization source |  | GNSS |
| Propagation condition |  | TDLA30-1400 |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | 10 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| The number of PSCCH symbols |  | 3 |
| Modulation order |  | QPSK |
| Payload size (without CRC) | Bits | 26 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits | Bits | 540 |
| Antenna configuration |  | 1x2 Low |
| Performance metric (Note 1) |  | SNR@1% BLER of PSCCH |
| Note 1: OCC index i (in TS 38.211) for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission. Note 2: The first OFDM symbol of a PSSCH and its associated PSCCH is duplicated as described in clauses 8.3.1.5 and 8.3.2.3 of TS 38.211 and not used for demodulation. |  |  |

Simulation results

Based on the simulation assumptions, we provide simulation results in Figure 1.

Figure 1 PSCCH simulation result

According to the simulation results, we can see that the corresponding SNR targeted 1% BLER is about 3.8dB.

Proposal 1: The required SNR targeted 1% BLER for PSCCH test case is about 3.8dB.

- Summary

In this contribution, we provide the PSCCH simulation result and make the following proposal:

Proposal 1: The required SNR targeted 1% BLER for PSCCH test case is about 3.8dB.

- Reference

- R4-2103991, “Updated simulation assumptions for V2X single link test cases”,  Huawei, HiSilicon