3GPP TSG-RAN WG4 Meeting # 99-e 		R4-2110273

# Electronic Meeting, May 19-27, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Link simulation results for gNB Rx-Tx time difference accuracy

Agenda item:	6.5.2.3.3

Document for:	Discussion

Introduction

At RAN4 #98-bis-e, further discussion was carried out on how to specify gNB accuracy requirements for NR positioning in TS 38.133 and agreements were captured in the WF [1].

Following further agreements on gNB Rx-Tx time difference were achieved in view of simulating link performance:

- Simulations are to be aligned to updated link simulation assumptions [2], adding SCS=60 kHz and two SRS symbol and comb size combinations.

- gNB Rx-Tx time difference accuracy is dependent on SRS SCS.

- Accuracy requirements for gNB measurements will be specified for AWGN propagation channel.

- Accuracy requirements will be specified for single sample/shot. For the lowest SRS BW group per SCS, performance for multiple shots can be investigated, if single shot performance is not satisfactory.

- Preliminary SRS BW grouping for gNB Rx-Tx time difference were agreed and shall follow Table 1.

Table 1: SRS BW grouping in FR1 and FR2

| SRS bandwidth in RB | SRS SCS [kHz] | gNB Rx-Tx time difference accuracy [Tc] |  |
| --- | --- | --- | --- |
|  |  | Ês/Iot ≥ -13 dB | Ês/Iot ≥ +3 dB |
| 24 ≤ BW ≤ 40 | 15 | TBD | TBD |
| 44 ≤ BW ≤ 84 |  | TBD | TBD |
| 88 ≤ BW ≤ 168 |  | TBD | TBD |
| 176 ≤ BW ≤ 264 |  | TBD | TBD |
| 48 ≤ BW ≤ 84 | 30 | TBD | TBD |
| 88 ≤ BW ≤ 168 |  | TBD | TBD |
| 176 ≤ BW ≤ 272 |  | TBD | TBD |
| 32 ≤ BW ≤ 40 | 120 | TBD | TBD |
| 44 ≤ BW ≤ 84 |  | TBD | TBD |
| 88 ≤ BW |  | TBD | TBD |

Note 1: 60kHz SCS can be added based on the simulation results

Note 2: Lower bound of BW ranges can be updated based on simulation results

- SRS BW grouping for gNB Rx-Tx time difference measurement accuracy will be decided based on link simulation results (from RAN4 #98-e).

- Baseline gNB Rx-Tx time difference measurement accuracy without margin is based on link simulation results (from RAN4 #98-e).

This contribution lists our baseline simulation results for gNB Rx-Tx time difference measurement accuracy for agreed SRS configurations in [2].

Simulation assumptions

The SRS configurations based on SRS BW grouping in Table 1 and including the settings for SRS comb size and # SRS symbols for FR1 and FR 2 in [2] were simulated.

In addition to Table 1, following SRS configurations for SCS=60 kHz were simulated:

- FR1: SRS BW of 48 RBs (40 MHz), 132 RBs (100 MHz),

- FR2: SRS BW of 132 RBs (100 MHz) and 264 RBs (200 MHz).

The simulations were carried out for the lower bound of each SRS BW group. No RF and implementation impairments are included.

Simulation results

Simulation results for gNB Rx-Tx time difference accuracy for the SRS configurations agreed in [2] are depicted below for FR1 and FR2. The results are reported for 90 percentile CDF of the timing error, i.e. 90% of measurements for given Ês/Iot ratios (+3 dB, -13 dB) experience a timing error less than the given figure. Results are derived for the lower bound of each SRS BW group in Table 1.

FR1

SCS = 15 kHz

The gNB Rx-Tx time difference accuracy for AWGN channel, derived from the 90-percentile of the CDF for the measured timing error according to [2], is listed in Table 2.

Table 2: gNB Rx-Tx time difference accuracy in Tc units (AWGN, SCS= 15 kHz).

| SRS BW in RB | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 15 kHz) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -13 dB |
| 24 | 512 | 2 / 1 | 128 | 128 |
|  |  | 2 / 2 | 128 | 128 |
|  |  | 4 / 4 | 128 | 128 |
|  |  | 4 / 12 | 128 | 128 |
|  |  | 8 / 8 | 128 | 128 |
|  |  | 8 / 12 | 128 | 128 |
| 44 | 1024 | 2 / 1 | 64 | 64 |
|  |  | 2 / 2 | 64 | 64 |
|  |  | 4 / 4 | 64 | 64 |
|  |  | 4 / 12 | 64 | 64 |
|  |  | 8 / 8 | 64 | 64 |
|  |  | 8 / 12 | 64 | 64 |
| 88 | 2048 | 2 / 1 | 32 | 32 |
|  |  | 2 / 2 | 32 | 32 |
|  |  | 4 / 4 | 32 | 32 |
|  |  | 4 / 12 | 32 | 32 |
|  |  | 8 / 8 | 32 | 32 |
|  |  | 8 / 12 | 32 | 32 |
| 176 | 4096 | 2 / 1 | 16 | 16 |
|  |  | 2 / 2 | 16 | 16 |
|  |  | 4 / 4 | 16 | 16 |
|  |  | 4 / 12 | 16 | 16 |
|  |  | 8 / 8 | 16 | 16 |
|  |  | 8 / 12 | 16 | 16 |

SCS = 30 kHz

The gNB Rx-Tx time difference accuracy for AWGN channel, derived from the 90-percentile of the CDF for the measured timing error according to [2], is listed in Table 3.

Table 3: gNB Rx-Tx time difference accuracy in Tc units (AWGN, SCS= 30 kHz).

| SRS BW in RB | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 30 kHz) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -13 dB |
| 48 | 1024 | 2 / 1 | 64 | 64 |
|  |  | 2 / 2 | 64 | 64 |
|  |  | 4 / 4 | 64 | 64 |
|  |  | 4 / 12 | 64 | 64 |
|  |  | 8 / 8 | 64 | 64 |
|  |  | 8 / 12 | 64 | 64 |
| 88 | 2048 | 2 / 1 | 16 | 16 |
|  |  | 2 / 2 | 16 | 16 |
|  |  | 4 / 4 | 16 | 16 |
|  |  | 4 / 12 | 16 | 16 |
|  |  | 8 / 8 | 16 | 16 |
|  |  | 8 / 12 | 16 | 16 |
| 176 | 4096 | 2 / 1 | 8 | 8 |
|  |  | 2 / 2 | 8 | 8 |
|  |  | 4 / 4 | 8 | 8 |
|  |  | 4 / 12 | 8 | 8 |
|  |  | 8 / 8 | 8 | 8 |
|  |  | 8 / 12 | 8 | 8 |

SCS = 60 kHz

The gNB Rx-Tx time difference accuracy for AWGN channel, derived from the symmetric 90-percentile of the CDF for the measured timing error according to [2], is listed in Table 4.

Table 4: gNB Rx-Tx time difference accuracy in Tc units (AWGN, SCS= 60 kHz).

| SRS BW in RB | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 60 kHz) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -13 dB |
| 48 | 1024 | 2 / 1 | 16 | 16 |
|  |  | 2 / 2 | 16 | 16 |
|  |  | 4 / 4 | 16 | 16 |
|  |  | 4 / 12 | 16 | 16 |
|  |  | 8 / 8 | 16 | 16 |
|  |  | 8 / 12 | 16 | 16 |
| 132 | 2048 | 2 / 1 | 8 | 8 |
|  |  | 2 / 2 | 8 | 8 |
|  |  | 4 / 4 | 8 | 8 |
|  |  | 4 / 12 | 8 | 8 |
|  |  | 8 / 8 | 8 | 8 |
|  |  | 8 / 12 | 8 | 8 |

FR2

SCS = 120 kHz

The gNB Rx-Tx time difference accuracy for AWGN channel, derived from the 90-percentile of the CDF for the measured timing error according to [2], is listed in Table 5.

Table 5: gNB Rx-Tx time difference accuracy in Tc units (AWGN, SCS= 120 kHz).

| SRS BW in RB | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 120 kHz) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -13 dB |
| 32 | 512 | 2 / 1 | 16 | 16 |
|  |  | 2 / 2 | 16 | 16 |
|  |  | 4 / 4 | 16 | 16 |
|  |  | 4 / 12 | 16 | 16 |
|  |  | 8 / 8 | 16 | 16 |
|  |  | 8 / 12 | 16 | 16 |
| 44 | 1024 | 2 / 1 | 8 | 8 |
|  |  | 2 / 2 | 8 | 8 |
|  |  | 4 / 4 | 8 | 8 |
|  |  | 4 / 12 | 8 | 8 |
|  |  | 8 / 8 | 8 | 8 |
|  |  | 8 / 12 | 8 | 8 |
| 88 | 2048 | 2 / 1 | 8 | 8 |
|  |  | 2 / 2 | 8 | 8 |
|  |  | 4 / 4 | 8 | 8 |
|  |  | 4 / 12 | 8 | 8 |
|  |  | 8 / 8 | 8 | 8 |
|  |  | 8 / 12 | 8 | 8 |

SCS = 60 kHz

The gNB Rx-Tx time difference accuracy for AWGN channel, derived from the symmetric 90-percentile of the CDF for the measured timing error according to [2], is listed in Table 6.

Table 6: gNB Rx-Tx time difference accuracy in Tc units (AWGN, SCS= 60 kHz).

| SRS BW in RB | FFT size | SRS comb size / # SRS symbols | AWGN (SCS = 60 kHz) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Ês/Iot = +3 dB | Ês/Iot = -13 dB |
| 132 | 2048 | 2 / 1 | 8 | 8 |
|  |  | 2 / 2 | 8 | 8 |
|  |  | 4 / 4 | 8 | 8 |
|  |  | 4 / 12 | 8 | 8 |
|  |  | 8 / 8 | 8 | 8 |
|  |  | 8 / 12 | 8 | 8 |
| 264 | 4096 | 2 / 1 | 8 | 8 |
|  |  | 2 / 2 | 8 | 8 |
|  |  | 4 / 4 | 8 | 8 |
|  |  | 4 / 12 | 8 | 8 |
|  |  | 8 / 8 | 8 | 8 |
|  |  | 8 / 12 | 8 | 8 |

Discussion

From the simulation results reported above, we observe:

- a clear dependency of the gNB Rx-Tx time difference accuracy on the SRS bandwidth, i.e. on number of corresponding RB’s and on the SCS, due to larger FFT size;

- no dependency of the gNB Rx-Tx time difference accuracy on the defined Ês/Iot ratios;

- no dependency of the gNB Rx-Tx time difference accuracy on the SRS symbol and comb size combination for high and low Ês/Iot ratios.

Thus, following observations are made:

There is a clear dependency of the gNB Rx-Tx time difference accuracy on the SRS bandwidth.

There is no dependency of the gNB Rx-Tx time difference accuracy on the defined Ês/Iot ratios.

There is no dependency of the gNB Rx-Tx time difference accuracy on the SRS symbol and comb size combination for high and low Ês/Iot ratios.

Following proposal is made:

The provided gNB Rx-Tx time difference accuracy results are taken into account in the discussion on SRS BW grouping and other SRS configuration parameter grouping.

Conclusion

This contribution lists our baseline simulation results for gNB Rx-Tx time difference accuracy for agreed SRS configurations in [2] as evaluated in section 3.

Based on above evaluations, following observations are made:

- There is a clear dependency of the gNB Rx-Tx time difference accuracy on the SRS bandwidth.

There is no dependency of the gNB Rx-Tx time difference accuracy on the defined Ês/Iot ratios.

There is no dependency of the gNB Rx-Tx time difference accuracy on the SRS symbol and comb size combination for high and low Ês/Iot ratios.

Following proposal for agreement is made:

- The provided gNB Rx-Tx time difference accuracy results are taken into account in the discussion on SRS BW grouping and other SRS configuration parameter grouping.

References

- R4-2105755, "WF on gNB positioning measurement requirements", source: Ericsson

- R4-2105757, "Updated link simulation assumptions for gNB positioning measurement", source: Huawei, HiSilicon