3GPP TSG-RAN WG4 Meeting # 99-e                               R4-2110399

Electronic Meeting, 19– 27 May, 2021

Source: 	Huawei, HiSilicon

Title: 	Discussion on n47 PC2 AMPR simulation results

Agenda Item:	9.14.6.3

Document for:	Approval

Introduction

In RAN4#98e meeting, WF [1] on MPR/AMPR simulation assumptions for PC2 NR V2X has been approved. In this paper, we’d like to bring our n47 PC2 AMPR simulation results and corresponding analysis.

AMPR for NS_33

For NS_33, the additional requirements are specified in clause 6.5E.3.4.2 and 6.5E.2.3.1 in TS 38.101-1, which are copied as below.

Table 6.5.3.4.2-1: Additional requirements for "NS_33"

| Protected band |  | Frequency range (MHz) |  |  | Maximum Level (EIRP2) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 5925 |  | - | 5950 | -30 | 1 | 1 |
| Frequency range | 5815 |  | - | 5855 | -30 | 1 | 3 |
| NOTE 1: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 2: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. NOTE 3: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set larger than (symbol length)*(number of points in sweep) to improve the measurement accuracy. |  |  |  |  |  |  |  |

Table 6.5E.2.2.1-1: Additional spectrum mask requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

In order to meet these additional requirements, AMPR for NS_33 should be specified for PC2 NR V2X UE. The following PC3 requirements can be reused for PC2 AMPR requirements.

When NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the additional maximum output power reduction specified as

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = A-MPRBase + Gpost connector* A-MPRStep

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

A-MPRBase  and A-MPRStep  are specified in Tables 6.2E.3-1, 6.2E.3-2 is allowed when network signalling value is provided. A-MPRBase is the default A-MPR value when no Gpost connector is declared. The supported post antenna connector gain Gpost connector is declared by the UE following the principle described in annex I in [11]. The A-MPRstep is the increase in A-MPR allowance to allow UE to meet tighter conducted A-SE and A-SEM requirements with higher value of declared Gpost connector.

Proposal 1: current the framework of NS_33 AMPR requirements for power class 3 can be reused for power class 2 NR V2X UE.

PC2 AMPR for NS_33 PSCCH/PSSCH

Based on the AMPR simulation assumptions approved in last meeting, an overview for PC2 NS_33 AMPR simulation results can be found as below. The detailed results are attached in the appendix.

Table 1 Overview for PC2 MPR simulation results

| BW | Fc | Modulate | Inner | Outer |
| --- | --- | --- | --- | --- |
| 10MHz | 5860 | QPSK | 10.9608 | 15.6444 |
| 10MHz | 5870 | QPSK | 1.2020 | 3.1518 |
| 10MHz | 5880 | QPSK | 1.5149 | 2.9870 |
| 10MHz | 5890 | QPSK | 1.3558 | 3.1635 |
| 10MHz | 5900 | QPSK | 1.2506 | 3.3304 |
| 10MHz | 5910 | QPSK | 1.4423 | 3.1551 |
| 10MHz | 5920 | QPSK | 1.49 | 3.1383 |
| 10MHz | 5860 | 16QAM | 11.5181 | 15.7444 |
| 10MHz | 5870 | 16QAM | 1.4567 | 3.3051 |
| 10MHz | 5880 | 16QAM | 1.6001 | 3.1606 |
| 10MHz | 5890 | 16QAM | 1.4280 | 3.1395 |
| 10MHz | 5900 | 16QAM | 1.3297 | 3.1566 |
| 10MHz | 5910 | 16QAM | 1.3103 | 3.3219 |
| 10MHz | 5920 | 16QAM | 1.3392 | 3.2175 |

It’s observed that current NR V2X UE NS_33 AMPR requirements for PC3 Fc > 5865 can be met based on the PC2 AMPR simulation results.

Table 6.2E.3.2-2: A-MPR for PSSCH/PSCCH by NS_33 (at other carrier frequency)

| Carrier frequency [MHz] | RB allocations | A-MPRBase (dB) |  |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | QPSK | 16QAM | 64QAM | 256QAM |  |
| 5870, 5880, 5890, 5900, 5910, 5920 | Inner | ≤ 3.0 |  | ≤ 5.0 | ≤ 6.0 | 0.5 |
|  | Outer | ≤ 4.5 |  |  |  |  |
| NOTE 1: Inner and Outer RB allocations are defined in clause 6.2E.2.1 NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |  |

Proposal 2: When Fc > 5865, current NR V2X UE NS_33 AMPR requirements for PC3 can be reused for power class 2 NR V2X UE.

For Fc = 5860, the PC2 AMPR requirements for NS_33 can be specified as table 1 according to the simulation results.

Table 1: PC2 A-MPR for PSSCH/PSCCH by NS_33 (at Fc =5860MHz)

| Carrier frequency [MHz] | Resources Blocks (LCRB) | Start Resource Block | A-MPRBase (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | ≥ 10 and ≤ 15 | 0 | ≤ 25 |  |  |
|  |  | ≥ 1 and ≤ 3 | ≤ 20 |  |  |
|  | ≥ 10 and ≤ 15 | ≥ 18 and ≤ 26 | ≤6.5 |  |  |
|  | ≥ 10 and ≤ 15 | ≥ 26 | ≤11 |  |  |
|  | ≥ 10 and ≤ 15 | ≥ 12 and ≤ 14 | ≤11 |  |  |
|  |  | ≥ 15 and ≤ 17 | ≤9.5 |  |  |
|  | ≥ 10 and ≤ 30 | ≥ 4 and ≤7 | ≤ 16.5 |  |  |
|  |  | ≥ 8 and ≤ 11 | ≤ 13.5 |  |  |
|  | ≥ 20 and ≤ 30 | ≥ 0 and ≤ 3 | ≤ 22 |  |  |
|  | ≥ 20 and ≤ 30 | ≥ 16 | ≤ 11 |  |  |
|  | ≥ 20 and ≤ 40 | ≥ 12 and ≤ 15 | ≤ 13.5 |  |  |
|  | 40 and 45 | 0 and 1 | ≤ 20 |  |  |
|  |  | ≥ 2 and ≤6 | ≤ 17 |  |  |
|  |  | ≥ 7 and ≤ 11 | ≤ 14 |  |  |
|  | ≥ 50 | ≥ 0 | ≤ 17 |  |  |
| NOTE 1: A-MPRstep =1.2 dB is applied for RBstart 0 and 1 and A-MPRstep =0.7 dB is applied for all other RBstart NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |

Proposal 3: When Fc = 5865, the PC2 AMPR requirements for NS_33 can be specified as table 1.

PC2 AMPR for NS_33 S-SSB

For Fc > 5865 and Fc = 5860, the PC2 S-SSB AMPR simulation results are shown as below.

Figure 1 PC2 S-SSB AMPR simulation results for Fc > 5865

Figure 2 PC2 S-SSB AMPR simulation results for Fc = 5860

According to the simulation results, the NS_33 A-MPR requirements for PC2 S-SSB can be specified as below.

Table 2 the proposed NS_33 AMPR requirements for PC2 S-SSB

| Carrier Frequency (MHz) | RBStart * 12*SCS [MHz] | A-MPRBase (dB) | AMPRStep (dB) |
| --- | --- | --- | --- |
| 5860 | ≤1.0 | ≤ 25 | 0.6 |
|  | >1.0 and ≤2.0 | ≤ 19 |  |
|  | >2.0 and ≤3.24 | ≤ 12 |  |
|  | >3.24 and ≤3.6 | ≤ 10 |  |
|  | >3.6 | ≤ 14 |  |
| 5870, 5880, 5890, 5900, 5910, 5920 | ≤1.0 | ≤ 7.0 | 0.85 |
|  | >1.0 and ≤1.6 | ≤ 6.5 |  |
|  | >1.6 and ≤2.6 | ≤ 5.8 |  |
|  | >2.6 and ≤3.24 | ≤ 4.5 |  |
|  | >3.24 and ≤4.32 | ≤ 5.5 |  |
|  | >4.32 | ≤ 6.5 |  |

Proposal 4: For S-SSB, the PC2 AMPR requirements for NS_33 can be specified as table 2.

PC2 AMPR for NS_33 PSFCH

For Fc > 5865 and Fc = 5860, the PC2 PSFCH AMPR simulation results are shown as below.

Figure 3 The AMPR region for NS_34 Fc = 5860

Figure 4 The AMPR region for NS_34 Fc > 5865

For the simultaneous PSFCH transmission when NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements

Table 3: A-MPR for simultaneous PSFCH by NS_33

| Channel Bandwidth [MHz] | Center Frequency [MHz] | RB allocation | A-MPRBase (dB) |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 ≤ NGap / NRB < 0.2 | 0.2≤ NGap / NRB < 0.4 | 0.4≤ NGap / NRB ≤ 1 |  |
| 10 | 5860 | NRB =1 | 25 |  |  | 1.0 |
|  |  | NRB > 1 | 22.0 |  |  |  |
|  | 5870, 5880, 5890, 5900, 5910, 5920 | NRB =1 | 5.5 |  |  | 0.8 |
|  |  | NRB > 1 | 16.5 | 12 | 20 |  |
| Note 1: NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart) |  |  |  |  |  |  |

Proposal 5: For PSFCH, the PC2 AMPR requirements for NS_33 can be specified as table 3.

Summary

Based on the discussion, all the observations and proposals are listed below:

Proposal 1: current the framework of NS_33 AMPR requirements for power class 3 can be reused for power class 2 NR V2X UE.

Proposal 2: When Fc > 5865, current NR V2X UE NS_33 AMPR requirements for PC3 can be reused for power class 2 NR V2X UE as below.

| Carrier frequency [MHz] | RB allocations | A-MPRBase (dB) |  |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | QPSK | 16QAM | 64QAM | 256QAM |  |
| 5870, 5880, 5890, 5900, 5910, 5920 | Inner | ≤ 3.0 |  | ≤ 5.0 | ≤ 6.0 | 0.5 |
|  | Outer | ≤ 4.5 |  |  |  |  |
| NOTE 1: Inner and Outer RB allocations are defined in clause 6.2E.2.1 NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |  |

Proposal 3: When Fc = 5865, the PC2 AMPR requirements for NS_33 can be specified as table 1 as below.

| Carrier frequency [MHz] | Resources Blocks (LCRB) | Start Resource Block | A-MPRBase (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | ≥ 10 and ≤ 15 | 0 | ≤ 25 |  |  |
|  |  | ≥ 1 and ≤ 3 | ≤ 20 |  |  |
|  | ≥ 10 and ≤ 15 | ≥ 18 and ≤ 26 | ≤6.5 |  |  |
|  | ≥ 10 and ≤ 15 | ≥ 26 | ≤11 |  |  |
|  | ≥ 10 and ≤ 15 | ≥ 12 and ≤ 14 | ≤11 |  |  |
|  |  | ≥ 15 and ≤ 17 | ≤9.5 |  |  |
|  | ≥ 10 and ≤ 30 | ≥ 4 and ≤7 | ≤ 16.5 |  |  |
|  |  | ≥ 8 and ≤ 11 | ≤ 13.5 |  |  |
|  | ≥ 20 and ≤ 30 | ≥ 0 and ≤ 3 | ≤ 22 |  |  |
|  | ≥ 20 and ≤ 30 | ≥ 16 | ≤ 11 |  |  |
|  | ≥ 20 and ≤ 40 | ≥ 12 and ≤ 15 | ≤ 13.5 |  |  |
|  | 40 and 45 | 0 and 1 | ≤ 20 |  |  |
|  |  | ≥ 2 and ≤6 | ≤ 17 |  |  |
|  |  | ≥ 7 and ≤ 11 | ≤ 14 |  |  |
|  | ≥ 50 | ≥ 0 | ≤ 17 |  |  |
| NOTE 1: A-MPRstep =1.2 dB is applied for RBstart 0 and 1 and A-MPRstep =0.7 dB is applied for all other RBstart NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |

Proposal 4: For S-SSB, the PC2 AMPR requirements for NS_33 can be specified as table 2 as below.

| Carrier Frequency (MHz) | RBStart * 12*SCS [MHz] | A-MPRBase (dB) | AMPRStep (dB) |
| --- | --- | --- | --- |
| 5860 | ≤1.0 | ≤ 25 | 0.6 |
|  | >1.0 and ≤2.0 | ≤ 19 |  |
|  | >2.0 and ≤3.24 | ≤ 12 |  |
|  | >3.24 and ≤3.6 | ≤ 10 |  |
|  | >3.6 | ≤ 14 |  |
| 5870, 5880, 5890, 5900, 5910, 5920 | ≤1.0 | ≤ 7.0 | 0.85 |
|  | >1.0 and ≤1.6 | ≤ 6.5 |  |
|  | >1.6 and ≤2.6 | ≤ 5.8 |  |
|  | >2.6 and ≤3.24 | ≤ 4.5 |  |
|  | >3.24 and ≤4.32 | ≤ 5.5 |  |
|  | >4.32 | ≤ 6.5 |  |

Proposal 5: For PSFCH, the PC2 AMPR requirements for NS_33 can be specified as table 3 as below.

| Channel Bandwidth [MHz] | Center Frequency [MHz] | RB allocation | A-MPRBase (dB) |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 ≤ NGap / NRB < 0.2 | 0.2≤ NGap / NRB < 0.4 | 0.4≤ NGap / NRB ≤ 1 |  |
| 10 | 5860 | NRB =1 | 25 |  |  | 1.0 |
|  |  | NRB > 1 | 22.0 |  |  |  |
|  | 5870, 5880, 5890, 5900, 5910, 5920 | NRB =1 | 5 |  |  | 0.8 |
|  |  | NRB > 1 | 16.5 | 12 | 20 |  |
| Note 1: NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart) |  |  |  |  |  |  |

- References

- R4-2103250, Way forward on simulation assumptions for PC2 NR V2X, Huawei, HiSilicon

- Appendix

- PC2 NS_33 PSCCH/PSSCH

Fc = 5860

Fc = 5870