3GPP TSG-RAN WG4 Meeting # 99-e 	R4-2110522

Electronic Meeting, May. 19-27, 2021

Title: 	Discussions and simulation results on NR V2X soft buffer test

Source: 	Huawei, HiSilicon

Agenda item:	6.2.3.3.2

Document for:	Discussion

Background

At last meeting, MCS for V2X soft buffer test was changed from 27 to 28. In this paper, we provide our update simulation results,

Simulation results

Simulation results are shown in Figure 2-1 and Table 2-1 and corresponding simulation assumptions are shown in appendix.

Figure 2-1: Simulation results for soft buffer test with n64

Table 2-1: Summary of simulation results for soft buffer test with n64

| Test metric | Ideal requirement | Impairment requirement |
| --- | --- | --- |
| SNR(dB)@5%BLER | 7.93 | 8.43 |

Conclusion

In this contribution, we provide our simulation results for V2X soft buffer test.

Reference

[1]   R4-2106035 Updated simulation assumptions for NR V2X multiple link test cases. Huawei, HiSilicon

Appendix

- According to the agreement of RAN4#98-e meeting, RAN4 will only define one set of performance requirements based on the results of harq-RxProcessSidelink = n64. Interested companies are encouraged to provide simulation results in next meeting based on the simulation assumptions given in Table 2.2-1 and Table 2.2-2.

Table 2.2-1： Simulation assumptions for soft buffer test

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Active cell(s) |  |  | None |
| Bandwidth |  | MHz | 20 |
| SCS |  | kHz | 30 |
| Subchannel size |  | RBs | 10 |
| Number of symbols for PSCCH |  | Symbol | 2 |
| Number of RBs for PSCCH |  | RBs | 10 |
| Active Sidelink UE(s) |  |  | 64 |
| Sidelink UE i, 0 ≤ i ≤ 63 | Sidelink Transmissions |  | First 31 UEs transmit PSCCH + PSSCH one by one circularly for every slot and in the first subchannel, the next 31 UEs transmit PSCCH + PSSCH one by one circularly for every slot and in the second subchannel, the last 2 UEs transmit PSCCH + PSSCH in the same slot as the first two UEs but in the third subchannel. |
|  | Time gap between initial transmission and retransmission | Slots | 31 |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 Low |
| Timing offset from receiving side (Note 3) |  | s | 0 |
| Frequency offset from receiving side (Note 4) |  | Hz | 0 |
| Propagation condition |  |  | AWGN |
| Test metric |  |  | SNR@5% BLER of PSSCH |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: Timing offset from receiving side is with respect to GNSS reference timing. Note 4: Frequency offset from receiving side is with respect to GNSS reference frequency. |  |  |  |

Table 2.2-2： FRC for PSSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Reference channel |  |  | R.PSSCH.2-1.x |
| Channel bandwidth |  | MHz | 20 |
| Subcarrier spacing |  | kHz | 30 |
| Allocated resource blocks |  | RB | 10 |
| CP-OFDM symbols for slot with PSFCH(Note 1) |  |  | 9 |
| CP-OFDM symbols for slot without PSFCH |  |  | 12 |
| Modulation order |  |  | 64QAM |
| MCS index |  |  | 27 |
| Number of MIMO layers |  |  | 1 |
| Number of DMRS REs |  |  | 12 |
| Number of REs for SCI format 1-A |  |  | 240 |
| 2nd stage SCI format 2-A configuration | payloads |  | 35 |
|  | α |  | 1 |
|  | βoffset |  | 2.5 |
| Overhead for TBS determination |  |  | 0 |
| Transport Block Size for slot with PSFCH |  | Bits | 3496 |
| Transport block CRC |  | Bits | 16 |
| Maximum number of HARQ transmissions |  |  | 2 |
| Binary Channel Bits for slots with PSFCH |  |  | 3816 |