3GPP TSG-RAN WG4 Meeting #99-e	R4-2110541

# Electronic Meeting, 19th - 27th May, 2021

Title: 	Updated simulation results on NR IAB-MT demodulation performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	6.3.6.3

Document for:	Information

Background

During RAN4#98-bis-e meeting, the way forward [1] and the simulation assumption [2] for NR IAB demodulation requirements was approved. In this contribution, we provide our updated simulation results about the demodulation requirements for NR IAB MT.

Simulation results

PDSCH

Table 2.1-1 Ideal simulation results on NR IAB PDSCH

| Test | CBW(MHz)/SCS(KHz) | Modulation and code rate | Rank | TDD UL-DL pattern | Propagation condition | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 40/30 | 16QAM, 0.48 | 1 | FR1.30-1 (7DS2U,S:6+4+4) | TDLA30-10 | 2x2 ULA Low | 30 | -1.15 |
| 2 | 40/30 | 16QAM, 0.48 | 1 | FR1.30-1 (7DS2U,S:6+4+4) | TDLA30-10 | 2x4 ULA Low | 30 | -4.13 |
| 3 | 40/30 | 16QAM, 0.48 | 3 | FR1.30-1 (7DS2U,S:6+4+4) | TDLA30-10 | 4x4, ULA Low | 70 | 8.63 |
| 4 | 100/120 | 16QAM, 0.48 | 1 | FR2.120-1 (DDDSU,S:10+2+2) | TDLA30-75 | 2x2 ULA Low | 30 | -1.14 |
| 5 | 100/120 | 64QAM, 0.46 | 1 | FR2.120-1 (DDDSU,S:10+2+2) | TDLA30-75 | 2x2 ULA Low | 70 | 8.51 |
| 6 | 100/120 | 16QAM, 0.48 | 2 | FR2.120-1 (DDDSU,S:10+2+2) | TDLA30-75 | 2x2 ULA Low | 70 | 10.93 |

Figure 2.1-1 Ideal simulation results on NR IAB PDSCH

PDCCH

Table 2.2-1 Ideal simulation results on NR IAB PDCCH

| Test | CBW(MHz)/SCS(KHz) | CORESET RB | CORESET duration | Aggregation level | DCI format | Payload (without CRC) | TDD UL-DL pattern | CCE-to-REG Mapping | Interleaver size | REG bundle size | Propagation condition | Antenna configuration | Pm-dsg (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 40/30 | 102 | 1 | 4 | 1_1 | 53 | FR1.30-1 (7DS2U,S:6+4+4) | Interleaved | 3 | 2 | TDLA30-10 | 1x2 Low | 1 | 1.94 |
| 2 | 40/30 | 90 | 1 | 8 | 1_1 | 53 | FR1.30-1 (7DS2U,S:6+4+4) | Interleaved | 3 | 6 | TDLA30-10 | 2x2 Low | 1 | -3.87 |
| 3 | 40/30 | 102 | 1 | 4 | 1_1 | 53 | FR1.30-1 (7DS2U,S:6+4+4) | Interleaved | 3 | 2 | TDLA30-10 | 1x4 Low | 1 | -2.81 |
| 4 | 40/30 | 90 | 1 | 8 | 1_1 | 53 | FR1.30-1 (7DS2U,S:6+4+4) | Interleaved | 3 | 6 | TDLA30-10 | 2x4 Low | 1 | -7.69 |
| 5 | 100/120 | 60 | 1 | 4 | 1_1 | 56 | FR2.120-1 (DDDSU,S:10+2+2) | Interleaved | 2 | 6 | TDLA30-75 | 1x2 Low | 1 | -0.33 |

Figure 2.2-1 Ideal simulation results on NR IAB PDCCH

Conclusion

In this contribution, we provide our updated simulation results about NR IAB MT demodulation performance.

Reference

R4-2106172, WF on Rel-16 NR IAB demodulation requirements, RAN4#98-bis-e, Nokia

R4-2106815, Updated simulation assumptions for NR IAB-MT demodulation requirements, RAN4#98-bis-e, Huawei, HiSilicon