3GPP TSG-RAN WG4 Meeting # 99-e               	R4-2110606

E-meeting,19th May  – 27th May, 2021

Agenda item:	9.6.2.1

Source: 	ZTE Corporation

Title: 	Initial simulation results for NR 1024QAM

Document for:	Approval

Background

In the previous RAN4 meetings, there were initial discussions on the simulation study for NR 1024QAM to investigate the reasonable EVM requirements. In this contribution, we want to share some initial simulation results for further discussion.

Discussion

Simulation assumptions are listed in the following Table 1.

Table 1. simulation assumption for NR 1024QAM vs 256QAM

| Parameter | Value |
| --- | --- |
| Carrier frequency, SCS, System BW | 3.5GHz, 30kHz, 100 MHz |
| Channel model | TDL-A or TDL-D in TR 38.901 with up to 30ns delay spread |
| UE speed | 0km/h |
| Number of UE antennas | 1T4R |
| Number of gNB antennas | 1T or 2T |
| Tx EVM | 2%, 2.5%, 3% |
| Rx EVM | 2%, 3%, 4% |
| MCS | 256QAM: MCS 24~26 in Table 5.1.3.1-2: MCS index table 2 for PDSCH MCS index; modulation; code rate x 1024; Efficiency. 24; 8; 841; 6.5703. 25; 8; 885; 6.9141. 26; 8; 916.5; 7.1602. 27; 8; 948; 7.4063. 1024QAM: MCS 24~26 in the following Table according to the agreement in RAN1 #104 MCS index; modulation; code rate x 1024; Efficiency. 23; 10; 805.5; 7.8662. 24; 10; 853; 8.3301. 25; 10; 900.5; 8.7939. 26; 10; 948; 9.2578. |
| DMRS type | DM-RS type 1 |
| Number of DMRS symbols | 1 |
| Number of scheduled RBs | 273 (200RBs for data) |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |
| Rank | Rank1 |
| Channel estimation | Realistic channel estimation |
| Metric | Crossover SNR at transition points between 256-QAM and 1024-QAM |

Based on the initial simulation results provided as following, with Tx EVM requirement from 2%-3%, NR 1024QAM could still have more than 20% performance gain than NR 256QAM. In addition, with the increasing Tx EVM requirement for NR 1024QAM, the crossover SNR between 256QAM and 1024QAM will increase from 21dB to 23dB.

Observation 1:  with Tx EVM requirement ranging from 2%-3%, NR 1024QAM could have more than 20% performance gain than NR 256QAM

cross SNR：21,21,21.5 dB

cross SNR：21, 21.5, 23.5 dB

cross SNR：23, 23, 23 dB

The simulation results for TDL-A channel will be provided later once it is ready.

Conclusion

In this contribution, we proved some initial simulation results for the performance comparison between NR 1024QAM and 256QAM and  some observations are made as following:

Observation 1:  with Tx EVM requirement ranging rom 2%-3%, NR 1024QAM could have more than 20% performance gain than NR 256QAM;

Reference

- R4-2105416,Way forward on UE RF requirements for DL1024QAM, Ericsson

- R4-2106858,Work plan for DL 1024QAM for NR FR1