3GPP TSG-RAN WG4 Meeting#99-e                               R4-2110612

E-meeting,19th May– 27th May, 2021

Agenda item:	9.17.2

Source: 	ZTE Corporation

Title: 	Discussion on phase discontinuity and power inconsistency tolerance across different repetitions

Document for:	Approval

Introduction

In the last RAN4 meeting, there were extensive discussions on phase discontinuity for NR coverage enhancement and reply LS [2] has been sent back to RAN1. However there are also some remaining open issues as listed in WF [1] left for further discussions, therefore in this contribution, we want to share some initial considerations for the further progress.

Discussion

## 2.1. Non-zero un-scheduled gap

For non-zero unscheduled gap, the following issues needs further discussion.

- Whether off power requirement can be met on the un-scheduled symbols

- The length of non-zero un-scheduled gap

In the last meeting, the feasibility of phase continuity and power consistency for non-zero un-scheduled gap case for a gap less than 14 symbols was confirmed when UE is not required to meet the existing off power requirements. Based on our understandings, during the non-scheduled gap, there are different approaches to simulate OFF status at UE side. E.g. 1) to keep PA operating point unchanged and PLL configuration unchanged and set power level of baseband at very low level to simulate the OFF status;

- to keep PA operating point unchanged and PLL configuration unchanged and set the power level of Tx chain as same as previous PUCCH/PUSCH transmission and set the attenuator if applicable as maximum level to simulate the OFF status.

- The cons of the above two approaches would introduce additional hardware implementation efforts, however with these approaches, UE OFF power could be still guaranteed which would well protect other surrounding UE’s receiver performance if there are downlink symbols in-between different repetitions.

- If exceptions for OFF power are allowed during non-zero unscheduled, then as mentioned before, other surrounding UE’s receiver performance cannot be guaranteed if there are downlink symbols scheduled in-between different PUSCH/PUCCH repetitions. Even if there are no downlink symbols scheduled in between different repetitions, then UE to transmit signal without network scheduling will also increase the harmful and unnecessary co-channel interference.

- Based on the above considerations, OFF power requirement for non-zero un-scheduled gap should still be guaranteed, however considering the less measurement time for OFF power compared with Rel-15/16, then some extended testing uncertainty would be considered in RAN5.

- Proposal 1: OFF power requirement for non-zero un-scheduled gap should still be guaranteed and some extended testing uncertainty could be considered due to less measurement time compared with Rel-15/16.

- Regarding the length of non-zero un-scheduled gap, we think this could be up to 14 symbols during which phase continuity and power consistency could be still maintained.

- Proposal 2: the length of non-zero un-scheduled gap could be up to 14 symbols;

## 2.2. Non-zero gap with other signals/channels for the UE

- Whether phase continuity can be maintained if the other signals/channels power in-between the repetitions can be different with the power of repetition or if PRB content of the channels/signals in between can be different

- For the above scenarios for other signals different from repetitions, we understand that this might be beneficial for SRS or PUCCH transmissions in-between PUSCH transmissions, however what’s exact UE behavior during the guard period is unknown and whether the remaining symbols except guard period during non-zero gap could accommodate other signals is not clear. In addition, if some phase error might be introduced randomly instead of with regular phase shift, then it might be also not possible to recover the phase continuity after different signal transmissions during non-zero gap.

- Proposal 3: not consider non-zero gap with other signals/channels with different configurations (e.g. power, PRB content) for UE in Rel-17.

## 2.3. Required tolerance of the phase continuity and amplitude consistency

In the last RAN4 meeting, the majority of interested companies support to have some LLS to evaluate the acceptable tolerance from network performance perspective. Therefore in this contribution, the following scenarios are evaluated to check the impacts of phase discontinuity and power inconsistency on the gNB performance and detailed simulation assumptions are listed in Annex 5.

Table 1. evaluation cases for phase continuity and power consistency

| Scenario | Configurations | Note |
| --- | --- | --- |
| Case 1: Phase discontinuity across different repetitions | 700MHz/4GHz, 2 repetition, Baseline: 0 Relative Phase error:10: 20: 150 | Relative phase discontinuity is added on top of 2nd transmission repetition |

Figure 1. The performance impact of phase error on joint channel estimation for PUSCH for 700MHz

Figure 2. The performance impact of phase error on joint channel estimation for PUSCH for 4GHz

Based on initial simulation results in Figure 1/2, it can be found that 150o phase error will cause significant performance degradation, the acceptable phase error between different repetitions should be within 10o-30o. [Simulation results for 4GHz will be added later]

Proposal 4: propose the phase error between different repetitions within 10o-30o.

Table 2. evaluation cases for the power consistency

| Scenario | Configurations | Note |
| --- | --- | --- |
| Case 2: Power inconsistency across different repetitions | 700MHz/4GHz, 2 repetition, Baseline : 0 Relative Amplitude error: 0: 0.5: 2dB | Relative power inconsistency is added on top of 2nd transmission repetition |

Figure 3. The performance impact of amplitude error on gNB receiver performance for 700MHz

Figure 4. The performance impact of amplitude error on gNB receiver performance for 4GHz

Based on initial simulation results in Figure 3/4, it can be found that 0.5dB amplitude error will also cause significant performance degradation (e..g at least 1.5-2dB deterioration @10% BLER), the acceptable phase error between different repetitions should be less than 0.5dB. [Simulation results for 4GHz will be added later]

Proposal 5: propose the amplitude error between different repetitions less than 0.5dB.

Conclusion

- In this contribution, we shared some further analysis on the remaining open issues listed in the approved WF [1] and some proposals and observations are made as following:

- Proposal 1: OFF power requirement for non-zero un-scheduled gap should still be guaranteed and some extended testing uncertainty could be considered due to less measurement time compared with Rel-15/16.

- Proposal 2: the length of non-zero un-scheduled gap could be up to 14 symbols;

- Proposal 3: not consider non-zero gap with other signals/channels with different configurations (e.g. power, PRB content) for UE in Rel-17.

Proposal 4: to propose the phase error between different repetitions within 10o-30o.

Proposal 5: to propose the amplitude error between different repetitions less than 0.5dB.

References

- R4-2105418,Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition, approved

- R4-2105417, Reply LS on PUCCH and PUSCH repetition, RAN4, approved.

- R4-2105477,Email discussion summary for [98-bis-e][140] NR_cov_enh

Annex: Simulation assumptions

- Table A-2 Simulation assumption for phase tolerance level and amplitude tolerance level

| Parameters | Values |  |
| --- | --- | --- |
| - Scenario | - Urban , O2I | - Rural, O2I |
| - Duplexing scheme and frequency | - 4GHz TDD | - 700MHz FDD |
| - Frame structure for TDD | - DDDSUDDSUU (S: 10D:2G:2U) for 4GHz |  |
| - Transmission bit rate for data channel (bit/s) | - 1 Mbps for eMBB | - 100 kbps for eMBB |
| - Subcarrier spacing | - 30kHz | - 15kHz |
| - Channel model for link-level simulation | - TDL-C | - TDL-C |
| - Pathloss model (select from LoS or NLoS) | - NLos | - NLos |
| - Delay Spread | - 300ns | - 300ns |
| - UE velocity | - 3 km/h | - 3 km/h for O2I |
| - Number of RF chains for BS | - 4 | - 4 |
| - Number of UE antennas | - 1 | - 1 |
| - Number of RF chains for UE | - 1 | - 1 |
| - Number of occupied RBs | - 15 | - 4 |
| - MCS | - MCS4 | - MCS2 |
| - DMRS overhead | - Type I, 2 DMRS symbol (one front- loaded and one additional), no multiplexing data |  |
| - Frequency hopping | - Disabled |  |
| - Channel estimation | - Practical |  |
| - Receiver type | - MMSE |  |
| - BLER Target | - 10% iBLER for eMBB |  |
| - Waveform | - DFT-s-OFDM |  |
| - Number of repetitions | - 2 repetitions with JCE |  |