3GPP TSG-RAN WG4 Meeting #98-bis-e                                       R4-2110716 Electronic Meeting, May 19-27, 2021

Source: 	Huawei, HiSilicon

Title: 	Consideration for RMR 900MHz band

Agenda Item:	8.5.2

Document for:	Discussion

# Introduction

A new WI on introduction of 900 MHz spectrum to 5G NR applicable for Rail Mobile Radio was approved in RAN#91e meeting [1].

This contribution provides our consideration on RMR 900MHz band from UE perspective.

# Discussion

The adjacent bands around RMR 900MHz are illustrated in Figure 1.

Figure 1: Operating bands around RMR 900MHz

Interference scenarios from RMR to B8/n8 can be summarized as

- RMR UL to B8/n8 UL

- RMR UL to B8/n8 DL

- RMR DL to B8/n8 UL

- RMR DL to B8/n8 DL

Scenario (a) and (c) are related to base station performance and are not discussed in this contribution. In scenario (b), the UL of RMR 900MHz is below 880MHz, namely further away from b8/n8 DL band, use of existing requirements is adequate to protect the performances in b8/n8 bands. The WI [1] limits the bandwidth to 5MHz, the extra room of 0.6MHz in RMR allocation away from adjacent channel would reduce the effective ACLR into b8/n8 by less than 0.5dB, which is not significant. However it could aggravate its impact in scenario (c), which is not addressed in this contribution.

Scenario (c) is analogous to an adjacent channel and the existing requirements are sufficient from UE receive perspective provided that the RMR 900MHz DL requirements are also in-line with the existing BS requirements.

The existing requirement of -50dBm/MHz in spurious co-existence for protected bands is sufficient for scenario (b). As the use of RMR bands is specific to Europe, the protected bands would only consist of those used in Europe.

Proposal 1: adopt the existing UE requirements for RMR 900MHz with specific details below.

Operation band is introduced

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| nxx | 874.4 MHz – 880 MHz | 919.4 MHz – 925 MHz | FDD |

Channel bandwidth is introduced

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| nxx | 15 | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 | yes |  |  |  |  |  |  |  |  |  |  |  |  |

Channel raster and NR-ARFCN are introduced.

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nxx | 100 | 174880 – <20> – 176000 | 183880 – <20> – 185000 |

Synchronization raster entries are introduced.

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| Nxx | 15 kHz | Case A | 2303 – <1> – 2307 |

UE Tx and Rx frequency separation is introduced.

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| nxx | 45 MHz |

UE maximum output power is introduced.

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| nxx |  |  |  |  | 23 | ±2 |

Two antenna port reference sensitivity for QPSK is introduced.

|  |  | Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| nxx | 15 |  | -97.0 |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -97.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Band nxx shall be added to Table 6.5.3.2-1.

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| nxx | E-UTRA Band 1, 20, 28, 32, 38, 40 NR Band n8, n91, n92, n93, n94 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |

Table 7.6.2-2 for in band blocking shall be updated with band nxx.

Table 7.6.3-2 for out of band blocking shall be updated with band nxx.

Table 7.6.4-1 for narrow band blocking shall be updated with band nxx.

# Conclusion

This contribution discusses the rationale to use existing NR requirements for RMR 900MHz band.

Proposal 1: adopt the existing UE requirements for RMR 900MHz with specific details given in section 2.

# References

[1] RP-210878, New WID on introduction of 900 MHz spectrum to 5G NR applicable for Rail Mobile Radio, UIC