3GPP TSG-RAN WG4 Meeting #99															R4-2110814

Electronic meeting, May 19th – 27th 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	NR-U - System parameters

Agenda item:	6.6.1

Document for:	Approval

# Introduction

NR-U have been introduced to NR Rel-16 and is currently in the maintenance phase. This contribution addresses some of the remaining open issues.

# Discussion

#### Band n96

When band n96 was originally defined it was on the basis on regulations provided by the FCC for operation in USA. Since then multiple countries, as South Korea [1] and Brazil [2] naming a few, have adopted the same frequency range (i.e.  5925 – 7125MHz) for unlicensed operation. This as wells as the fact that other countries are considering the same would imply that the restriction of n96 for US operation only should be revisited.

Proposal 1:	The restriction of n96, for US operation only, should be discussed by RAN4.

Currently the note in 38.101-1 reads:
NOTE 14: This band is applicable in the USA only subject to FCC Report and Order [FCC 20-51]

As captured in WF from RAN4#98bis [3] two options are proposed:

- Option 1: Modify the note according to R4-2106273:
This band is applicable in the USA subject to FCC Report and Order FCC 20-51 and in Europe subject to CEPT ECC Decision (20)01 for the 5945 to 6425 MHz frequency range.

- Option 2: Modify the note to generic text:
This band is only applicable subject to regional and/or country specific restrictions.

To limit the need for future modifications of the note it is suggested to adopt option 2 and introduce a generic note for band n96.

Proposal 2:	Modify the note for n96 according to option 2 making it generic.

It should be emphasised that even by removing the regional restriction in the 3GPP specification for unlicensed operation in band n96 they are still enforced by NS signalling for the UE and regulatory compliance requirements for the BS. Further, there should be no doubt that if/when regulations are ready for licensed operation in either the same or parts of the same frequency range a band shall per normal RAN4 procedure be defined accordingly.

Observation 1: 	Enabling wider use of band n96 for unlicensed operation does not in any way preclude a future introduction of a licensed band covering either the entire or parts of the same frequency range.

#### Intra-cell guardband for NR-U operation

During RAN4#97 a CR [4] was agreed addressing the intra-cell guard bands applicable for shared spectrum operation in wideband operation mode. During the final discussions related to this CR a part of the text in section 5.3.3 of 38.101-1 was put in brackets. In specific: “[For a UE supporting wideband operation], the nominal intra-cell guard bands…”.

Reviewing the discussion during RAN4#98 we propose to remove these brackets as we have no concerns with this wording.

Proposal 3: Resolve the brackets in section 5.3.3 of TS 38.101-1 by removing the brackets.

# Conclusion

In this contribution, we comment on how remaining unresolved for NR-U can be concluded.

Proposal 1:	The restriction of n96, for US operation only, should be discussed by RAN4.

Proposal 2:	Modify the note for n96 according to option 2 making it generic.

Observation 1: 	Enabling wider use of band n96 for unlicensed operation does not in any way preclude a future introduction of a licensed band covering either the entire or parts of the same frequency range.

Proposal 3: Resolve the brackets in section 5.3.3 of TS 38.101-1 by removing the brackets.

# References

- Korea’s Ministry of Science and ICT, "Technical standards for radio equipment for radio stations", URL: https://www.law.go.kr/admRulLsInfoP.do?admRulSeq=2100000196974

- National Telecommunications Agency (ANATEL), ACT NO. 1306 OF 26 FEBRUARY 2021

- R4-2105383, WF on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia

- R4-2017845, Correction to the intra-cell guard band definition for wideband operation, Ericsson