3GPP TSG-RAN WG4 Meeting #99-e		R4-2110842
Electronic Meeting, 19th May – 27th May, 2021

Agenda item:	9.2.3

Source:	OPPO

Title:	Power setting for EN-DC test

Document for:	Approval

1	Introduction

A new Work Item on FR1 TRP TRS was approved in the meeting of RAN #91-e [1]. As initiating the core part discussion, this paper presents our consideration and proposal on power configuration for EN-DC (1 CC LTE with 1 CC NR).

2	Discussion

For the UEs which do not support Dynamic Power Sharing, the traditional procedure can be used to ensure that the DUT transmits with its maximum power as described in TS 36.521-1 below.

For the UEs which support Dynamic Power Sharing, the transmitted power on NR will be affected by the power transmitted on LTE side considering the total allowed transmit power is restricted by Power Class level. Therefore, the power on LTE side should be controlled under a reasonable range.

The proposed configuration is setting Pmax of LTE as half of maximum output power for EN-DC defined in TS 38.101-3 Table 6.2B.1.3-1. The reason of this proposal is based on the scenario that LTE and NR have similar network coverage in the field, and therefore the two radio link of LTE and NR have similar transmit power.

Proposal: the power configuration for EN-DC is setting Pmax of LTE as half of maximum output power for EN-DC defined in TS 38.101-3 Table 6.2B.1.3-1.

3	Conclusion

Proposal: the power configuration for EN-DC is setting Pmax of LTE as half of maximum output power for EN-DC defined in TS 38.101-3 Table 6.2B.1.3-1.

References

- RP-210807, New WID: Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)