3GPP TSG-RAN WG4 Meeting #99-e		R4-2110843
Electronic Meeting, 19th May – 27th May, 2021

Agenda item:	9.1.3.2

Source:	OPPO

Title:	Quantify the FR2 blocking issue

Document for:	Approval

1	Introduction

In the last meeting of RAN4 #98-bis-e, the agreement on the FR2 blocking issue is “the blocking issue of 3D-MPAC system is not properly presented, more study is needed.” Based on the agreement, the three-step simulation/measurement approach was proposed to quantify the blocking issue of Probe 3, which was regarded as starting point in the WF [1]

This paper presents the proposal to improve the approach and further specifically quantify the blocking issue.

2	Discussion

The three-step simulation/measurement approach [2] is capable of quantify the effect of blocking issue ON THE CENTRE of the test zone, as the blue dot shown in Figure 1, because the reference antenna is placed in the centre of the test zone during the measurement procedure. However, the worst case will probably happen on the edge of the test zone where is closer to the pedestal, as the red curve shown in Figure 1. Therefore, to quantify the effect of blocking issue, simulation/measurement only on the centre of the test zone is not sufficient.

Figure 1	Illustration of FR2 blocking issue

Observation: simulation/measurement only on the centre of the test zone is not sufficient to quantify the effect of blocking issue.

In order to quantify the blocking issue under the worst case, validation/measurement on the edge of the test zone should be taken into account. Considering the measurement effort in terms of test time consuming, the proposal is to perform the three-step approach, besides on the centre of the test zone, on the following two additional test points, as the red dots shown in Figure 2.

M1: (0, 0, RTZ)

M2: (RTZ*sin(180-161.7), 0, RTZ*cos(180-161.7))

Note: RTZ is the radius of the test zone.

Figure 2	Suggested test points when performing the three-step approach

Proposal: besides the centre of the test zone, the three-step approach shall be performed on two additional test points, which are

M1: (0, 0, RTZ)

M2: (RTZ*sin(180-161.7), 0, RTZ*cos(180-161.7))

Note: RTZ is the radius of the test zone.

3	Conclusion

Observation: simulation/measurement only on the centre of the test zone is not sufficient to quantify the effect of blocking issue.

Proposal: besides the centre of the test zone, the three-step approach shall be performed on two additional test points, which are

M1: (0, 0, RTZ)

M2: (RTZ*sin(180-161.7), 0, RTZ*cos(180-161.7))

Note: RTZ is the radius of the test zone.

References

- R4-2106092 WF on NR MIMO OTA v4

- R4-2106144 Summary_326_after 2nd round