3GPP TSG-RAN WG4 Meeting # 99-e 	R4-2110845

Electronic Meeting, 19th – 27th May, 2021

Agenda Item: 9.19.2

Source: MediaTek Inc.

Title: Discussion on RRM core requirements impact for RedCap

Document for: Discussion

# Introduction

In this paper, the RRM core impact on reduced capability (RedCap) specs impact are analyzed and our views are provided.

# Discussion

In Rel-17 WID on support of reduced capability NR devices [1], the objective are specified for UE complexity reduction features. Thus, the RRM core requirement impact on RedCap shall be identified and the overview is provided as in Table 1.

Table 1: The overview of the RRM core requirement impact of the RedCap in Rel-17

| Section in TS 38.133 | requirement | impact | reason |
| --- | --- | --- | --- |
| 4 | IDLE mode | Yes | - There is a new DRX cycle length for RRC Idle mode. New tables for eDRX cycle need to be introduced in a similar manner to eDRX tables in TS 36.133 - Pending on RAN2 decision on RRM relaxations for neighbouring cells for RedCap devices |
| 5 | INACTIVE mode | Yes | - There is a new DRX cycle length for RRC Inactive mode - Pending on RAN2 decision on RRM relaxations for neighbouring cells for RedCap devices |
| 6 | CONNECTED mode | FFS | - Potential relaxation to the delay requirements |
| 8.1 | RLM | FFS | - Reduced minimum number of Rx branches |
| 8.4 | UL carrier RRC reconfiguration | FFS | - Pending on the conclusion of RF session on whether to introduce SUL for Redcap UEs |
| 8.5 | Link recovery (BFD/CBD) | FFS | - Reduced minimum number of Rx branches |
| 9.1.2 | measurement gap | FFS | - Reduce the measurement gap patterns |
| 9.1.3 | measurement capability (# of layers, cells and beams) | FFS | - Reduce the number of frequency layers, cells and beams |
| 9.2 | Intra-frequency measurement | YES | - Introduce scheduling availability of UE performing measurements in HD-FDD bands on FR1 (because it cannot transmit and receive at the same time) - Reduce/simplify the requirements of Carrier-specific scaling factor because CA is excluded in RedCap - Up to RAN4 discussion whether to extend the measurement delay or relax the measurement accuracy - Pending on RAN2 decision on RRM relaxations for neighbouring cells for RedCap devices |
| 9.3 | Inter-frequency measurement | FFS | - Up to RAN4 discussion whether to extend the measurement delay or relax the measurement accuracy - Pending on RAN2 decision on RRM relaxations for neighbouring cells for RedCap devices |
| 9.4 | Inter-RAT measurement | FFS | - This topic is not discussed in RAN2 and hence the inter-RAT can be either included or excluded based on RAN4 discussions. |
| 9.5 | L1-RSRP measurements | FFS | - Up to RAN4 discussion whether to extend the measurement delay or relax the measurement accuracy |
| 10 | Measurement accuracy | FFS | - Potential relaxation for Redcap UEs. Can be postponed to performance part |
| A.3.6 | Antenna configurations | YES | - Reduced minimum number of Rx branches. Can be postponed to performance part |

Some of the requirements in Table 1 may or may not be impacted by the RedCap RRM requirements, hence we need to further study these topics in RAN4 meeting.

Proposal 1: The requirements of idle/inactive mode, connected mode, RLM, UL carrier RRC reconfiguration, link recovery, measurement gap, measurement capability, intra-frequency measurement, inter-frequency measurement, inter-RAT measurement, L1-RSRP measurements, Measurement accuracy and antenna configurations shall be further discussed in the work item of RedCap.

# Summary

In this paper, the overview impact on RRM requirements is provided and we have the following proposals:

Proposal 1: The requirements of idle/inactive mode, connected mode, RLM, UL carrier RRC reconfiguration, link recovery, measurement gap, measurement capability, intra-frequency measurement, inter-frequency measurement, inter-RAT measurement, L1-RSRP measurements, Measurement accuracy and antenna configurations shall be further discussed in the work item of RedCap.

# References

- RP-210918, “Revised WID on support of reduced capability NR devices”.