3GPP TSG-RAN WG4 Meeting #99-e	R4-2110873

Electronic Meeting, 19 – 27 May, 2021

Title: 	Discussion on remaining issues for PRS-RSRP measurement requirements

Source: 	Huawei, HiSilicon

Agenda item:	6.5.1.2

Document for:	Discussion

Introduction

RRM requirements for PRS-RSRP measurements were discussed in RAN4#98-bis-e, and the outcomes are captured in the WF [1]. Based on [1] the following issues are to be further discussed:

- Measurement period when configured for DL-TDOA or multi-RTT

In this paper we will provide our views on the above open issues for PRS-RSRP measurement.

Discussion

- PRS-RSRP requirements when configured for DL-TDOA or multi-RTT - Option 1 (QC) - According to a prior agreement, RAN4 may revise the number of samples (Nsample) in the requirement if more samples are needed to meet PRS-RSRP accuracy requirements. - Option 2a (vivo, HW, CATT, OPPO, Intel) - UE behaviour is not defined when PRS-RSRP is configured additionally to RSTD or UE Rx-Tx measurement. - Current requirements in clause 9.9.3 also apply for the case when PRS-RSRP is measured for DL-TDOA or Multi-RTT. - Option 2b (Nokia, Intel) - Regarding UE behavior, a UE continues to measure PRS-RSRP over the entire RSTD/UE Rx-Tx measurement period, when PRS-RSRP is configured together with RSTD/UE Rx-Tx. - RSRP requirements in clause 9.9.3 also apply for the case when configuring PRS-RSRP with RSTD or UE Rx-Tx. - Option 3 (Ericsson, Nokia) - UE behavior when PRS-RSRP is configured together with RSTD/UE Rx-Tx and the required PRS-RSRP measurement period is shorter than that for RSTD/UE Rx-Tx (configured without PRS-RSRP), then the PRS-RSRP measurement continues over the entire RSTD/UE Rx-Tx measurement period.

When UE is configured with PRS-RSRP measurement for DL-TDOA, the PRS-RSRP is measured on the same set of PRS resources as RSTD. In [1] it was agreed that

- When PRS-RSRP is configured for DL-TDOA, RSTD and RSRP are performed over the same measurement period.

This agreement means that when PRS-RSRP is configured for DL-TDOA, the measurement period requirements will be specified in clause 9.9.2 of 38.133 which defines RSTD measurement period, or more accurately the measurement period for DL-TDOA. In this sense, there is no need to apply the requirements in clause 9.9.3 for PRS-RSRP measurement configured for DL-TDOA.

This would also resolve the compatibility issue raised by some companies in RAN4#98-bis-e, e.g. in case of HO, UE would continue the PRS-RSRP measurement if configured for DL-TDOA (same as RSTD), and would restart the PRS-RSRP measurement if configured for multi-RTT (same as UE Rx-Tx).

Proposal 1: Add the following texts in clause 9.9.2 (9.9.4) of 38.133:

“When PRS-RSRP is configured for DL-TDOA (multi-RTT), RSTD (UE Rx-Tx) and RSRP measurements are performed over the same measurement period defined in this clause.”

This means the requirements in clause 9.9.3 of 38.133 apply only for PRS-RSRP measurement configured for DL-AoD, which is same as the current applicability.

Conclusions

In this paper we provided our views on PRS-RSRP measurement requirements.

Proposal 1: Add the following texts in clause 9.9.2 (9.9.4) of 38.133:

“When PRS-RSRP is configured for DL-TDOA (multi-RTT), RSTD (UE Rx-Tx) and RSRP measurements are performed over the same measurement period defined in this clause.”

Reference

- R4-2105851, WF on UE PRS measurement requirements, Huawei, HiSilicon