3GPP TSG-RAN4 Meeting #99-e	R4-2110874

# Electronic Meeting, May 19–27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2089 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to update PRS-RSRP measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are some remaining open issues in PRS-RSRP measurement period: - PRS resource muting is not accounted - Parameter Lprs is not specified - Impact of MG reconfiguration is not accounted - Measurement period with serving cell change for PRS-RSRP configured for DL-TDOA and multi-RTT is specified in clause 9.9.3, but it should be specified in clause 9.9.2 and 9.9.4 based on RAN4#98-bis-e agreement that When PRS-RSRP is configured for DL-TDOA, RSTD and RSRP are performed over the same measurement period. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the PRS-RSRP measurement period requirements for above issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PRS-RSRP measurement period requirements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.9.3.5	Measurement Period Requirements

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured PRS frequency layers, within TPRS-RSRP,total ms.

If measurement gaps and processing time T have overlap between different frequency positioning frequency layers,

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1*maxTeffect,i

Editor’s note: FFS the PRS-RSRP measurement period when measurement gaps and processing time T do not have overlap between different positioning frequency layers.

where

i is the index of PRS frequency layer,

L is total number of positioning frequency layers,

TPRS-RSRP,i=CSSFi*NRxBeam,i*NPRS,islotN'LPRS,iKiN*Nsample-1*Teffect,i+Tlast

where

CSSFi is the carrier specific scaling factor for PRS-RSRP measurements specified in clause 9.1.5.2,

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if PRS layer i is in FR1 and NRxBeam,i=8 if PRS layer i is in FR2,

LPRS,i Ki is parameter K for positioning frequency layer i as defined in clause 5.1.6.5 of TS 38.214 [26], and only accounts for duration of PRS resource instances within measurement gap and are not muted.

NPRS,islot is the maximum number of DL PRS resources of frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot  in clause 6.4.3 of TS 37.355 [34],

Nsample is the number of PRS-RSRP measurement samples and Nsample= 4,

Tlast = Ti + LPRS,i is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time,

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in frequency layer i,

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi is the least common multiple between TPRS,i and MGRPi ,

TPRS,i=LCMj(Tresource,j*Nmuting,j) where

Tresource,j is the configured periodicity of PRS resource j on the positioning frequency layer i which is fully or partially overlapped with measurement gap, and

Nmuting,j=X*Y is the scaling factor if muting option 1 is applied on PRS resource j, where X is the size of NR-MutingPattern-r16 for mutingOption1-r16, and Y is the value of dl-prs-MutingBitRepetitionFactor.

MGRPi is the measurement gap repetition period in frequency layer i.

If frequency layer i has more than one DL PRS resource set with different PRS periodicities, TPRS,i is the least common multiple of PRS periodicities among the DL PRS resource sets on frequency i.

When PRS-RSRP measurements are configured for DL-AoD, the time TPRS-RSRP,i starts from the first MG instance aligned with DL PRS resources of positioning frequency layer i closest in time after both the NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

The requirements in this section apply, provided no PRS symbols are dropped during the measurement period TPRS-RSRP,Total within measurement gaps due to collisions with other signals; otherwise, a longer measurement period may be used.

If handover occurs while PRS-RSRP measurements are being performed then the UE shall complete the ongoing PRS-RSRP measurements session. The UE shall also meet the PRS-RSRP measurement requirements in this clause and measurement accuracy requirements in clause 10.1.24. However, in this case the PRS-RSRP measurement period TPRS-RSRP,total.HO shall be as follows:

TPRS-RSRP, total,HO=TPRS-RSRP, total+KNHO*Teffect+THO   ms

where

NHOK is the number of times handover occurs during TPRS-RSRP,total.HO;

Teffect is the largest Teffect,i among all PRS layers;

THO is the time during which the PRS-RSRP measurement may not be possible due to handover; it can be up to Tinterrupt as defined in clause 6.1.

If during the measurement period of one or more positioning frequency layers, the measurement gap is reconfigured, the measurement period can be longer.

<End of Change 1>