3GPP TSG-RAN WG4 Meeting # 99e 	R4-2110957

# Electronic Meeting, 19 – 27 May 2021

Source:		Huawei, HiSilicon

Title:			TP for TR 37.xxx for DC_2A-66A_n41A

Agenda item:		8.39.2

Document for:		Approval

# Introduction

This contribution is a text proposal for TR 37.xxx to include DC_2A-66A_n41A according to the request in [1].

Reference

[1] RP-210816, New WID on PC2 EN-DC with x LTE band + y NR band (x= 2, 3, 4, y=1; x=1, 2, y=2)

# Text Proposal

<Start of Text Proposal>

## 6.x	DC_2A-66A_n41A

### 6.x.1	Transmitter Characteristics

#### 6.x.1.1	Maximum Output Power

Table 6.x.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n41A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 6.x.1.2	Co-existence study

According to the PC3 DC_2A_66A-n41A study in 37.716-21-11, the Rx impacts are identified as below,

- Co-existence analysis for DC_2_n41 shows that there is no impact from DC_2_n41 UL to Band 66 DL.

- Co-existence analysis for DC_66_n41 shows that there is IMD4 impact from DC_66_n41 UL to Band 2 DL.

Thus, additional MSD for IMD 4 should be considered to mitigate the impact of the interference for PC2 DC_2A_66A-n41A combination.

### 6.x.2	Receiver Characteristics

#### 6.x.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 6.x.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_2A_66A-n41A are defined in table 6.x.2.2.1-1.

Table 6.x.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n41A | 2 | 1860 | 5 | 25 | 1940 | 22.6 | IMD4 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 5 | 25 | 2685 | N/A | N/A |

#### 6.x.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_2A_66A-n41A are the same as the Case A defined in table 6.x.2.1.1-1.

<End of Text Proposal>