3GPP TSG-RAN WG4 Meeting # 99-e 			R4-2111221

# Electronic Meeting, May 19-27, 2021

Source: 		Nokia, Nokia Shanghai Bell

Title: 			On 1900MHz RMR RAN4 requirements impact due to ECC Decision (20)02

Agenda Item: 	8.6.1

Document for:	Approval

# Introduction

During RAN#91e meeting, a new Work Item [1] has been approved on introduction of Rail Mobile Radio (RMR) 1900MHz spectrum (Uplink/Downlink: 1900 MHz – 1910 MHz). This document discusses impact to RAN4 requirements due to ECC Decision (20)02.

# Discussion

Below is the summary of relevant ECC Decision 20(02) requirements relevant to objectives of this Work Item. Proposals have been made how they should be addressed (on top of generic changes applicable to introduction of each 3GPP frequency band) in RAN4 specifications. Since the focus of this Work Item is on UE power class 3, requirements for RMR cab-radio are not considered in this document.

Table 1: General in-block requirement
mandatory for uncoordinated deployment

| RMR channel BW | Maximum e.i.r.p. |
| --- | --- |
| 10 MHz | = 65 dBm/10MHz (Note 1) |
| Note 1: In case an administration wishes to allow higher e.i.r.p., coordination or other mitigation measures are required. |  |

Table 2: Baseline requirement

| Frequency range | e.i.r.p. limit |
| --- | --- |
| 1920-1980 MHz | -43 dBm/5MHz |

Proposal 1: Since additional ECC BS transmitter requirements are not conducted, it is proposed to define them as 3GPP BS declarations.

For radio access technologies other than GSM-R, the following parameters apply:

Maximum output power: 23 dBm;

ACLR: 30 dB minimum;

Uplink power control is mandatory and shall be activated.

Since generic RF requirements defined in 38.101-1 cover ECC requirements above, there is no impact to 38.101-1 due to ECC UE power class 3 requirements.

Proposal 2: For UE power class 3, there is no impact to 38.101-1 due to ECC Decision (20)02.

Table 3: Requirements on wideband RMR BS receiver characteristics

| Parameter | Value |
| --- | --- |
| Level of the wanted signal | RefSens + 3 dB |
| Maximum 5 MHz LTE interfering signal in 1805-1880 MHz | -20 dBm |
| The antenna connector of the BS receiver is the reference point. The reference sensitivity (RefSens) is the minimum mean power received at the antenna connector at which a specified minimum performance shall be met. These requirements cover both blocking and third-order intermodulation. |  |

Proposal 3: It is proposed to consider new blocking requirement (conducted) for 1900MHz RMR band.

# Conclusion

This document proposes how to address ECC requirements in 3GPP specifications. The following proposals are made:

Proposal 1: Since additional ECC BS transmitter requirements are not conducted, it is proposed to define them as 3GPP BS declarations.

Proposal 2: For UE power class 3, there is no impact to 38.101-1 due to ECC Decision (20)02.

Proposal 3: It is proposed to consider new blocking requirement (conducted) for 1900MHz RMR band.

# References

[1] RP-210879	New WID on introduction of Rail Mobile Radio (RMR) 1900MHz spectrum; UIC; RAN#91-e