3GPP TSG-RAN WG4 Meeting#99-e                	R4-2111256

Electronic Meeting, 19 – 27 May 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on inter-frequency measurements for FR1 HST CA

Agenda item:	9.7.2.1.3

Document for:	Discussion

Introduction

During the RAN4#98bis-e meeting, a way forward on NR RRM enhancements for FR1 HST [1] was approved. According to [1], it was agreed to enhance inter-frequency measurements in connected mode for CA while the support of such enhancements is up to UE capability.

- Inter-frequency measurement - Define RRC Connected state inter-frequency measurement enhancements - Support of HST inter-frequency measurement enhancements is up to UE capability. Details are FFS - FFS whether enhancements for RRC IDLE inter-frequency measurements are needed

In this contribution, we discuss a possible enhancement for inter-frequency enhancements in the connected mode and our observations on the need of inter-frequency measurement enhancements in the idle mode.

Discussion

As the target UE speed and deployment scenario is the same as for the single carrier case, the cell detection time for inter-frequency with time index (Tidentify_inter_with_index) and without time index (Tidentify_inter_without_index) should be on the same order as the corresponding cell detection time for intra-frequency. As such, the same methodology for Rel-16 FR1 HST can be applied. The proposed enhancements are shown in Tables 1-3.

Table 1: Time period for PSS/SSS detection (Frequency range FR1) for HST

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, N Note 4 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(N Note 4* M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | N Note 4 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 4: When highSpeedMeasFlag-r16 is not configured, N = 8; otherwise N = 5. |  |

Table 2: Time period for time index detection (Frequency range FR1) for HST

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120ms, 3 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(3 M2 Note 3 ) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 3 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 |  |

Table 3: Measurement period for inter-frequency measurements with gaps (Frequency FR1) for HST

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, 5 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle≤ 160ms | Max(200ms, Ceil(5 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| 160 ms ≤DRX cycle ≤ 320ms | 4 M2 Note 3 DRX cycle CSSFinter |
| DRX cycle > 320ms | Y Note 4 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 NOTE 4: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms |  |

Proposal 1: For HST, the inter-frequency measurements in connected mode are enhanced according to Tables 1-3.

In the idle mode, inter-frequency measurement would have lower priority than intra-frequency measurements. Further, it increases battery energy consumption of UE. Intra-frequency measurements seem to be sufficient to ensure operation.

Observation 1: It is not clear whether idle mode inter-frequency measurements are needed, considering it increases battery energy consumption of UE.

Conclusion

The document has discussed and proposed a possible enhancement for inter-frequency measurement for FR1 HST CA.

Proposal 1: For HST, the inter-frequency measurements in connected mode are enhanced according to Tables 1-3.

Table 1: Time period for PSS/SSS detection (Frequency range FR1) for HST

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, N Note 4 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(N Note 4* M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | N Note 4 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 4: When highSpeedMeasFlag-r16 is not configured, N = 8; otherwise N = 5. |  |

Table 2: Time period for time index detection (Frequency range FR1) for HST

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120ms, 3 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(3 M2 Note 3 ) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 3 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 |  |

Table 3: Measurement period for inter-frequency measurements with gaps (Frequency FR1) for HST

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, 5 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle≤ 160ms | Max(200ms, Ceil(5 M2 Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| 160 ms ≤DRX cycle ≤ 320ms | 4 M2 Note 3 DRX cycle CSSFinter |
| DRX cycle > 320ms | Y Note 4 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 NOTE 4: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms |  |

Observation 1: It is not clear whether idle mode inter-frequency measurements are needed, considering it increases battery energy consumption of UE.

References

[1]    R4-2105793, WF on FR1 HST RRM, CMCC