3GPP TSG-RAN WG4 Meeting # 99-e                                                        	R4-2111458

# Electronic Meeting, May 19-27, 2021

Agenda Item:	9.1.3.1

Source: 			Huawei, HiSilicon

Title:                 TP to TS38.151: on DL Pmax and additional restriction for FR1 band frequency >3GHz

Document for:	Approval

# 1	Introduction

As discussed in [1], we propose the following TP to TS38.151 to revise the definition of DL Pmax and additional restriction for FR1 band frequency >3GHz

# 2	Text Proposal to TS 38.151

--------------Start of text proposal -------------

### 6.1.2	Total Radiated Multi-antenna Sensitivity (TRMS)

The average TRMS of free space data mode portrait (FS DMP), free space data mode landscape (FSDML), and free space data mode screen up (FS DMSU), is defined as the FR1 MIMO OTA requirement. The averaging shall be done in linear scale for the TRMS results at these DUT positions, according to the formula:

where

Such that MODE is one of {FS_DMP, FS_DML, FS_DMSU}, and {PMODE,70,0, …, PMODE,70,11} are the measured sensitivity values at each azimuth position at the 70% throughput outage.

If 1 azimuth position does not result in a defined measured sensitivity at 70% throughput, SMODE,70 is calculated using the 11 measured sensitivities and the maximum downlink RS-EPRE PRS-EPRE-MAX (substitution approach) for the one missing result. PRS-EPRE-MAX is the maximum downlink RS-EPRE supported by the test system, and is defined as

[-80dBm/15kHz (or equivalent 77dBm/30kHz)].

The TRMS shall be measured at the mid channel as specified in TS 38.508-1 subclause 4.3.1 [7]. The average TRMS shall be lower than the average TRMS requirements specified in Clause 6.2.

The additional criterion in azimuthal orientations shall be met:

- The EUT must meet 70% throughput in 11 of total 12 azimuthal orientations. If the EUT fails to meet this criterion even under maximum downlink power condition (i.e. PRS-EPRE-MAX), the EUT shall fail the FR1 MIMO OTA test.

- For FR1 band frequency <3GHz, the EUT must meet 90% throughput in [10] of total 12 azimuthal orientations. If the EUT fails to meet this criterion even under maximum downlink power condition (i.e. PRS-EPRE-MAX), the EUT shall fail the FR1 MIMO OTA test.

- For FR1 band frequency >3GHz, the EUT must meet 90% throughput in [9] of total 12 azimuthal orientations. If the EUT fails to meet this criterion even under maximum downlink power condition (i.e. PRS-EPRE-MAX), the EUT shall fail the FR1 MIMO OTA test.

--------------End of text proposal -------------

# Reference

[1] R4-2104512 3GPP TS 38.151 v0.3.0, vivo