3GPP TSG RAN WG4 ad-hoc Meeting 									R4-B12ah-0013

Chicago, U.S.A, September 29-30, 2010

# Title:	Further analysis of Band 12 UE Interference Issues

Source:	Samsung

Agenda Item:	3

Document for:		Discussion

# Introduction

The partitioning of lower 700MHz by FCC is shown in Figure 1 below, where band 12 UL and DL coexist with digital TV and mediaFLO.  High Power TV (HPTV) channels, Ch50 and Ch51, are allowed up to 1 MW ERP emission. Low Power TV (LPTV) channels in Block D and Block E are allowed up to 50 kW ERP. On the other hand, the terrestrial cellular network is allowed up to 1kW ERP per 1 MHz.

Figure 1: Region 2 Lower 700 MHz Spectrum and Band 12

Currently some operators have Block B and Block C in a significant portion of the geographic areas while some other operators have Block A in majority of the geographical areas, and Block B and Block C in some geographical areas. LTE with 5 and 10 MHz channel bandwidths would be deployed in Band 12.

There are many challenging interference issues in Band 12 [1-2][4-11], which are summarized in Section 2 for both the UE and the eNB. A detailed analysis of the UE interference issues in Band 12 are presented in Section 3.

# Overview of Issues on Region 2 (US) 700 MHz Spectrum

According to the past discussions in RAN4 [1], the main issues for UE are as follows:

Issue 1: The de-sense of UE due to Tx noise in Rx band leaked through duplexer

Issue 2: UE out-of-band emission into Channel 51 and Block D receivers

Issue 3: LPTV interference to Band 12.

On the other hand, the main issues for eNodeB are as follows:

Issue 4: The interference from HPTV and LPTV Transmitters to eNodeB Receiver

Issue 5: Interference from eNodeB to Block E Broadcast Rx

In this contribution, we focus on the analysis of interference issues at the UE terminal side, with the understanding that the interference issues 4&5 at the eNB side can be resolved by eNB implementation of receivers that are capable of interference cancellation or mitigation.

# Analysis of UE Interference issues

It is important to note that throughout the discussion and analysis in this section, we assume a UE duplexer that spans the whole band 12.

## Issue 1:De-sense of UE due to Tx noise

A Band 12 LTE PA output noise in the Rx band (with 30MHz offset) is typically at -123dBm/Hz. A Band 12 duplexer provides Tx  Rx isolation in the Rx band is typically 52dB. This yields additive noise from Tx to the Rx at the LNA input to be -175dBm/Hz. Meanwhile, we may assume a 5dB noise floor of the RX at the LNA input, which yields thermal noise = -174+5 dBm/Hz = -169dBm/Hz. Therefore, the De-sense contribution is roughly raising the Rx noise at LNA input by 1dB to -168dBm/Hz. This falls within the budget for Rx band noise. Total noise including Tx De-sense at LNA input = -168dBm/Hz + 66.5dBHz (4.5MHz effective bandwidth). This yields -101.5dBm Rx noise.

Using a 5MHz Channel Bandwidth signal for example, Refsense per TS 36.101requires -97dBm at antenna, which translate to -100dBm at the LNA input (assume a 3dB typical Front-End loss) for a single Rx.

Therefore, SNR achievable per Rx = 1.5dB, which is more than sufficient for QPSK-1/3 (Refsense condition) to provide good headroom for potential EVM degradation in the Rx downstream. Moreover, in 2x Rx MIMO condition, SNR should improve by at least another 3dB, and hence providing the Refsense margin commonly required from operators.  Furthermore, there is good chance that better than 5dB NF (an assumption in this calculation) is achievable for a receiver at LNA input.

In summary, we do not anticipate Tx noise De-sense to be a show stopper based on current information.

UE Tx  Rx De-sense Summary

| Parameters | @ UE LNA input | Units | Comments |
| --- | --- | --- | --- |
| Rx Band Noise @ PA out | -123 | dBm/Hz | Typical. Band 12 PA Module |
| Duplexer Isolation | 52 | dB | Typical. Band 12 SAW Duplexer |
| Rx Thermal Noise | -169 | dBm/Hz | Assume NF=5dB@ LNA input |
| Total Rx Noise | -101.5 | dBm | Use 4.5MHz effective bandwidth |
| Desired Signal Level | -100 | dBm | TS36.101 Refsense. Assume 3dB Front-end loss. |
| SNR | 1.5 | dB | Single Rx |
| 2x Rx MIMO effect | +3 | dB | At least 3dB improvement due to 2x MIMO |
| Effective SNR | 4.5 | dB |  |
| Margin | 4.5 | dB | SNIR = 0dB for QPSK-1/3 |

Summary: De-sense of UE due to Tx noise is manageable, and sufficient margin is achievable.

## Issue 2: UE out-of-band emission into Channel 51 and Block D receivers

Since the method of analysis is similar for Ch 51 and for block D, we focus on block D analysis in this section.

We consider two scenarios here, namely the out of band emission with and without 10dB MPR reduction at UE UL. Note that the signaling support for the MPR reduction has already been included in RAN4 spec 36.101.

Without MPR reduction:

ACLR of Band 12 PA in Block C into Block D LPTV Rx = +23dBm – 33dBc = -10dBm/5MHz. ACLR offset adjustment to center of Block D = -3dB (Block D Fcenter is 1MHz away from ACLR offset). We further assume LPTV in another UE is at approximately 5m distance, with an Antenna – Antenna isolation = 50dB. As a result, the total spurious emission at LPTV Block D antenna due to Block C UE UL for 5MHz bandwidth is equal to -10dBm -53dB = -63dBm.

Assuming LPTV receiver sensitivity at antenna = -85dBm for QPSK, and LPTV receiver NF at antenna = 10dB. Thermal noise of LPTV receiver at antenna is -97dBm/5MHz.

Therefore, without MPR reduction the resulting SNR = -19dB even with a 2x MIMO LPTV receiver, and therefore unable to support the required LPTV Rx sensitivity.

UE Tx ACLR Effects on Channel 51 & Block D Receivers Summary

| Parameters | @ UE Antenna | Units | Comments |
| --- | --- | --- | --- |
| Block C UL ACLR | -10 | dBm/5MHz | Max.out = 23dBm. PA specified ACLR = -33dBc |
| Offset Adjustment | -3 | dB | Block D Fcenter is 1MHz further out. |
| Antenna isolation | 50 | dB | 5m distance between UE & DTV antennae |
| LPTV Rx NF | 10 | dB |  |
| Thermal Noise LPTV Rx | -97 | dBm/5MHz |  |
| Total Noise | -63 | dBm | Includes UL ACLR and Thermal Noise |
| Desired Signal | -85 | dBm | LPTV Rx Refsense |
| SNR | -22 | dB | Single Rx |
| 2x Rx MIMO effect | +3 | dB | At least 3dB improvement due to 2x MIMO |
| Effective SNR | -19 | dB | Unable to support LPTV Refsense |
| Margin | -19 | dB | SNIR = 0dB for QPSK-1/3 |

With 10dB MPR reduction in Band 12 UL:

We set Band 12 UL 10dB below Pmax. New ACLR of Band 12 PA in Block C into Block D LPTV Rx = +13dBm – 48dBc = -35dBm/5MHz. Here the better ACLR of 48dBc (compared to 33dBc above) is due to the back-off  from the nonlinear region of the PA around maximum transmission power. Furthermore, assuming offset adjustment and antenna isolation to be identical yields, -88dBm/5MHz spurious in Block D, resulting in total noise in LPTV receiver at UE antenna = -87.5dBm/5MHz.

Noise level is able to support -85dBm sensitivity at LPTV antenna for Block D in QPSK, with a resulting SNR =2.5dB for single Rx antenna, and SNR =5.5dB for 2x MIMO diversity receiver.

Example Reduced UL power Operating Scenario Summary

| Parameters | @ UE Antenna | Units | Comments |
| --- | --- | --- | --- |
| Block C UL ACLR | -35 | dBm/5MHz | Max.out = 13dBm. PA specified ACLR = -48dBc |
| Offset Adjustment | -3 | dB | Block D Fcenter is 1MHz further out. |
| Antenna isolation | 50 | dB | 5m distance between UE & DTV antennae |
| LPTV Rx NF | 10 | dB |  |
| Thermal Noise LPTV Rx | -97 | dBm/5MHz |  |
| Total Noise | -87.5 | dBm/5MHz | Includes UL ACLR and Thermal Noise |
| Desired Signal | -85 | dBm | LPTV Rx Refsense |
| SNR | 2.5 | dB | Single Rx |
| 2x Rx MIMO effect | +3 | dB | At least 3dB improvement due to 2x MIMO |
| Effective SNR | 5.5 | dB | Unable to support LPTV Refsense |
| Margin | 5.5 | dB | SNIR = 0dB for QPSK-1/3 |

## Summary: Interference due to UE out of band emission can be addressed with an A-MPR table specifically designed for Band 12. The details of this table are FFS.

## Issue 3: LPTV interference to Band 12

Figure 2: MediFLO LPTV interference to Band 12

The LPTV interference into Band 12 is illustrated in Figure 2 above.  In this section, we present our analysis on this issue.  Clearly, the current design platform based on the existing 3GPP TS36.101 Standards will not be able to support a LPTV Block E interferer at -30dBm/6MHz into the Band 12 LTE UE downlink without any redesign.

However, the order of impact from the LPTV interferer can be significantly mitigated when the level is limited to -35dBm/6MHz or lower. It is determined to be sufficient to work for a current design platform with minimal optimization. The table below shows the analysis for such a scenario.

We also point the reader to Annex A for a set of field measurement results on  interference level at UE antenna, which seems to be in  the similar range: -28dBm/6MHz with 5dB antenna gain and -33dBm/6MHz without 5dB antenna gain, all at a distance of 1.4km.

| Parameters | @ UE Antenna |  |  | Units | Comments |
| --- | --- | --- | --- | --- | --- |
|  | Block AB | Block A | Block B |  |  |
| MediaFLO | E Block |  |  | - |  |
| T/R Duplexer Isolation | -58 |  |  | dB | Typ. Duplexer min. Spec = -55dB |
| Duplexer IL | 3 |  |  | dB |  |
| RFIC IIP3 | -10 |  |  | dBm | High Band: -13dBm, Low Band: >-10dBm |
| LTE Tx Power | 19 |  |  | dBm | @Antenna |
| LTE UE Case 3 Analysis |  |  |  |  |  |
| MediaFLO(MF) Interferer | -35 | -35 | -35 | dBm | Proposed level |
| Reference Sensitivity | -94 | -97 | -97 | dBm | TS36.101 Refsense Band 12 |
| LTE In-band Signal Level | -80 | -83 | -91 | dBm | Refsense + 6dB for In-band Blocking Test Refsense + 14dB for Adjacent Channel Test |
| IM3 Product | -96 | NA | -94 | dBm | 3rd order product in RX channel, MF+TX |
| XMOD Product | -93.5 | -93.5 | NA | dBm | Crossmod. product: MF+UE TX |
| MF ACLR Leakage | -103 | -104 | -116 | dBm | Assume that the MFLO transmitter uses the same spectrum mask as DTV (FCC regulation) |
| Total Noise | -91.2 | -93.1 | -93.9 | dBm | Sum of all noise and interference products |
| Desensitization | -2.8 | -3.9 | -3.1 | dB | Relative to Reference Sensitivity |

From our analysis above,  this existing design with -35dBm/6Mz LPTV interference is able to provide in the worse case 2.9dB margin above the required 0dB SNR for QPSK rate 1/3 transmission without including 2x MIMO gain.

Summary:  With our analysis, a -35dBm/6Mz LPTV interference will leads to an SNR value that is at least 2.9dB above the SNR requirement for QPSK rate 1/3 transmission.  A higher level LPTV interferer at -30dBm/6MHz for example will require more discussions in terms of Band 12 design.

# Suggested Spec Changes

Based on the above analysis and considering the need for providing some additional implementation margin, we suggest to change the in band blocking requirement in 36.101.

# Conclusion

In this contribution, we analyzed the UE interference issues in Band 12, assuming that the UE duplexer spans the whole band 12. The outcome of the analysis is summarized below:

- De-sense of UE due to Tx noise is manageable, and sufficient margin is achievable.

- Interference due to UE out of band emission can be addressed with an A-MPR table specifically designed for Band 12. The details of this table are FFS.

- With our analysis, a -35dBm/6Mz LPTV interference will achieve an SNR value that is 2.9dB above the required 0dB SNR for QPSK rate 1/3 transmission.  A higher level LPTV interferer at -30dBm/6MHz for example will require more discussions in terms of Band 12 design.

- Higher LPTV interferers are likely possible in future design improvements

We suggest RAN4 to take the above analysis into account when deciding on the need for any further specification change in 3GPP.

References

R4-102649, Band 12 Solution Approach by Guard Band Designation, Huawei,  June 2010

R4-101750, Band 12: A Discussion on the Issues and the Possible Solutions, Huawei, US Cellular, May 2010.

3GPP TR 25.822 V0.4.0 (2007-11), UMTS 700 MHz Work Item Technical Report.

R4-081108, TS36.101: Lower 700 MHz Band 15(Formerly known as Band 17), Motorola, April 2008.

R4-081109, TS36.101: TP for E-UTRA UE normal / additional channel bandwidth, Motorola, April 2008.

R4-081110, Impact of UE Self-interference on LTE FD-FDD operation (Band 12), Motorola, May 2008.

R4-081111, TS36.101: TP for 700 MHz spectrum emission mask, Motorola, April 2008.

R4-081356, On the introduction of Band 15, Ericsson, June 2008.

R4-081324, Performance and coexistence issues in the Lower 700 MHz band, AT&T, June 2008.

R4-082559, UE Receiver Performance Specifications for Band 17, AT&T, September 2008.

R5-100760, Band 12: Dual Duplexer Approach and Relaxation of Test Frequencies, Huawei, February 2010.

R5-100813, LS on Band 12: Possible Tx/Rx Performance Requirement Relaxation, R5 to R4, February 2010.

FCC Public Notice, RM No. 11592, Wireless Telecommunications Bureau Seeks Comment on Petition for Rulemaking Regarding 700 MHz Band Mobile Equipment Design and Procurement Practices, 18 February 2010.

3GPP TS 36.101 V8.7.0 (2009-09), User Equipment (UE) radio transmission and reception

3GPP TS 36.104 V8.7.0 (2009-09), Base Station (BS) radio transmission and reception

3GPP TS 36.508 V8.3.0 (2009-09), Common test environments for User Equipment (UE) conformance testing