3GPP TSG-RAN WG4 Meeting #78 NB-IOT AH	                        R4-78AH-0003

Kista, Stockholm, Sweden, 3 - 4 May, 2016

Agenda item:	4.5

Source: 	Mediatek inc.

Title: 	NB-IoT UE RF RX Requirements

Document for:	Approval

## Introduction

This contribution provides considerations and rationale for UE receiver REFSEN and OBB RF performance requirements. All of them have critical impacts on NB-IoT implementation, especially for cost, size and system performance.

## RX requirements

Base on [1], REFSENS spec. would start from a reasonable and realistic approach. It was agreed to trade-off with OBB requirement to see whether relaxing the RF requirement specification will help to benefit device cost, size and battery life.

Thanks to HD FDD operation; the front-end should be quite simple and similar to SAW-less application. Even with filter, the filter should be cost effective and low insertion loss. Although it benefits inband signal with low loss, the rejection of OBB would be smaller comparing with SAW RX architecture. So OBB spec. needs to be relaxed or NB-IoT UE would suffer more cost, no matter area or current, than LTE UE. It is not reasonable for such low cost IoT devices to be costlier than the cellular phone application; especially the objectives of IoT system are long battery life and lower cost.

Additional rationale for such relaxing RF requirements for NB-IoT is that, at least for such kind of quasi-static system, the proposed coverage enhancement feature could cover and compensate most of the high interference scenarios. For example even in presence of strong blocker – device still can function in the network while working at very low SNR conditions. There is no reason that the requirements for NB-IoT are more stringent than 2G and LTE systems.

In this document, base on REFSENS spec., the relaxed specs. for OBB are also proposed. The relaxed requirements take the cost into consideration and prevent the requirement to be more stringent than LTE system.

## REFSENS requirements

Base on [1], the formula for REFSENS is listed below.

PREFSENS (dBm)= -174dBm + 10log10B + NF + IM + Rb + SNR

    - -174dBm	noise floor at room temperature

    - B		operating bandwidth in Hz

    - NF		Noise figure in dB of the receiver in antenna connector

    - IM		Implementation and production margin

    - Rb		Band dependant relaxation factor

    - SNR		Required Signal to Noise Ratio in baseband

Observation 1: SAW-Less Rx paths are used to support 11 NB-IoT operating bands by removing band selection SAW filter. The Rx front-end insertion loss and complexity thus facilitate extended coverage range and lower cost.

Implication: NF = NFlegacy – 2 dB, due to SAW filter removal

Observation 2: Operating temperature range of IoT is wider than cell phone. AEC-Q100 defines -40 to 85 degree as Grade 3, which is mostly seen in M2M application.

Implication: IM = IMlegacy + 0.5 dB, due to meeting Grade 3 requirement for M2M application

Due to half duplex operation, the front-end design should be simpler than modern Cellular system. 7dB of NF, that is tighten from 9dB of legacy NF requirement, would be a practical assumption for mobile phone manufactures. 3dB is reserved for IM, that is relaxed from 2.5dB of legacy implementation margin, due to different design consideration. It includes the antenna switch for band and/ or TR switch. It includes the match loss, various kinds packages of different applications. It also includes the mass production margin of the devices and much wider temperature range request from M2M application. And it also includes Rb that is planned to reserve for band dependent variation. They are all design relative factors and would vary depending on the applications, supporting bands, operation regions and environments.

Observation 3: The residue timing and frequency offset or the drift of the frequency due to crystal oscillator imperfection would impact the signal estimation of receiver.  This was not taken into consideration in WF [1].

Implication = degrade 0.2 dB, due to frequency drifting of the clock.

According to proposed testing signals [1], required SNR is 0.8dB if everything is prefect but only noise is added. The result is already presented in [2]. In reality, the physical implement including all the details needs extra 1.5dB, which also reserved in legacy modern system. The only thing that lacks to take into consideration in WF [1] is the drift caused by timing and frequency offsets. It should also reserve 0.2 dB in practical implementation. The overall 2.5dB including non-ideal implementation margin proposed in [2] is a reasonable number for SNR. The overall REFSENS is around -109dBm. One thing needs to note; it is not fair to compare NB-IoT performance to LTE, since the applications, the objectives, and the requirements are quite different. If the spec. is not flexible to fulfil more applications, it would be limited in a very narrow region. It is why we propose leave some margins in IM to keep the flexibility to include as many as applications as possible. 7dB NF already shows the benefit that gain from HD FDD. It also means that the coverage would be maintain once the application really needs this supporting. But for the most of ones that don’t stay in this extreme condition, extremely good performance is not necessary, and even the variety of battery is limited.

Proposal #1:  Set -109dBm for REFSENS test, which is defined from 7dB of NF, 3dB of IM, and 2.5dB of SNR requirements.

## OBB requirements

Observation 4: OBB2 and OBB3 seen by SAW-less Rx LNA and mixer are increased by about 10-dB. It is not the case in LTE system.

Implication = Blocking level is higher than SAW RX. It is dealt with RF active circuitries, which is power hungry and not favoured by low power of NB-IoT.

Observation 5: The worst case operating scenarios with max interferer levels is statistically rare in the real field. It should have margin to compromise both low cost and coverage requirement using new techniques that is proposed by NB-IoT

Implication = In rare cases with max interferer levels, NB-IoT coverage enhancement feature by power boosting and repetition can boost SINR even when the wanted signal is increased in OBB testing case.

Since it is reasonable to adopt SAW-less design for ultra-low cost NB-IoT solution, OBB become quite serious problem due to lower rejection. So there are several contributions [3-4] indicating the need for relaxing the OBB requirements in NB-IoT compared to GSM and LTE specs. Thanks to SAW filter’s help, OBBs are always not the issue for LTE UE. The bottleneck is OBB2, since the filter can only provide around 8~10dB rejection for it. Once the SAW filter is removed, the result is completely different and OBB2 now is the boundary for RX design. To keep the same cost and current consumption for OBB2 and OBB3 in SAW-less application, the wanted signal need to be increased by 14dB rather than only 6dB from sensitivity level.

It is quite justified in NB-IoT since the worst operating scenarios with max blocking levels are statistically rare and thus it should allow some performance degradation while keeping lower cost and power consumption in favor of typical operating cases with expected no or low interferers. Furthermore, in NB-IoT, performance degradation in the worst OBB cases will be compensated by retransmissions. It only impacts very limited usage coverage since IoT device does not transmit all the time.

As for the benefit of the current saving, comparing to REFSENS+14dB, the evaluation results [2] show more than 30~40% current increasing in LO distortion network and VCO driving current if only REFSENS+6dB is accepted since it needs to deal with more blocking in the circuitries. It costs 2 years for a 10 year operation scenario. It is quite significant in the wearable devices or some small battery capacity applications.

Proposal #2:  Use the existing LTE OOB blocking requirements with maximum blocker level of -15dBm in Range 2, 3, and 4 (as specified for LTE in 3GPP TS 36.101) also for NB-IOT in all operating scenarios (stand-alone, in-band and guard-band).

Proposal #3: In all OBBs blocking cases mentioned above assume wanted signal 14dB above the REFSENS level.

## Summary

In this contribution we have made proposals for REFSENS and OBB specification requirements for NB-IoT UE receiver.

## References

- RAN4 #78, R4-162828. Sony. WF on REFSENS for NB-IoT

- RAN4 #78, R4-162610. MediaTek Inc., NB-IoT UE Rx RF Requirements

- R4-77AH-IoT-0136. Nokia Networks. WF for NB-IoT Receiver characteristics.

- RAN4 #77, R4-157323. Qualcomm. Multiband support considerations in NB-IoT type devices