3GPP TSG-RAN WG4 Meeting #78 NB-IOT AH	R4-78AH-0009

Stockholm, Sweden, 3 – 4 May 2016

Agenda Item:	5.4.7

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	Proposals on BS RX core requirements for NB-IoT

Document for:	Approval

# 1.	Introduction

The revised WI proposal to support Narrowband Internet of Things (NB-IoT) was approved at RAN#70 [1]. One objective of the WI is to specify the Base Station (BS) radio transmission and reception for NB-IoT, including the BS emission mask.

The updated way forward documents on BS receiver (RX) requirements were approved at RAN4#78Bis [2, 3], where several topics are for FFS. In this paper, we provide our proposals for the way forward on these topics, in order to complete the Rel-13 WI as targeted.

# 2.	Discussion

# 2.1	Reference sensitivity level

The FRC parameters for this requirement are FFS in [3]. This requirement is the minimum mean power received at the antenna connector at which a throughput requirement shall be met for a specified reference measurement channel. Traditionally this requirement has been specified for low SNR wanted signals. However, there is a testing aspect to take into account, as it may be difficult to generate accurate power levels well below -100 dBm. Since the reference sensitivity level is re-used in many other RX tests, the measurement should be made as accurate and simple as it could be. Therefore, it will probably be advantageous to use a channel with a higher SNR requirement in this requirement. The channels with lower SNR requirements can be specified for the performance requirements.

On the other band, how to address the band edge issue for guard-band or stand-alone operation is FFS in [3]. Here we propose to consider case-by-case for each operating band identified for NB-IoT operation, following the usual practice for UE REFSEN requirement.

# 2.2	Dynamic range

The FRC and SINR for this requirement are FFS in [2]. This requirement is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal inside the received channel bandwidth. Hence, we propose to adopt a FRC that would results in a higher SINR for successful demodulation, which would represent a more challenging case for the BS design.

On the other band, if was discussed in RAN4#78Bis on what implementation margin (IM) should be adopted for this requirements. Here we propose to reuse 2.5 dB IM which was adopted for E-UTRA for this requirement (refer to Table 7.3-1 in [4]).

# 2.3	In-channel selectivity

The need for this requirement is FFS in [2]. This requirement is specified as a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of an interfering signal received at a larger power spectral density. As the current E-UTRA requirement is tested with an interfering signal around 20 dB higher than the wanted signal, and the current power boosting for NB-IoT PRB is specified to be 6dB, we see no need for this requirement for in-band or guard-band operation.

On the other hand, stand-alone operation has only one NB-IoT PRB, hence it would not be straightforward to specify this requirement for stand-alone operation.

# 2.4	ACS/Blocking

For stand-alone operation, the wider bandwidth interfering signal in TS36.104 [5], the receiver desensitization level, and the need for narrow-band blocking are FFS. These requirements are a measure of the receiver ability to receive a wanted signal at its assigned channel in the presence of an unwanted interferer. For the wider bandwidth interfering signal in TS36.104, we propose to adopt the more general channel bandwidth among all operating bands (i.e. 5 MHz E-UTRA) interfering signal as 1.4 MHz E-UTRA channel bandwidth is not specified in some of the NB-IoT operating bands (e.g. Band 1). On the other hand, we propose to consider using NB-IoT interfering signal if this is considered by operators as a more likely scenario in actual deployment. For the receiver desensitization level, we propose to allow higher desensitization level at least for the ACS requirement, following the current approach in TS 36.104 where narrower bandwidth option are allowed higher desensitization level (refer to Table 7.5.1-3 of TS 36.104) Also the ACS is not flat in practice, and the wanted signal is closer to the interfering signal and hence will see a higher average interference PSD than the corresponding E-UTRA wanted signal. For the need for narrow-band blocking, if (narrow-band) NB-IoT interfering signal is adopted for the ACS/Blocking requirement, we see no need for the narrow-band blocking requirement from requirement coverage viewpoint.

For in-band or guard-band operation, the receiver desensitization level is FFS. Again we propose to allow higher desensitization level at least for the ACS requirement, following the current approach in TS 36.104 where narrower bandwidth option are allowed higher desensitization level (refer to Table 7.5.1-3 of TS 36.104).

# 2.5	Receiver intermodulation

The requirement is not included in the agreed way forward in [2]. This requirement is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.

For stand-alone operation, we propose to use a CW signal and a 5 MHz E-UTRA signal (the more general channel bandwidth among all operating bands) as the interfering signals, and a NB-IoT interfering signal can be used instead of a 5 MHz E-UTRA signal if NB-IoT interfering signal is adopted for the ACS/Blocking requirement.

For in-band or guard-band operation, we propose to reuse the current interfering signals as specified in Table 7.8.1-2 of TS 36.104. Note that for guard-band operation, the two interfering signals should be shifted to ensure the intermodulation products will fall within the NB-IoT PRB receive frequency range.

# 3.	Conclusion

In this contribution, we provide our proposals for the way forward on BS RX requirements for NB-IoT. Our proposals are summarized below.

For Reference sensitivity level

- to use a channel with a higher SNR requirement in this requirement, while the channels with lower SNR requirements can be specified for the performance requirements;

- to consider case-by-case for each operating band identified for NB-IoT operation.

For Dynamic range

- to adopt a FRC that would results in a higher SINR for successful demodulation;

- to reuse 2.5 dB IM.

For In-channel selectivity

- not to specify the In-channel selectivity requirement for NB-IoT operation.

For ACS/Blocking

- to adopt the more general channel bandwidth among all operating bands (i.e. 5 MHz E-UTRA) interfering signal, or consider using NB-IoT interfering signal;

- to allow higher desensitization level at least for the ACS requirement;

- not to specify the narrow-band blocking requirement if (narrow-band) NB-IoT interfering signal is adopted for the ACS/Blocking requirement.

For Receiver intermodulation

- For stand-alone operation, we propose to use a CW signal and a 5 MHz E-UTRA signal as the interfering signals, and a NB-IoT interfering signal can be used instead of a 5 MHz E-UTRA signal if NB-IoT interfering signal is adopted for the ACS/Blocking requirement.

- For in-band or guard-band operation, we propose to reuse the current interfering signals as specified in Table 7.8.1-2 of TS 36.104. Note that for guard-band operation, the two interfering signals should be shifted to ensure the intermodulation products will fall within the NB-IoT PRB receive frequency range.

# References

[1]	RP-152284, “Revised Work Item: Narrowband IoT (NB-IoT)”, Huawei, HiSilicon.

[2]	R4-162820, 	“Way forward on BS RX requirements for NB-IoT”, Huawei, Ericsson, CMCC.

[3]	R4-162831, 	“Way forward on BS REFSENS for NB-IoT”, Ericsson, Huawei, Docomo, Nokia.

[4]	R4-082475, “TR 36.804 v1.3.0”, Ericsson

[5]	3GPP TS 36.104 v13.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.