3GPP TSG-RAN WG4 Meeting #78 NB-IOT AH	R4-78AH-0113

Kista, Stockholm, Sweden, 3 – 4 May 2016

Agenda Item:	5.5.4&5.5.5

Source: 	CMCC

Title: 	ACS and blocking for standalone NB-IoT

Document for:	Approval

- Introduction

RAN4 has agreed a WF on BS Rx requirement for NB-IoT [1]. In this contribution, we discuss the open issues for ACS and blocking requirements based on the following agreements.

| Wanted signal | Interfering signal | ACS/Interfering signal level | frequency offset between RF bandwidth edge and interfering signal center frequency | Wanted signal desensitivity | Note |
| --- | --- | --- | --- | --- | --- |
| Standalone NB-IoT in 36.104 | NB-IoT | 45 dB | 100kHz | FFS dB | ACS |
|  | Option 1: 5MHz LTE | -43 dBm | +/-7.5MHz | FFS dB | In-band blocking |
|  | Option 2: 1.4 MHz LTE | -43 dBm | +/- 2.1 MHz |  |  |
| Standalone NB-IoT in 37.104 | NB-IoT | 45 dB | 100kHz | FFS dB | ACS |
|  | 5M UTRA | -40 dBm | +/- 7.5 MHz | FFS dB | In-band blocking |

  - For standalone in-band blocking there are two options, based on further study one option will be adopted.

  - The need of Narrow-band blocking is FFS

- Discussion

2.1 in-band blocking

For in-band blocking two options with the same interfering level but different interfering signal bandwidth were proposed. As NB-IoT is a narrow band system, its receiver rejection characteristics is much more sensitive to frequency separation. 1.4MHz interfering signal whose frequency offset is much smaller than 5MHz signal will cause more interference to NB-IoT. In addition, 1.4MHz is also a practical deployment scenario. To guarantee good system performance, it is proposed to use option 2 for in-band blocking requirement.

Proposal#1: use 1.4MHz LTE as interfering signal for standalone NB-IoT in-band blocking requirements.

2.2 Narrowband blocking

E-UTRA narrowband blocking is a measure of the receiver ability to receive a wanted signal at its assigned channel frequency in the presence of an adjacent channel 1RB LTE signal. Compared to ACS requirement, narrowband blocking is to verify the receiver rejection on higher power density and narrowband interfering signal. However, the agreed ACS requirement for NB-IoT has already covered these requirements. Hence, we propose not to specify narrowband blocking requirement for standalone NB-IoT.

Proposal#2: do not specify narrowband blocking requirement for standalone NB-IoT.

2.3 Wanted signal desensitivity

For E-UTRA narrowband blocking test, the wanted signal mean power is PREFSENS + 6dB; and for E-UTRA ACS test, the wanted signal mean power depends on the channel bandwidth. However, if we look at the MSR specification in TS 37.014 or GSM specification in TS45.005, the wanted signal mean power for GSM/EDGE is PREFSENS + 3dB. In addition, based on the simulation conclusion, NB-IoT loss is smaller than 3dB with appropriate ACS. It seems 3dB is more appropriate to narrow band system, so wanted signal desensitivity for standalone NB-IoT is proposed to be 3dB.

Proposal#3: the want signal desensitivity for standalone NB-IoT  is proposed to be 3dB.

- Conclusion

Proposal#1: use 1.4MHz LTE as interfering signal for standalone NB-IoT in-band blocking requirements.

Proposal#2: do not specify narrowband blocking requirement for standalone NB-IoT.

Proposal#3: the want signal desensitivity for standalone NB-IoT  is proposed to be 3dB.

- Reference

[1] R4-162820, Way forward on BS RX requirements for NB-IoT, Huawei, Ericsson, CMCC