3GPP RAN WG4 Meeting #78-NB-IoT	R4-78AH-0114

Kista, Sweden, 3 – 4 May 2016

Source: 	Sony

Title: 	WF on Spurious Response for NB IOT

Agenda Item:	4.5.6

Document for:	Approval

# Introduction

Spurious response requirement is complementary to out of band blocking requirement as it allows some blocking relaxation at the limited number of frequencies for which the out of band blocking requirement is not met.

# Discussion

## Current E-UTRA approach

The current E-UTRA approach (TS 36.101, section 7.7.1) is applying the relaxed Range1 OOB blocking requirements at the spurious response frequencies in Range 2, 3 and Range 4 (Range 4 might no be relevant to NB-IOT). For example for channel BW=1.4MHz:

- Psignal = REFSENS+6dB

- Pinterferer(CW) = -44dBm

The number of spurious response frequencies is defined as a number of exceptions in OOB test and applied for each assigned frequency channel when measured using a 1MHz step size (TS 36.101, section 7.6.2.1):

- in frequency range 1, 2 and 3, up to exceptions are allowed

- in frequency range 4, up to exceptions are allowed

## Spurious response requirement for NB-IOT

We propose to apply the legacy E-UTRA spurious response requirements also for NB-IOT, i.e.:

Table 7.7.1-X: Spurious response parameters for NB-IOT

| Parameter | Unit | Level |
| --- | --- | --- |
| Psignal | dBm | REFSENS+6 |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |
| Number of spurious response frequencies |  | 24 (in OOB range 2, 3) |

# Conclusion

Proposal: 	Apply the legacy E-UTRA spurious response requirements also for NB-IOT

Table 7.7.1-X: Spurious response parameters for NB-IOT

| Parameter | Unit | Level |
| --- | --- | --- |
| Psignal | dBm | REFSENS+6 |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |
| Number of spurious response frequencies |  | 24 (in OOB range 2, 3) |

# References

R4-78AH-0115		“WF on Out of Band Blocking for NB-IoT”, Sony