3GPP RAN WG4 Meeting #78-NB-IoT	R4-78AH-0117

Kista, Sweden, 3 –  4 May, 2016

Source: 	Sony

Title: 	NB IoT UL Transmission Gaps

Agenda Item:	4.4.3

Document for:	Approval

# Introduction

An LS was received from RAN1 on uplink transmission gaps in NB-IoT [1]. RAN1 has discussed frequency drift of HD-FDD NB-IoT UE under a long continuous uplink transmission, considering that the UE may use a low cost oscillator.  RAN1 agreed to provide uplink compensation gaps for such a transmission in order to provide opportunities for the UE to correct its frequency offset using the downlink signals [2]. The LS contained the following question:

RAN1 respectfully ask RAN4 the following:

- Assuming that a single set of values would be specified, please advice on one value for the maximum uplink transmission duration and one value for the gap length considering in-band operation under extreme coverage.

A late submission on this topic was provided during RAN4 #78 bis [3].

## Agreement in RAN1

In order to allow for NB-IoT UEs (which are HD-FDD in nature) to maintain frequency synchronisation with the network, in the RAN1 AdHoc2 in Sophia Antipolis, it was agreed to insert transmission gaps during long UL transmissions, as follows:

•       Introduce uplink transmission gaps for long uplink (i.e. NB-PUSCH/NB-PRACH) transmissions.

–      During uplink transmission gaps, the UE may switch to the DL and performs time/frequency synchronization

–      Uplink transmission gap is defined by a period X and a gap length Y.

•       A minimum period Xmin is defined in the specifications

•       FFS whether other periods X > Xmin are defined

•       Y > 0

•       All uplink transmissions of duration greater than or equal to X msec applies transmission gap with gap length Y and periodicity X until the uplink transmission completes

–      The values of the period X and gap length Y are:

•       Alt-1: Defined in the specifications

## Outline

This document considers the following aspects related to UL transmission gaps:

- Transmission period, X

- Transmission gap duration, Y

# Discussion

## Transmission Period, X

In order to not impact the decoding performance of the eNodeB, the UL transmission period (X) should be short enough such that the UE can maintain its frequency error within ±0.1ppm. In [4], it was agreed the frequency error should be within ±0.1ppm for normal coverage mode. It is preferred to have the same specification for extended coverage mode as for normal coverage mode.

Proposal 1: In order to have the same specification for extended coverage mode as for normal coverage mode the frequency error should be kept within ±0.1ppm.

The transmission period, X, will include an initial frequency error which will change due to frequency drift.

To determine the maximum UL transmission period, X, the drift of the output frequency of the UE’s crystal oscillator, measured in Hz / sec need to be estimated. The drift of output frequency depends significantly on the temperature change of the UE during transmission and this is the only considered effect in this document. The temperature change is mainly due to self heating where the PA is the main heat generator.

For a low cost crystal oscillator, a typical characteristic for the change in oscillator frequency is:

∆ff=at-t0+bt-t02+ct-t03 [ppm]

Where t = actual temperature

t0 = 26°C

a ≈ -0.3

b ≈ ±0.0005

c ≈ 0.00001

Assuming that 50% of the ±0.1ppm frequency tolerance can be assigned to initial frequency tracking error and 50% can be assigned to frequency drift, the maximum allowed change in frequency due to frequency drift is ±0.05ppm.

Considering a device with a weight of 0.5g and a heat capacity of 1J/K/g (~plastic and glass), for a dissipation power during transmit of 300mW:

- Temperature gradient ≈ 0.6°C/s

- Frequency drift f/f ≈ -0.18 ppm/s

- Max Frequency Error Due to Frequency drift within ±0.05ppm

Hence the time to reach the maximum frequency offset is approximately 0.05 / 0.18 = 280ms.

Therefore as a very rough estimate, allowing for some margin, we could say X (transmission length) ≤ 200ms.

In [5], it was proposed to evaluate the uplink demodulation performance at the eNB with certain UE frequency error model and identify the performance degradation for certain uplink transmission period. We consider the uplink transmission period should be limited by the hardware characteristics (e.g. oscillator) so that the UE frequency error can be kept within ±0.1ppm. Simulation study is not required to determine the uplink transmission period.

Observation 1: The uplink transmission period should be limited by the hardware characteristics (e.g. oscillator) so that the UE frequency error can be kept within ±0.1ppm. Simulation study is not required to determine the uplink transmission period.

Proposal 2: In order to keep frequency error within ±0.1ppm, the transmission period, X, should be approximately 200ms.

## Transmission Gap Duration, Y

The required duration of the transmission gap, Y, could be estimated based on either the time to perform initial synchronisation (based on the NB-PSS / NB-SSS signals and receiver synchronisation algorithms), or the time required for the UE frequency tracking algorithms (based on reference signals) to obtain adequate frequency tracking.

The transmission period, X, has been analysed in the above section with a view to keeping the UE’s frequency error within ±0.1ppm. At such a frequency error, the frequency tracking algorithm is capable of correcting the frequency tracking error and it is not necessary to perform initial synchronisation during the transmission gaps.

Hence we estimate the transmission gap duration, Y, based on the time required by the frequency tracking algorithm. This estimate is based on a UE implementation that makes use of the maximum number of known signals for the purposes of frequency tracking: NB-RS, NB-PSS, NB-SSS and NB-PBCH.

#### Resource Mapping in NB-IoT

The mapping of NB-PBCH, NB-PSS and NB-SSS to subframes is shown in Figure 1 below. NB-PBCH is always located in subframe 0 in each radio frame. NB-PSS is always located in subframe 5 and NB-SSS is located in subframe 9 [3]. There is no agreement yet on the NB-SSS periodicity. There were some proposals on NB-SSS with 10ms, 20ms, and 40ms periodicity. In this case, we assume the NB-SSS is located in each odd radio frame (20ms periodicity).

Figure 1: NB-PBCH, NB-PSS, and NB-SSS sub-frame mapping.

The resource element mapping of NB-RS, NB-PBCH, NB-PSS, and NB-SSS in a sub-frame is shown in Figure 2.

Figure 2: NB-PBCH, NB-PSS, and NB-SSS resource element mapping

#### Frequency Tracking

When the UE performs frequency offset estimation and correction (e.g. in connected mode), it is assumed that the UE has performed the initial synchronization stage and successfully decoded NB-PBCH. A UE can regenerate the NB-PSS, NB-SSS, and NB-PBCH and re-use these signals for frequency tracking purposes. This brings the advantage that the transmitted NB-PSS, NB-SSS, and NB-PBCH can be used as reference symbols to assist frequency offset estimation. If the regenerated symbols (NB-PSS, NB-SSS, and NB-PBCH) are denoted as and the received symbols are denoted as  then the least square estimate of the channel is formed as . The frequency offset estimation can be based on the correlation of the channel estimate in OFDM symbol l, with the channel estimate at the same sub-carrier in OFDM symbol n + l. The distance of NB-RS from the same antenna port is 7 OFDM symbols. In this estimate, we consider frequency offset estimation based on OFDM symbols at a distance equal to 7. Thus, the correlation outputs obtained from NB-PBCH, NB-PSS, NB-SSS, and NB-RS can be jointly combined. If the OFDM symbols distance equals to 7 then the number of resource element pairs that can be used for frequency estimation in each NB-PBCH, NB-PSS, NB-SSS sub-frame is 44 pairs. To improve the estimation, the accumulation of correlation outputs across subframes can be performed.

An investigation of the performance of frequency tracking for the eMTC case is found in [7]. The performance evaluation is based on the theoretical mean square error (MSE) of frequency estimation in an AWGN channel and no imperfections are assumed. The details of the MSE derivation can be found in [8]. Here, we use the same approach for the NB-IoT case. We consider an eNB that has 2 Tx antennas and a UE with 1 Rx antenna. The evaluation result in terms of RMS frequency error versus SNR is shown in Figure 3.

Figure 3: Frequency offset estimation performance using NB-PBCH, NB-PSS, NB-SSS across subframes

It can be observed that the joint usage of NB-PBCH, NB-PSS, NB-SSS, and NB-RS can significantly improve the frequency offset estimation performance. Furthermore, performing frequency offset estimation with a measurement gap length of 20 subframes (i.e. 20ms) can yield an RMS frequency estimation error of around 80 Hz (which is approximately 0.1 ppm at an 800MHz carrier frequency) at the SNR operating point of a DL-power-boosted in-band NB-IoT carrier. In practice, we should also consider the UE operation in a fading channel.  A 5 dB margin is allocated to consider the operation in fading channel. Considering this margin, the total transmission gap duration should be approximately 60 ms.

Proposal 3: The UL transmission gap duration should be approximately [60] ms.

# Conclusion

In this contribution we have considered aspects of UL transmission gaps for NB-IoT and make the following observation and proposals.

Proposal 1: In order to have the same specification for extended coverage mode as for normal coverage mode the frequency error should be kept within ±0.1ppm.

Observation 1: The uplink transmission period should be limited by the hardware characteristics (e.g. oscillator) so that the UE frequency error can be kept within ±0.1ppm. Simulation study is not required to determine the uplink transmission period.

Proposal 2: In order to keep frequency error within ±0.1ppm, the transmission period, X, should be approximately 200ms.

Proposal 3: The UL transmission gap duration should be approximately [60]ms.

# References

- R1-162069, 		“LS on uplink transmission gap in NB-IoT”

- R1-163379, 		“Final Report of 3GPP TSG RAN WG1 #2AH_NB-IoT” , MCC

- R4-162809,		“NB IoT UL Transmission Gaps and Draft Response LS to RAN1”, Sony

- R4-162066		“Frequency error for NB-IoT”, NTT DOCOMO, INC.

- R4-162874 		“Way Forward on continuous uplink transmission in NB-IoT”, Intel Corporation

- R1-163378,		“Final Report of 3GPP TSG RAN WG1 #84”,  MCC

- R1-153085, 		“Tracking of Frequency Offset for MTC”, Sony

- Wang Q, et.al.,”Carrier Frequency Synchronization in the Downlink of 3GPP LTE”, IEEE PIMRC Conf, 2010.