3GPP TSG RAN WG4 Meeting #78AH	R4-78AH-0155

Kista, Sweden May 3 – 4, 2016

Agenda Item:	6.8

Source: 	Ericsson

Title:	Analysis of Uplink Transmit Timing in NB-IoT

Document for:	Discussion

# Introduction

- UE transmit timing requirements:

  - RAN4 is to study the UE transmit timing for NB-IOT due to 180 KHz bandwidth.

A way forward on further studies on RRM aspects for NB-IoT [1] was agreed at RAN4#78. One of the agreed studies concerns UE transmit timing requirements:

  - Onset of a new UL repetition period shall be considered to be an initial transmission.

  - UE autonomous UL timing updates are not allowed during UL repetition periods.

At the RAN4#78bis we presented an analysis [2] and the group agreed on a way forward [4] for the RAN4#78AH. The way forward contains two working assumptions:

- Analyses on the feasibility of the working assumptions (1-2).

- Analyses on the time needed to detect a change in downlink timing given the use of DL repetitions.

- Analyses on feasibility of specifying timing error requirements based on a 1.92MHz timing resolution, although the UE implementation may use a lower sample rate.

- Analyses on whether relaxations compared to timing error requirements for legacy UE in 1.4MHz cell are needed.

Based on those working assumptions companies were invited to provide the following analyses:

In this contribution we provide an revised version of [2].

# Background

NB-IoT downlink

The NB-IoT downlink has been specified to be the equivalent of the LTE downlink but over a smaller bandwidth. Particularly, the NB-IoT downlink bandwidth corresponds to 1 PRB (180kHz) and the system bandwidth to 200 kHz, to compare with the minimum LTE bandwidth of 6 PRBs (1080kHz) and system bandwidth 1.4 MHz.

One consequence of adopting the LTE downlink is that when a normal cyclic prefix is used the minimum sampling rate at which an inter-symbol distance can be represented by a whole number of samples is 1.92MS/s, whereby the normal cyclic prefix is 10 samples (5.2µs) in the first OFDM symbol in a slot, and 9 samples (4.7µs) in the remaining ones. At this sampling rate the sample period is 0.52µs. For NB-IoT, having a lower bandwidth, it would be attractive to use a sample rate of say 240kS/s whereby a 16pt instead of a 128pt FFT can be used in the UE receiver. As 240kS/s is a factor 8 lower than 1.92MS/s, it would mean that the sample period would be 4.2µs, and the normal cyclic prefix would be 1.25 and 1.125 samples in length for first and remaining OFDM symbols in a slot, respectively; see Figure 1.

Figure 1: Cyclic prefix sizes at sampling rate 240kS/s.

In case of equidistant sampling, e.g. to allow synchronization towards the serving cell while carrying out intra-frequency neighbour cell search and measurements, the lower sampling rate in combination with normal cyclic prefix results in that there will be a gradual phase shift of subsequent OFDM symbols in a slot due to the fractional sample length of the cyclic prefix; see Table 1. Since this phase change is systematic it can be compensated for in the channel estimation. In order to avoid inter-symbol interference it is essential that the start of a subframe is well aligned with the start of a sample period, something that can be achieved e.g. by adjusting the sampling phase of the ADC. Hence, using a sampling frequency of 240kS/s is feasible from downlink demodulation point of view.

Table 1: Exemplary start positions for sampling OFDM symbols at 240kS/s, and resulting offsets to actual start of OFDM symbol.

|  |  | Normal cyclic prefix |  | Extended cyclic prefix |  |
| --- | --- | --- | --- | --- | --- |
| Symbol |  | Symbol start [sample] | Offset [samples] | Symbol start [sample] | Offset [samples] |
| slot 0 | 0 | 1 | -0.25 | 2 | -2 |
|  | 1 | 18 | -0.375 | 22 | -2 |
|  | 2 | 35 | -0.5 | 42 | -2 |
|  | 3 | 52 | -0.625 | 62 | -2 |
|  | 4 | 69 | -0.75 | 82 | -2 |
|  | 5 | 86 | -0.875 | 102 | -2 |
|  | 6 | 103 | -1 |  |  |
| slot 1 | 0 | 121 | -0.25 | 122 | -2 |
|  | 1 | 138 | -0.375 | 142 | -2 |
|  | 2 | 155 | -0.5 | 162 | -2 |
|  | 3 | 172 | -0.625 | 182 | -2 |
|  | 4 | 189 | -0.75 | 202 | -2 |
|  | 5 | 206 | -0.875 | 222 | -2 |
|  | 6 | 223 | -1 |  |  |

Legacy time tracking algorithms are generally based on finding the sample position that maximizes the correlation to a prior known signal, e.g. SSS or CRS. As a consequence, when using 1.92MS/s, it is possible in terms of resolution to come as close as to within ±0.26µs (half a sample period) from the actual timing. When the received downlink path changes the detected downlink timing will be changed in a granularity of 0.52µs (whole samples).

If applying the legacy time tracking algorithm on NB-IoT with 240kS/s, the consequence would be that the UE can come as close as to within ±2.1µs from the actual timing, and the granularity of the detected change in downlink timing would be 4.2µs.

NB-IoT uplink

The NB-IoT uplink has been specified in two configurations with respect to subcarrier spacing: 15 kHz and 3.75 kHz, respectively. The first configuration, 15 kHz subcarrier spacing, reuses the same OFDM/SC-FDMA symbol boundaries as in LTE, meaning that the normal cyclic prefix is 5.2µs and 4.7µs for first and following symbols in a slot, respectively.

The second configuration, 3.75 kHz subcarrier spacing, is defined over a 2ms NB-slot which carries 7 symbols. The duration of a normal cyclic prefix is 8.33µs and a symbol is 275µs.

Figure 2: Uplink slot structure for 3.75 kHz subcarrier option with normal cyclic prefix.

Uplink timing requirements in LTE

Existing uplink transmit timing requirements in [3] cover two aspects:

- UE autonomous timing changes due to changed downlink timing ([3] clause 7.1)

- UE application of timing advance commands transmitted by the eNodeB ([3] clause 7.3)

Regarding the UE autonomous change of uplink transmit timing, a distinction is made between an initial transmission, comprising the first transmission in DRX On time as well as PRACH transmission, and subsequent transmissions, comprising the remaining scenarios.

For an initial transmission, the uplink transmit timing shall be within ±Te seconds from the ideal uplink timing, which for an LTE cell with smallest bandwidth corresponds to ±0.8µs. For subsequent transmissions, in case the UE detects a change in downlink timing, it shall modify the uplink transmit timing under the following side conditions:

- maximum timing adjustment in one correction shall not exceed 17.5Ts (0.6 µs),

- minimum aggregated adjustment rate shall be 7Ts (0.2µs) per second, and

- maximum aggregated adjustment rate shall be 17.5Ts (0.6 µs) per 200ms.

Hence a change in downlink timing by one sample requires about 200ms to accommodate.

Coverage enhancement

In order to facilitate enhanced coverage, i.e., operation in coverage that is worse than assumed in legacy LTE, repetitions on downlink and uplink have been specified by RAN1. A message is repeated a number of times, thus allowing the UE or the eNodeB to accumulate the received signal until the message can be decoded. The repetition periods may extend to hundreds or even thousands subframes. By accumulating the message before decoding it or decoding each received message and using soft combining, the message can be retrieved although buried in noise.

Half-duplex operation

NB-IoT UEs have been specified to operate in half-duplex mode only, meaning that during the time there is downlink reception there is no uplink transmission, and vice versa. This means that while transmitting, the UE does not get any information on changes in downlink timing or receives timing advance commands.

# Considerations on uplink transmit timing requirements

The following aspects need to be taken into account regarding the uplink transmit timing in a NB-IoT device:

- Transmit timing tolerance at initial transmissions

- Rate of UE-autonomous change of uplink transmit timing in subsequent transmissions

- Handling of coverage enhancement

Initial transmit timing tolerance

The two uplink subcarrier configurations (15 and 3.75 kHz) have different sensitivity with respect to transmit timing errors since for the 15 kHz option the normal cyclic prefix is 5.2 or 4.7µs whereas for the 3.75 kHz option it is larger, 8.33µs. Hence a larger offset from the ideal transmit timing can be accepted for the 3.75 kHz without leading to inter-symbol interference and associated leakage to adjacent subcarriers i.e. inter-carrier interference.

For the 15kHz option and particularly in in-band and guard band deployment scenarios it is likely that the whole uplink spectrum will be processed by a single FFT by which leakage caused by timing errors on the uplink not only will affect the performance of the NB-IoT cell but may also degrade the performance of the LTE cell. It is therefore anticipated that an initial tolerance similar to that for an LTE cell with smallest bandwidth, i.e.  ±0.8µs, would be needed.

Proposal 1: For the 15 kHz uplink subcarrier spacing option the initial transmit timing tolerance shall be similar to the LTE requirement for a cell with 6RBs bandwidth.

For the 3.75kHz option the initial transmit timing tolerance potentially can be somewhat relaxed since the cyclic prefix is larger, and since it is less likely that this option would be used in an in-band or guard-band deployment scenario where the eNodeB uses a single FFT for the whole uplink bandwidth. If used in an in-band or guard-band deployment it is more likely that the NB-IoT bandwidth will be suppressed via filtering before processing the LTE uplink. However, the view from other infrastructure vendors is needed before RAN4 can decide on the necessary tolerance.

Proposal 2: For the 3.75 kHz uplink subcarrier spacing option infrastructure vendors first need to discuss in which scenarios, e.g. whether in in-band and guard-band deployment, this option will be used, before the required tolerance can be determined.

The required initial uplink transmit timing accuracy may be challenging to meet when using a sampling rate of 240kS/s on the downlink and using legacy time tracking algorithms, since the resolution of the detected downlink path may be too coarse. Particularly, the sample period is 4.2µs at 240kS/s. Hence UE vendors need to investigate whether other than legacy time tracking algorithms can be used, that would allow tracking of the downlink timing in fractions of a sample, e.g., in 1/8th of a sample, thus facilitating reduced sample rate 240kS/s on the downlink while maintaining a granularity in the downlink timing tracking corresponding to 1.92MS/s.

Proposal 3: In order to facilitate reduced sampling rate on the downlink while maintaining a sufficient resolution of the uplink transmit timing, UE vendors shall look in to time tracking algorithms that can track the downlink timing in fractions of a sample.

Rate of UE-autonomous change of uplink transmit timing in subsequent transmissions

The rate at which the UE is allowed to change the uplink transmit timing autonomously has been determined with the possibility for the eNodeB to compensate changes with timing advance should there not be reciprocity between the uplink and downlink propagation paths, e.g., that the downlink propagation time increases while the uplink propagation time remains constant. There are however three problems associated with the LTE legacy behavior if applied to NB-IoT.

- Firstly, NB-IoT operates in half-duplex mode and if configured with repetitions in order to facilitate coverage enhancement, the UE cannot receive during the uplink transmission repetition period. Thus the eNodeB anyway cannot send timing advance commands to the UE;

- Secondly, the mere fact that repetitions have to be used implies that the eNodeB may have to receive multiple repetitions of the message and the reference signals before it can assess the timing correction to command to the UE;

- Thirdly, changing the timing gradually during the repetition period, according to the legacy rules, will degrade the accumulated message since gradual timing changes will introduce gradually changing linear phase on the receiver side, by which different instances of the message may grate each other.

Moreover, if adopting the LTE legacy UE behavior, and if the UE operates at 240kS/s on the downlink using the LTE legacy time tracking algorithms (whole sample positions only), a change in downlink timing will be in steps of 4.2µs which would, again according to LTE legacy rules, take 1.5s to accommodate. This again stresses the need for time tracking algorithms that can maintain a timing resolution corresponding to 1.92MS/s regardless of the actual downlink sample rate in use, whereby very abrupt uplink transmit timing changes can be avoided.

For the reasons provided above it seems not feasible to allow UE autonomous timing corrections during repetition periods. Rather, the onset of transmission in each repetition period would better be considered an initial transmission, i.e., be handled similar to a first transmission in DRX on time in LTE legacy, and UE autonomous uplink timing changes be avoided in subsequent transmissions during the repetition period.

Proposal 4: When uplink repetitions are configured for which the number of repetitions exceeds 1, the onset of the repetition period shall be considered an initial transmission, and no UE autonomous timing corrections are to be applied during the subsequent transmissions of the same message during the remainder of the repetition period.

For the case when no uplink repetitions have been configured, or repetitions have been configured but the repetition number is 1 (e.g. dynamically configured for NB-PUSCH via DCI), it may be feasible to adopt the LTE legacy behavior with respect to the rate at which the UE changes the uplink transmit timing. In this case, there may be a tighter interaction between the UE and the eNodeB and additionally the latter does not need to accumulate received messages in order to be able to decode it successfully. Moreover channel estimation and the alike can be carried out on individual subframes or NB slots, whereby the eNodeB can adapt to minor changes in the uplink transmit timing.

Proposal 5: When no uplink repetitions are configured, or the number of repetitions is 1, the LTE legacy behavior on rate of UE autonomous timing corrections shall be used as baseline.

# Summary and Conclusions

We have analyzed the uplink transmit timing requirements in NB-IoT using the LTE legacy UE behavior as baseline, and propose the following:

Proposal 1: For the 15 kHz uplink subcarrier spacing option the initial transmit timing tolerance shall be similar to the LTE requirement for a cell with 6RBs bandwidth.

Proposal 2: For the 3.75 kHz uplink subcarrier spacing option infrastructure vendors first need to discuss in which scenarios, e.g. whether in in-band and guard-band deployment, this option will be used, before the required tolerance can be determined.

Proposal 3: In order to facilitate reduced sampling rate on the downlink while maintaining a sufficient resolution of the uplink transmit timing, UE vendors shall look in to time tracking algorithms that can track the downlink timing in fractions of a sample.

Proposal 4: When uplink repetitions are configured for which the number of repetitions exceeds 1, the onset of the repetition period shall be considered an initial transmission, and no UE autonomous timing corrections are to be applied during the subsequent transmissions of the same message during the remainder of the repetition period.

Proposal 5: When no uplink repetitions are configured, or the number of repetitions is 1, the LTE legacy behavior on rate of UE autonomous timing corrections shall be used as baseline.

A draft CR for [3] regarding UE autonomous uplink timing adjustments is provided in [5].

# References

- R4-161294 “Way forward capturing the proposals on further studies in RAN4 for NB-IOT”, Ericsson, Nokia Networks, Alcatel-Lucent, Huawei, HiSilicon, Intel Corporation, ZTE, Qualcomm Incorporated

- R4-161945 “Analysis of Uplink Transmit Timing in NB-IoT”, Ericsson

- 3GPP TS 36.133

- R4-162866 “Way Forward on UE Transmit Timing”,  Ericsson, Nokia

- R4-78AH-0159 “pCR Uplink transmit timing adjustments in NB-IoT“, Ericsson