3GPP TSG RAN WG4 Meeting #79AH	R4-79AH-0079

Hong Kong, China, 28 - 30 June, 2016

Agenda Item:	2.3.9

Source: 	Nokia

Title: 	NB-IoT UE RLM Test Cases

Document for:	Discussion

# Introduction

NB-IoT UE RLM requirements were agreed in RAN4#79 [1].  In this paper, we discuss the method, configurations and parameter settings for the NB-IoT UE RLM test cases.

# Discussion

Test Purpose

The purpose of the RLM tests is to verify that the NB-IoT UE properly detects the in-sync and out-of-sync for the purpose of monitoring downlink radio link quality of the serving cell when DRX is or is not used. This test will partly verify the NB-IoT radio link monitoring requirements in clause 7.23 [1].

Test Method

For legacy RLM tests, UE CQI reporting is used. For NB-IoT, however, this approach is not feasible, since NB-IoT UE does not support CQI reporting, and other measurement reporting. Thus, we need first find some other indicator of UE Out-of-sync/In-sync status.

Test Environment

Next we will discuss the test configurations.

- NPDCCH related parameters

  - DCI format

    - Format N1

  - Aggregation level (AL)

    - AL = 2

  - Maximum NPDCCH Repetition level (RL)

    - Out-of-sync: Rmax

    - In-sync: Rmax/4

    - From UE behaviour point of view it is clear that it should monitor the radio link quality according to the configured Rmax, but from test point of view we cannot define test case for each possible configuration of Rmax, so the issue is to select the most typical configurations for the test case development. Our preference is to use the largest possible Rmax as agreed in the NPDCCH simulation assumptions for normal and enhanced coverage [4], i.e. ? for normal coverage and ? for enhanced coverage.

- Time period T1-T5

  - The lengths of time period T1-T5 are depending on the evaluation time. For non-DRX tests the durations need to be discussed due to new evaluation time requirements for normal and enhanced coverage, respectively. For DRX tests the existing durations can be re-used as the evaluation time requirements are same as for Cat-M1 CEModeA.

  - T1 is set as 5 times Out-of-sync evaluation time in Cat-M1 CEModeA test cases. We think the same approach can be re-used for NB-IoT, so the value of 2s should be used.

  - T2, during T2 UE should not trigger RLF at SNR2, and the duration is twice the Out-of-sync evaluation time in Cat-0 test cases. We think the same setting can be used for Cat-M1 also, so T2 should be 0.8s for Mode A and 4s for Mode B.

  - T3, is depending on the setting T310, and as we see no need to change T310 (2s used in Cat-0 test cases), we think the same value of 1.46s can be re-used for Cat-M1 tests.

  - T4, during T4 UE should not trigger In-sync. As T310 is already running UE should follow non-DRX requirement for the monitoring of In-sync, and in Cat-0 tests T4 is set as 4 times In-sync evaluation period. Therefore, we think the value of 0.8s can be used for Cat-M1 tests.

  - T5, T5 should be long enough to cover the verification point, but so far we do not have clear view for the verification point, and we are open to proposals.

- PCFICH/PDCCH/PHICH RMC

  - PCFICH and PHICH RMC are not needed as NB-IoT UE is not decoding them.

- NPDSCH

  - New NPDSCH RMC should be developed.

- OCNG

  - New OCNG patterns should be developed.

- SNR1-SNR5

  - SNR levels used in the Cat-M1 tests are derived as below, and we think the same approach can be used.

    - SNR2 = Qout + margin1

    - SNR3 = Qout – margin1

    - SNR4 = Qin – margin2

    - SNR5 = Qin + margin2

    - SNR1 = SNR5

  - The values for Qin and Qout should come out as averaged simulation results for NPDCCH performance with selected coverage level or levels. In addition, the values for the margins should be determined for both normal coverage and enhanced coverage.

- Propagation condition

  - The same channel model, i.e., ETU1, as used in demodulation performance may be used for RLM test cases,

# Conclusions

In this paper we discussed the method, configuration and parameter settings for NB-IoT RLM tests. Unlike legacy NB-IoT UEs, where CQI reporting can be configured for UE RLM tests, NB-IoT UEs do not support CQI and other measurement reporting. Therefore, it needs further study on how to conduct RLM tests for NB-IoT UEs.

Proposal 1: NB-IoT RLM test cases may be developed based on existing test cases for eMTC UEs in CEModeA.

Proposal 2: Since NB-IoT UEs do not support CQI and other UE measurement reporting, further study is needed on how to conduct NB-IoT RLM tests without the use of UE measurement reporting.

# References

- R4-164901	CR for Radio Link Monitoring for NB-IoT	QUALCOMM UK Ltd

- R4-164639	CR: Cat-M1 PRACH test cases for HD-FDD in CEModeA	Nokia

- R4-164891	HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEModeA	Qualcomm Incorporated

- R4-164902	HD-FDD Radio Link Monitoring Test for In-sync for Cat-M1 UE in CEModeA	Qualcomm Incorporated

- 3GPP TS 36.211

- 3GPP TS 36.213