3GPP TSG-RAN WG4 Meeting AH-1801	R4-1800022

San Diego, California, USA, 22 – 26 January 2018

Agenda Item:	4.4.1.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	TP to TR 38.817-02: NR BS in-band and out-of-band boundaries for FR1 (5.9)

Document for:	Approval

# 1.	Introduction

The way forward on in-band and out-of-band boundaries for FR1 was agreed in RAN4#84bis [1]. The agreements have not been recorded in the NR BS TR 38.817-02 [2], although some of them have been implemented in the NR BS TS 38.104 [3].

This contribution provides a text proposal to record the agreements, which cover both conducted and radiated transmitter and receiver characteristics, into a new sub-clause in the General clause in TR 38.817-02 [2] for future reference.

# 2.	Text Proposal

<New clause>

## 5.9	In-band and out-of-band boundaries for FR1

Agreements of the boundary between OBUE and spurious emissions are summarized in below tables 5.9-1 and 5.9-2. The boundary is valid regardless of whether it is single-RAT case or MSR case, and NR new band or refarming band (except Band 46). If wider band(s) than current maximum bandwidth of band is specified, the values of the boundary should be discussed. (Wider than 200MHz for E-UTRA, wider than 900MHz for NR).

# Table 5.9-1: ΔfUEM (NR or E-UTRA single-RAT case)

|  |  | E-UTRA |  | NR |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 36.104/141 (non-AAS type) | 37.105/145-1/145-2 (AAS type) | 38.104/141 (non-AAS type(1-C)) | 38.104/141 (AAS type(1-H/1-O)) |
| ΔfUEM | Operating band <100MHz | 10MHz | 10MHz | 10MHz | 10MHz |
|  | 100MHz≦Operating band ≦200MHz | 10MHz | 40MHz | 10MHz | 40MHz |
|  | 200MHz<Operating band ≦900MHz |  |  | 40MHz | 40MHz |
|  | 900MHz<Operating band |  |  |  |  |

# Table 5.9-2: ΔfUEM (NR and E-UTRA MSR case)

|  |  | E-UTRA |  | NR |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 37.104/141 (non-AAS type) | 37.105/145-1/145-2 (AAS type) | 37.104/141 (non-AAS type(1-C)) | 37.105/145-1/145-2 (AAS type(1-H/1-O)) |
| ΔfUEM | Operating band <100MHz | 10MHz | 10MHz | 10MHz | 10MHz |
|  | 100MHz≦Operating band ≦200MHz | 10MHz | 40MHz | 10MHz | 40MHz |
|  | 200MHz<Operating band ≦900MHz |  |  | 40MHz | 40MHz |
|  | 900MHz<Operating band |  |  |  |  |

Agreements of the in-band / out-of-band boundary on receiver characteristics are summarized in below tables 5.9-3 and 5.9-4. The boundary is valid regardless of whether it is single-RAT case or MSR case, and NR new band or refarming band (except Band 46). If wider band(s) than current maximum bandwidth of band is specified, the values of the boundary should be discussed. (Wider than 200MHz for E-UTRA, wider than 900MHz for NR).

# Table 5.9-3: ΔfOOB (NR or E-UTRA single-RAT case)

|  |  | E-UTRA |  | NR |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 36.104/141 (non-AAS type) | 37.105/145-1/145-2 (AAS type) | 38.104/141 (non-AAS type(1-C)) | 38.104/141 (AAS type(1-H/1-O)) |
| ΔfOOB | Operating band <100MHz | 20MHz | 20MHz | 20MHz | 20MHz |
|  | 100MHz≦Operating band ≦200MHz | 20MHz | 60MHz | 20MHz | 60MHz |
|  | 200MHz<Operating band ≦900MHz |  |  | 60MHz | 60MHz |
|  | 900MHz<Operating band |  |  |  |  |

# Table 5.9-4: ΔfOOB (NR and E-UTRA MSR case)

|  |  | E-UTRA |  | NR |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 37.104/141 (non-AAS type) | 37.105/145-1/145-2 (AAS type) | 37.104/141 (non-AAS type(1-C)) | 37.105/145-1/145-2 (AAS type(1-H/1-O)) |
| ΔfOOB | Operating band <100MHz | 20MHz | 20MHz | 20MHz | 20MHz |
|  | 100MHz≦Operating band ≦200MHz | 20MHz | 60MHz | 20MHz | 60MHz |
|  | 200MHz<Operating band ≦900MHz |  |  | 60MHz | 60MHz |
|  | 900MHz<Operating band |  |  |  |  |

<End of change>

# References

[1]	R4-1711779, “WF on in-band and out-of-band boundary”, NTT DOCOMO, INC., Nokia, Ericsson, Huawei, ZTE.

[2]	R4-1714547, “TR 38.817-02 v0.5.0: General aspects for BS RF for NR”, Ericsson.

[3]	3GPP TR 38.104 v15.0.0, “NR; Base Station (BS) radio transmission and reception”.