3GPP TSG-RAN4 Meeting Adhoc#1801 	R4-1800050

# San Diego, USA, 22th– 26th January, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on TS38.133 Maximum allowed layers for multiple monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-1-8 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current specification 38.133, if a UE is configured with EN-DC, the UE shall be capable of monitoring at least [7] NR inter-frequency carriers configured by PSCell and at least [7] NR inter-RAT carriers configured by E-UTRA PCell. And the UE shall be capable of monitoring total of at least [7] effective NR carriers. It means that the NR frequency layers configured by E-UTRA PCell and PSCell shall be exact the same. However, the NR inter-frequency layers configured by PSCell and NR inter-RAT carriers configured by E-UTRA PCell can be configured independent and different. Thus more than [7] total effective NR carriers is more reasonable, and from network perspective, it is more flexible. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the number of total effective NR carrier from [7] to [8]. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is too restrictive for network to configure the NR frequency layers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 9.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

9.1.3	UE Measurement capability

9.1.3.1	Monitoring of multiple layers using gaps for NSA

The requirements in this section are applicable for UE capable of E-UTRA-NR dual connectivity operation with E-UTRA PCell.

When monitoring of multiple inter-frequency E-UTRAN and inter-RAT NR carriers as configured by E-UTRA PCell, and inter-frequency NR carriers as configured by PSCell carriers using gaps (or without using gaps provided the UE supports such capability) is configured, the UE shall be capable of performing one measurement of the configured measurement type (SS-RSRP, SS-RSRQ, SS-SINR, E-UTRAN RSRP, E-UTRAN RSRQ, E-UTRAN RS-SINR measurements, etc.) of detected cells on all the layers.

For UE configured with the E-UTRA-NR dual connectivity operation, the effective total number of frequencies excluding the frequencies of the PSCell, SCells, E-UTRA PCell, and E-UTRA SCells being monitored is Nfreq, NSA, which is defined as:

Nfreq, NSA = Nfreq, NR, NSA + Nfreq, E-UTRA,

where

Nfreq, E-UTRA is the number of E-UTRA inter-frequency carriers being monitored (FDD and TDD) as configured by E-UTRA PCell.

Nfreq_NSA_NR ≤ Nfreq_NSA_NR_inter-RAT + Nfreq_NSA_NR_inter-freq

where

Nfreq_NSA_NR_inter-RAT is the number of NR inter-RAT carriers being monitored as configured by E-UTRA PCell [15]

Nfreq_NSA_NR_inter-freq is the number of NR inter-frequency carriers being monitored as configured by PSCell

9.1.3.2	Maximum allowed layers for multiple monitoring

If a UE is configured with E-UTRA-NR dual connectivity operation, the UE shall be capable of monitoring at least:

-	Depending on UE capability, [7] NR inter-frequency carriers configured by PScell, and

-	Depending on UE capability, [7] NR inter-RAT carriers configured by E-UTRA PCell [15], and

-	Depending on UE capability, [6] E-UTRA TDD inter-frequency carriers configured by E-UTRA PCell [15], and

-    Depending on UE capability, [6] E-UTRA FDD inter-frequency carriers configured by E-UTRA PCell [15],

In addition to the requirements defined above, the UE shall be capable of monitoring a total of at least [13] effective carrier frequency layers comprising of any above defined combination of NR, E-UTRA FDD and E-UTRA TDD layers. The UE shall be capable of monitoring a total of at least [8] effective NR carrier frequency layers configured by E-UTRA PCell and/or PScell.

Note: The E-UTRA-NR dual connectivity capable UE configured with PSCell shall fulfil the requirements defined in only one of Section 9.1.3.2 and Section 8.2.1.1b.1 of [15].