3GPP TSG-RAN4 Meeting Adhoc#1801 	R4-1800053

# San Diego, USA, 22th– 26th January, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on MRTD for EN-DC operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-1-8 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The MTTD and MRTD requirements for EN-DC operation are completed in RAN4#85 meeting, however, there are some TBD values which need to be determined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the TBD to the proposed value for MTTD and MRTD requirements Delete MRTD requirement for intra-band E-UTRA-NR dual connectivity with collocated deployment |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MTTD and MRTD requirements are not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 7.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

7.5	Maximum Transmission Timing Difference

7.5.1	Introduction

A UE shall be capable of handling a relative transmission timing difference between subframe timing boundary of E-UTRA PCell and slot timing boundaries of PSCell to be aggregated E-UTRA-NR dual connectivity.

7.5.2	Minimum Requirements for E-UTRA-NR Dual Connectivity

For inter-band E-UTRA-NR dual connectivity, the UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1.

Table 7.5.2-1 Maximum uplink transmission timing difference requirement for asynchronous operation

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 500 |
| 15 | 30 | 250 |
| 15 | 60 | 125 |
| 15 | 120Note1 | 62.5 |
| Note1 : For intra-band FDD-FDD E-UTRA-NR dual connectivity, 120kHz is not applied. |  |  |

For inter-band TDD-TDD (E-UTRA PCell-PSCell) and inter-band TDD-FDD (E-UTRA PCell-PSCell or PSCell-E-UTRA PCell) combinations, the UE shall meet the requirements in Table 7.5.2-1 provided that the UE indicates that it is capable of asynchronous dual connectivity [16].

For inter-band TDD-TDD (E-UTRA PCell-PSCell) and inter-band TDD-FDD (E-UTRA PCell-PSCell or PSCell-E-UTRA PCell) combinations, the UE shall meet the requirements in Table 7.5.2-2 provided that the UE indicates that it is capable of synchronous dual connectivity only [16].

Table 7.5.2-2 Maximum uplink transmission timing difference requirement for synchronous operation in inter-band TDD-TDD and TDD-FDD combinations

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | [34.82] |
| 15 | 30 | [34.66] |
| 15 | 60 | [34.76] |
| 15 | 120 | [34.54] |

For intra-band FDD-FDD E-UTRA-NR dual connectivity with collocated deployment, the UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1 provided the UE indicates that it is capable of asynchronous dual connectivity [TS 38.331].

For intra-band TDD-TDD E-UTRA-NR dual connectivity with collocated deployment, only synchronous and collocated operation is allowed, thus no uplink transmission timing difference is applicable.

7.6   Maximum Receive Timing Difference

7.6.1	Introduction

A UE shall be capable of handling a relative receive timing difference between subframe timing boundary of E-UTRA PCell and slot timing boundaries of PSCell to be aggregated for E-UTRA-NR dual connectivity.

A UE shall be capable of handling a relative receive timing difference between slot timing boundary of different carriers to be aggregated NR carrier aggregation.

7.6.2	Minimum Requirements for E-UTRA-NR Dual Connectivity

For inter-band E-UTRA-NR dual connectivity, the UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell at the UE receiver as shown in Table 7.6.2-1 below.

Table 7.6.2-1 Maximum receive timing difference requirement for asynchronous operation

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz)Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 500 |
| 15 | 30 | 250 |
| 15 | 60 | 125 |
| 15 | 120 | 62.5 |
| Note1 : DL Sub-carrier spacing is min{SCSSS, SCSDATA}. Note2 : For intra-band FDD-FDD E-UTRA-NR dual connectivity, 120kHz is not applied. |  |  |

For inter-band TDD-TDD (E-UTRA PCell-PSCell) and inter-band TDD-FDD (E-UTRA PCell-PSCell or PSCell-E-UTRA PCell) combinations, the UE shall meet the requirements in Table 7.6.2-1 provided that the UE indicates that it is capable of asynchronous dual connectivity [16].

For inter-band TDD-TDD (E-UTRA PCell-PSCell) and inter-band TDD-FDD (E-UTRA PCell-PSCell or PSCell- E-UTRA PCell) combinations, the UE shall meet the requirements in Table 7.6.2-2 provided that the UE indicates that it is capable of synchronous dual connectivity only [16].

Table 7.6.2-2 Maximum receive timing difference requirement for synchronous operation in inter-band TDD-TDD and TDD-FDD combinations

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz)Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | [33] |
| 15 | 30 |  |
| 15 | 60 |  |
| 15 | 120 |  |
| Note1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. |  |  |

For intra-band FDD-FDD E-UTRA-NR dual connectivity with collocated deployment, the UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell as shown in Table 7.6.2-1 provided the UE indicates that it is capable of asynchronous dual connectivity [16].