3GPP TSG-RAN WG4 Meeting #AH1801	R4-1800056

San Diego, US, 22nd – 26th Jan 2018

Source: 	Nokia

Title: 					DC_(n)71B MSD

Agenda Item: 			4.1.2.2

Document for:	Approval

# 1	Introduction

In previous RAN4 meeting DC_(n)71B MSD values were tentatively agreed and captured into TS 38.101-3 in brackets. Specification was approved by RAN78. In previous RAN4 meeting we provided MSD results [1] which were not fully accurate due to error in simulator. In this contribution we provide new MSD results for those cases which were selected as MSD specification for DC_(n)71B.

# 2	Discussion

Tentative agreement for DC_(n)71B MSD captured into TS 38.101-3 is presented in Table 1.

Table 1: Reference sensitivity (MSD) for intra-band DC bandwidth class

| MSD / DC bandwidth class B |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation [LCRB] | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)71B | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | [0] | FDD |
|  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | [1.8] |  |
| DC_(n)71B | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | [0] |  |
|  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | [1.6] |  |
| DC_(n)71B | 71 | 668 | 10 | 10 (RBend = 49) | 622 | [0] |  |
|  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | [1.7] |  |

Our simulation results for the case when NR signal is OFDM is presented in Table 2

Table2: MSD with OFDM agressor

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode / OFDM |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC configuration | EUTRA and NR band | UL fc (MHz) | UL/DL BW (MHz) | UL LCRB (RB) | DL fc (MHz) | MSD (dB) | Duplex mode |
| DC_(n)71B | 71 | 665,5 | 5 | 5 (RBend=24) | 619,5 | 0 | FDD |
|  | n71 | 675,5 | 15 | 15 (RBstart=0) | 629,5 | 0,4 |  |
|  | 71 | 670,5 | 15 | 15 (RBend=74) | 624,5 | 0 |  |
|  | n71 | 680,5 | 5 | 5 (RBstart=0) | 634,5 | 0,4 |  |
|  | 71 | 668 | 10 | 10 (RBend=49) | 622 | 0 |  |
|  | n71 | 678 | 10 | 10 (RBstart=0) | 632 | 0,4 |  |

Our simulation results for the case when NR signal is DFT-s-OFDM is presented in Table 3

Table 3: MSD with DFT-s-OFDM agressor

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode / DFT-s-OFDM |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC configuration | EUTRA and NR band | UL fc (MHz) | UL/DL BW (MHz) | UL LCRB (RB) | DL fc (MHz) | MSD (dB) | Duplex mode |
| DC_(n)71B | 71 | 665,5 | 5 | 5 (RBend=24) | 619,5 | 0 | FDD |
|  | n71 | 675,5 | 15 | 15 (RBstart=0) | 629,5 | 0,3 |  |
|  | 71 | 670,5 | 15 | 15 (RBend=74) | 624,5 | 0 |  |
|  | n71 | 680,5 | 5 | 5 (RBstart=0) | 634,5 | 0,3 |  |
|  | 71 | 668 | 10 | 10 (RBend=49) | 622 | 0 |  |
|  | n71 | 678 | 10 | 10 (RBstart=0) | 632 | 0,4 |  |

Our results indicate that the MSD specification for DC_(n)71B is correct and sufficient thus we propose that

Proposal: Remove brackets from DC_(n)71B MSD requirement.

# 3	Conclusion

In this contribution we have provided new results for DC_(n)71B MSD requirement and made following proposal.

Proposal: Remove brackets from DC_(n)71B MSD requirement.

# 4	References

[1] R4-1712449, Simulation results for MSD of DC_71b+n71b, Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc, RAN4#85