3GPP TSG-RAN WG4 Meeting #AH1801	R4-1800057

San Diego, US, 22nd – 26th Jan 2018

Source: 	Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc.

Title: 					Sub-6 MPR WF

Agenda Item: 			4.3.2.3.1

Document for:	Approval

# 1	Introduction

In this contribution we have a way forward proposal for sub-6 MPR.

# 2	Discussion

In RAN4 #84bis tentative MPR values were agreed in [1]. RAN4#85 did not confirm these values but agreed three MPR values for inner FDT-s-OFDM modulations. Furthermore contribution [3] proposed modification to 256-QAM proposal in [1] as it was based on results from one company only.

Building on these three contributions we propose Sub-6 MPR as presented in Table 1.

- MPR values with white background are based on [1]

- MPR values with green background are agreed [2]

- MPR value with yellow background are based on [3]

Table 1: WF for Sub-6 GHz MPR

|  |  | Sub-6 MPR [dB] |  |
| --- | --- | --- | --- |
| WF type | Modulation | Outer allocation | Inner allocation |
| DFT-S-OFDM | pi/2-BPSK | 0 | 0 |
|  | QPSK | 1 | 0 |
|  | 16-QAM | 1.5 | 0.5 |
|  | 64-QAM | 2 |  |
|  | 256-QAM | 4.5 |  |
| CP-OFDM | QPSK | 3 | 0.5 |
|  | 16-QAM | 3 | 1.5 |
|  | 64-QAM | 3 |  |
|  | 256-QAM | 6 |  |

# 3	Conclusion

In this contribution we have made WF proposal for Sub-6 MPR and it is presented on Table 1.

# 4	References

[1] R4-1711559, WF on MPR for Sub-6 NR, Qualcomm Incorporated, Nokia, RAN4 #84bis

[2] R4-1714047, WF on MPR for sub6GHz, NTT DOCOMO, INC., RAN4 #85

[3] R4-1712446, NR 256-QAM MPR, Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc., RAN4 #84bis