3GPP TSG-RAN WG4 Meeting #AH1801	R4-1800066

San Diego, US, 22nd – 26th Jan 2018

Source: 	Nokia, Nokia Shanghai Bell

Title: 					UTRA A-MPR

Agenda Item: 			4.3.2.9

Document for:	Approval

# 1				Discussion

In RAN4#85 we presented a study of necessary A-MPR for FR1 UE to be able to meet the UTRAACLR requirement. Need for A-MPR was simulated against MPR presented in Table 1.

Table 1: tentative MPR used in UTRA A-MPR study [1]

| Modulation | MPR (dB) |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM PI/2 BPSK | ≤ 0 | ≤ 0 |
| DFT-s-OFDM QPSK | ≤ 1 | ≤ 0 |
| DFT-s-OFDM 16 QAM | ≤ 1.5 | ≤ 0.5 |
| DFT-s-OFDM 64 QAM | ≤ 2 |  |
| DFT-s-OFDM 256 QAM | ≤ 4 |  |
| CP-OFDM QPSK | ≤ 3 | ≤ 0.5 |
| CP-OFDM 16 QAM | ≤ 3 | ≤ 1.5 |
| CP-OFDM 64 QAM | ≤ 3 |  |
| CP-OFDM 256 QAM | ≤ 6 |  |

Studies presented in [1] showed that no A-MPR is necessary to be able to meet the UTRAACLR requirement.

In this meeting we have made proposal for final NR FR1 MPR [2], this almost identical for what was used in [1] except in one case the MPR is more in [2] thus we can make following proposal.

Proposal: No A-MPR is specified for UE to be able to meet the UTRAACLR requirement.

# 2	References

[1] R4-1712445, NR A-MPR for UTRA, Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc., RAN4#85

[2] Sub-6 MPR WF, Nokia, Nokia Shanghai Bell, RAN4 NR AH1801