3GPP TSG-RAN WG4 AH1801 Meeting                      	 R4-1800114

San Diego, CA, USA, January 21st – 26th, 2018

Agenda Item:	4.6.5.1

Source:	MediaTek Inc.

Title:	Discussion on cell detection requirement for NR

Document for:		Discussion

- Introduction

At the RAN4 #85 meeting, the newest agreement for intra-frequency PSS/SSS detection TPSS/SSS_sync requirement is as follow [1].

- For UE configured with PSCell only (no LTE or NR SCells), PSS/SSS detection requirement: • FR1 : TPSS/SSS_sync =max([600], [5 or 6]*SMTC_periodicity), • FR2 : TPSS/SSS_sync =max([], []*SMTC_periodicity)

In this contribution, we will continue to discuss the PSS/SSS detection delay requirement (TPSS/SSS_sync) in intra-frequency and also give the simulation results in FR2.

- Intra-frequency PSS/SSS detection simulation results in FR2

In RAN4 NR #84bits meeting, [2] summarized each company’s simulation results in the following table. Now, we also provide our simulation results of PSS/SSS detection latency in FR2. Considering not too many companies submitted the PSS/SSS detection simulation results in FR2, we expect more companies could submit PSS/SSS detection results in FR2 before determining the final requirement.

The detailed simulation parameters and assumptions are given in Appendix.

Table 1: The 90%-ile PSS/SSS acquisition time from contributions

(Carrier frequency = 30 GHz, SINR = -6dB)

|  |  | R4-1709441 [3] | R4-1709622 [4] | R4-1709735 [5] | Mediatek |
| --- | --- | --- | --- | --- | --- |
| SCS 120kHz | AWGN | 1 | 1 | 1 | 1 |
|  | TDL-A | - | 3 | - | 3 |
|  | TDL-B | - | 3 | - | 2 |
|  | TDL-C | - | 2 | - | 2 |
|  | TDL-D | - | 2 | - | 1 |
|  | CDL-A | 3 | - | 2 |  |
| SCS 240kHz | AWGN | 1 | 1 | 1 | 1 |
|  | TDL-A | - | 3 | - | 4 |
|  | TDL-B | - | 4 | - | 4 |
|  | TDL-C | - | 3 | - | 2 |
|  | TDL-D | - | 3 | - | 1 |
|  | CDL-A | 3 | - | 2 |  |
| Cell ID Combination w/ or w/o beamforming |  | Case 4 w/o RX BF | Case 2 w/o RX BF | Case 2 w/ RX BF | Case 2 w/o RX BF |

Based on simulation results regarding PSS/SSS detection in 30 GHz carrier frequency case shown in Table 1, we can make following observations.

Observation 1: At least 4 attempts is required for PSS/SSS detection in FR2 without considering additional delay due to Rx beam sweeping for targeting 90% detection probability.

- Intra-frequency PSS/SSS detection requirement without measurement gaps

  - Intra-frequency PSS/SSS detection requirement when no DRX is used

In last meeting, RAN4 had made an agreement that the performance requirement structure for intra-frequency PSS/SSS detection is TPSS/SSS_sync =max([600]ms, [5 or 6]*SMTC_periodi), but did not make the final decision on the number of SMTC periodicities.

For small SMTC periodicities (such as SMTC periodicity = 10ms, 20ms, 40ms), 600 is always larger than [5 or 6]*SMTC_period, so the requirement is irrelevant to the down selection between 5 and 6. Nevertheless, for large SMTC periodicities (such as SMTC periodicity = 80ms, 160ms), it’s possible to expect more challenging in AGC tuning because it may not directly apply the AGC setting from last SMTC occasion to the next occasion. At the same time, we did not consider enough RF loss and UE real fixed implementation in the simulation. Therefore, it is proposed to use one more SMTC periodicity for larger.

Proposal 1: The intra-frequency PSS/SSS detection requirement in FR1 is max(600ms, 6*SMTC_period) when no DRX is configured.

  - Intra-frequency PSS/SSS detection requirement when DRX is used

DRX operation within RRC_Connected state has been defined with the expectation that it will allow a UE to take advantage of inactivity periods for power saving while the network remains in control of UE mobility. In legacy LTE, it is proposed that requirements should be split into ‘Short DRX cycle and ‘Long DRX cycle [6].

Short DRX cycles

The ‘Short DRX’ range can be defined as the range for which the performance requirement is the same as non-DRX RRC_Connected case. This applies because the length of those periods is expected to be no larger than the periodicity of RRM-related activity.

Long DRX cycles

The implementation of DRX in RRC_Connected state only really makes sense if the UE is allowed to have a somewhat lower rate of activity than non-DRX RRC_Connected state. Otherwise, despite a decrease in data Rx/Tx activity the power saving benefit would be rather limited if UE still needs to perform Intra-RAT and Inter-RAT monitoring as frequent as non-DRX.

Midpoint

In legacy LTE, it was proposed to define the cut-off between short and large DRX as 40ms [6] for which normal LTE-mobility requirement are applicable.

In NR, we can use the same idea to define the performance requirement in DRX scenarios. It is desirable to follow same principle in measurement requirement for long DRX periods and keep non-DRX requirements when DRX period roughly equals RRM-related activity.

Proposal 2: The midpoint between long and short DRX in detection requirement should be 40ms.

Considering the trade-off between UE power saving and mobility performance when large DRX is in use, it’s better to re-use the LTE requirement in NR (the blue elements in Table 2, where TSSB denotes the SMTC periodicity), although we know that NR is fundamentally more power consuming in general, e.g., more cell IDs, additional dimension in beam-level, higher sampling rate and the mis-alignment between SMTC and DRX On-duration.

Different to LTE, NR SMTC periodicities is configurable, which could be {5ms, 10ms, 20ms, 40ms, 80ms, 160ms}. Currently, the same requirement is shared by non-DRX and the condition ‘DRX cycle < SMTC periodicity’ in the agreed intra-frequency TP [7].

| DRX cycle | TPSS/SSS_sync |
| --- | --- |
| No DRX, or DRX cycle < SMTC periodicity | TBD |
| …. | TBD |

In other word, the same requirement is applied to all the combinations of DRX cycle and TSSB marked as yellow in Table 2. But for DRX larger than 40ms, it could be found that performing PSS/SSS detection would be rather expensive in terms of power consumption compared to LTE. E.g., if assuming DRX cycle = 80ms, SMTC = 80ms, the requirement is 0.6s for PSS/SSS detection, but it is 2.8s in LTE. Since there is no issue found in DRX mobility with LTE requirements, to give UE additional flexibility for power saving, we suggest to revise the requirement as the Table 2.

Proposal 3: It should re-use the LTE requirement in NR for power saving under large DRX scenarios.

For another group scenarios, when both TSSB and DRX cycle are short but DRX cycle is equal to or larger than TSSB (the green elements in Table 2), it’s reasonable to follow the non-DRX requirement. These combinations are also missing in current TP [7].

Table 2. The requirement table for DRX cycle and SMTC combinations

| DRX cycle (ms) | SMTC periodicity (ms) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 20 | 40 | 80 | 160 |
| 10 | max{600, 6xTSSB} | max{600, 6xTSSB} | max{600, 6xTSSB} | max{600, 6xTSSB} | max{600, 6xTSSB} | max{600, 6xTSSB} |
| 20 | max{600, 6xTSSB} | max{600, 6xTSSB} | max{600, 6xTSSB} | max{600, 6xTSSB} | max{600, 6xTSSB} | max{600, 6xTSSB} |
| 32,40 | max{600, 6xTSSB} | max{600, 6xTSSB} | max{600, 6xTSSB} | max{600, 6xTSSB} | max{600, 6xTSSB} | max{600, 6xTSSB} |
| 64,80 | 30x DRX | 30x DRX | 30x DRX | 30x DRX | max{600, 6xTSSB} 30x DRX | max{600, 6xTSSB} 30x DRX |
| 128,160 | 15x DRX | 15x DRX | 15x DRX | 15x DRX | 15x DRX | max{600, 6xTSSB} 15x DRX |
| (160,2560] | 15x DRX | 15x DRX | 15x DRX | 15x DRX | 15x DRX | 15x DRX |

We believe that it’s better to re-use LTE requirement structure based on only DRX cycle in NR to trade off the power saving and performance flexibility. The requirement table for intra-frequency PSS/SSS detection per single carrier could be as follow.

Proposal 4: The Intra-frequency PSS/SSS detection requirement table without measurement gap used is as follow.

Table 3. Time period for PSS/SSS detection, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync |
| --- | --- |
| No DRX, or DRX cycle≤ 0.04 | max(600ms, 6 x SMTC period) Note 1 |
| 0.04 <DRX cycle≤ 0.08 | 30x DRX |
| 0.08< DRX cycle≤ 2.56 | 15x DRX |
| Note 1 : If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

When DRX On-duration is not always aligned with SMTC, it should be allowed to have some relaxation in requirement. Detail can be referred on our anther Tdoc [8].

Proposal 5: When DRX On-duration is not always aligned with SMTC, it should be allowed to have some relaxation in requirement.

  - Intra-frequency PSS/SSS detection requirement when multi-CCs are used

In LTE, PSS and SSS comes in every 5ms. Therefore a single cell search engine is able to handle 5 CCs serially. However, NR cell search in CA case is different to LTE. In NR, the SMTCs for different CCs can be independently configured. Therefore, a worst case happens when the SMTC occasions of all CCs collide with each other, as illustrated by the red circle in Figure 1. In fact, we think this could be the typical deployment of SMTC because SMTC occasions need to align with gap. In this case, it is impossible to only use one single engine to perform parallel cell search for multiple CCs. Adding more search engines is one solution, but the cost will be significantly increased. Actually, compared to LTE, a single cell search engine in NR is already far complicated, because of the need to support more PCI hypotheses, more new features, such as multiple beams per cell and different numerologies such as 240KHz etc.

Figure 1. SMTC occasions of different CC collided with one another

For FR1, it could be possible to use more buffer size to dump the data for balancing the processing time; but for FR2, especially for 240 KHz, due to the higher sampling rate and more SSBs, the buffer size could become too large. It is extremely high-cost to handle this worst case which SMTC occasions appear at the same time of all CCs. Therefore, to ease UE’s implementation cost of cell search engine, it is suggested to further scale the delay requirements by adding parameters of configured NR CCs (NNR_CC,FR1 and NNR_CC,FR2).

Regarding the long delay in a multi-CC scenario, we think it should be fine because CA is typically used for low mobility UEs. For high-mobility UEs, network can configure single CC or less number of CCs to UE in order to obtain acceptable mobility performance.

Regarding DRX, which is used for UE with low data rate demand, we believe that CA is usually not configured for DRX UEs. Therefore, the delay requirements can still be scaled based on NNR_CC,FR1 and NNR_CC,FR2, although NNR_CC,FR1 and NNR_CC,FR2 are typically 1 for DRX UEs.

Proposal 6: TPSS/SSS_sync without measurement gap requirement is defined as follow.

In FR1, TPSS/SSS_sync,per CC = max(600ms, 6 x SMTC period);

In FR2, TPSS/SSS_sync,per CC = max(TBDms, TBDNote1 x SMTC period);

when multiple CCs are configured: TPSS/SSS_sync = TPSS/SSS_sync,per CC x(NNRcc,FR12+NNRcc,FR2), for No DRX, or DRX cycle <0.04s, where NNR_CC,FR1 is the number of configured NR FR1 CCs to the UE, NNR_CC,FR2 is the number of configured NR FR2 CCs to the UE. (Note 1: Not yet consider Rx beam sweeping in FR2.)

In the requirement definition, we should consider the worst case which all CCs have the same longest SMTC periodicity and collide with each other. But in the real field, considering the different SMTC offset configuration, there is a large probability to avoid each CC colliding together for long SMTCs. So the real time needed for multi-CCs’ PSS/SSS detection will be decreased dramatically. For example, when 4 FR1 CCs share the same periodicity of 160ms with different offsets, the requirement is still TPSS/SSS_sync = 2x6x160ms = 1920ms, but actually UE can finishes all detections within 6x160ms = 960ms.

- Summary

In this paper, we provided simulation results for PSS/SSS detection in FR2 and also propose intra-frequency PSS/SSS detection requirement without measurement gap.

Observation 1: The PSS/SSS detection without Rx beamforming should be at least 4 attempts for targeting 90% detection probability in frequency range FR2.

Proposal 1: The intra-frequency PSS/SSS detection requirement in FR1 is max(600ms, 6*SMTC period) when no DRX is configured.

Proposal 2: The midpoint between long and short DRX in detection requirement should be 40ms.

Proposal 3: It should re-use the LTE requirement in NR for power saving under large DRX scenarios.

Proposal 4: The Intra-frequency PSS/SSS detection requirement table without measurement gap used is as follow.

| DRX cycle | TPSS/SSS_sync |
| --- | --- |
| No DRX, or DRX cycle≤ 0.04 | max(600ms, 6 x SMTC period) Note 1 |
| 0.04 <DRX cycle≤ 0.08 | 30x DRX |
| 0.08< DRX cycle≤ 2.56 | 15x DRX |
| Note 1 : If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Proposal 5: When DRX periodicities are not align with SMTC periodicities, it should be allowed to have some relaxation in requirement.

Proposal 6: TPSS/SSS_sync without measurement gap requirement is defined as follow.

In FR1, TPSS/SSS_sync,per CC = max(600ms, 6 x SMTC period);

In FR2, TPSS/SSS_sync,per CC = max(TBDms, TBDNote1 x SMTC period);

when multiple CCs are configured: TPSS/SSS_sync = TPSS/SSS_sync,per CC x(NNRcc,FR12+NNRcc,FR2), for No DRX, or DRX cycle <0.04s, where NNR_CC,FR1 is the number of configured NR FR1 CCs to the UE, NNR_CC,FR2 is the number of configured NR FR2 CCs to the UE.(Note 1: Not yet consider Rx beam sweeping in FR2.)

- References

- R4-1713943, “Ad hoc minutes for NR RRM on interruption and measurement”, Ericsson

- R4-1710943, “Discussion on cell identification requirements”, NTT DOCOMO

- R4-1709441, “Link level evaluation results on NR-PSS/SSS detection”, NTT DOCOMO

- R4-1709622, “Updated simulation results of PSS/SSS detection in NR”, Huawei, HiSilicon

- R4-1709735, “PSS/SSS detection in NR: updated link level simulation results”, Qualcomm

- R4-081073, “UE Measurement performance requirements for LTE_RRC_Connected with large DRX cycles”, Nokia, May. 2008

- R4-1714253, “TP to TS 38.133: Intrafrequency NR measurement requirements”, Ericsson

- R4-1800109, “Power consumption issue for non-aligned SMTC and DRX”, Mediatek

- Annex A: Simulation Assumptions

Table A1: Cell-specific Parameters for PSS/SSS Detection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| RB Utilization | % | 100 | 100 |
| Power boosting for PSS and SSS | dB | 0 | 0 |
| Power boosting for Data and Control | dB | 0 | 0 |
| Data Modulation | - | QPSK | QPSK |
| CP Length | - | Normal | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 | 1 |
| SS burst set periodicity | ms | 40 | 40 |
| Relative Delay of 1st Path (synchronous) | μs | 0 | CP/2 |
| Relative Delay of 1st Path (asynchronous) | ms | 0 | 1.5 |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| Es/Noc for K=1 | dB | 6.4 | 1.25 (SINR: -6dB) |
| Number of Tx antennas | - | 1 | 1 |
| PSS Sequence ID | - | Shows in Table 3, 4 | Shows in Table 3, 4 |
| SSS Sequence ID | - | Shows in Table 3, 4 | Shows in Table 3, 4 |
| Propagation Condition | - | AWGN, EPA5, ETU30 TDL-A/B/C/D Note1 |  |
| Noc Model Note3 | - | AWGN noise |  |
| NOTE 1: The channel models of TDL/A/B/C/D are used for simplified link level evaluations, which are defined in TR 38.900. The delay spread is assumed as 100ns. NOTE 2: Noc value doesn’t include the simulated BS signals’ power. |  |  |  |

Table A2: UE-specific Parameters for Cell Detection

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of Cell 1 and Cell 2 by the UE | No |
| False detect threshold | Required as in a real UE implementation |
| UE having apriority knowledge of system being synchronous or asynchronous | No |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both PSS and SSS sequence IDs. |
| CP length detection | Not simulated |
| Receive antennas | 2, 4 (uncorrelated. Without receiver beamforming) |

Table A3: Cell ID Combinations to be simulated

| Case # | Cell 1 (Interferer Cell) |  | Cell 2 (Desired Cell) |  | Scenario |
| --- | --- | --- | --- | --- | --- |
| 1 | psc1 | ssc1 | psc2 | ssc2 | Synchronous |
| 2 | psc1 | ssc1 | psc1 | ssc3 | Synchronous |
| 3 | psc1 | ssc1 | psc2 | ssc2 | Asynchronous |
| 4 | psc1 | ssc1 | psc1 | ssc3 | Asynchronous |

Table A4: PSS, SSS indices for simulations

| PSS Indices |  | SSS Indices |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Label | Code index | Label | Code index (m0, m1) | Cell ID | (,) |
| psc1 | 43 | ssc1 | (20, 0) | 337 | (112, 1) |
| psc2 | 86 | ssc2 | (40, 0) | 674 | (224, 2) |
|  |  | ssc3 | (35,.0) | 673 | (224, 1) |